گزارش ماهیانه بروزرسانی داده ها در نرم افزار OFM

Update Current OFM Database and Report Monthly

South Azadegan Oil Field

(November 2024)

کارفرما:

شرکت پترو پارس با مسئولیت محدود

مشاور:

شرکت خدمات مهندسی نفت کیش (KPE)

| 03 | November-2024 | Foad Saberi | Mehdi Pirayeh | Akbar Fotouhi |
| --- | --- | --- | --- | --- |
| Rev | Date | Prepared | Checked | Approved |

# Table of Contents

Title											        Page No.

Table of Contents	2

List of Figures	3

List of tables	3

Summary	5

1	OFM Project	5

2	Updated OFM Model	9

3	Updated OFM results	9

# List of Figures

Title											        Page No.

Figure ‎1-1: OFM Schema Tables	5

Figure ‎1-2: OFM base map	6

Figure ‎3-1: Cumulative oil production Comparison of South Azadegan oil field reservoirs	9

Figure ‎3-2: Well Status of South Azadegan Wells (Reservoir by Reservoir) (up to November 2024)	16

Figure ‎3-2: Well Status of South Azadegan Wells (Active, Not Active Wells) (up to November 2024)	17

Figure ‎3-3: Cumulative total producer wells and active wells of south Azadegan oil field reservoirs (up to November 2024)	29

Figure ‎3-4: Cumulative Production for South Azadegan field (up to November 2024)	30

Figure ‎3-5: Cumulative Production for Sarvak Reservoir of South Azadegan (up to November 2024)	30

Figure ‎3-6: Cumulative Production for Kazhdumi Reservoir of South Azadegan (up to November 2024)	31

Figure ‎3-7: Cumulative Production for Gadvan Reservoir of South Azadegan (up to November 2024)	31

Figure ‎3-8: Cumulative Production for Fahliyan Reservoir of South Azadegan (up to November 2024)	32

Figure ‎3-9: Cumulative oil production Bubble map and top 10 producer wells of South Azadegan Field (up to November 2024)	33

Figure ‎3-10: Cumulative oil production Bubble map and top 10 producer wells of Sarvak reservoir (up to November 2024)	34

Figure ‎3-11: Cumulative oil production Bubble map and top 10 producer wells of Kazhdumi reservoir (up to November 2024)	36

Figure ‎3-12: Cumulative oil production Bubble map and top 10 producer wells of Gadvan reservoir (up to November 2024)	38

Figure ‎3-13: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to November 2024)	40

# List of tables

Title											        Page No.

Table ‎3-1: Well status classification of existed wells of South Azadegan Field in OFM (up to November 2024)	10

Table ‎3-1: Well status classification of existed wells( Active,Not Active) of South Azadegan Field in OFM (up to November 2024)	10

Table ‎3-2:Azadegan Wells Which Were Suspended In This Month(November 2024)	11

Table ‎3-3:Azadegan Wells Which Were Ongoing In This Month(November 2024)	11

Table ‎3-4:Azadegan Wells Which Were completed In This Month(November 2024)	12

Table ‎3-4:Azadegan Wells Which Were Closed In This Month(November 2024)	15

Table ‎3-5: Azadegan wells status, oil production rate and cumulative	18

Table ‎3-6: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to November 2024)	35

Table ‎3-7: Cumulative oil production top 10 producer wells of Kazdumi reservoir (up to November 2024)	37

Table ‎3-8: Cumulative oil production top 10 producer wells of Gadvan reservoir (up to November 2024)	39

Table ‎3-9: Cumulative oil production top 10 producer wells of Fahliyan reservoir (up to November 2024)	41

List of abbreviation

| Description | Abbreviation |
| --- | --- |
| Schlumberger Oil Field Manager | OFM |
| Total Depth | TD |
| Rotary Table Elevation | RTE |
| Gas Oil Ratio | GOR |
| Basic sediment and water | BSW |
| American Petroleum Institute gravity | API |
| Drill Stem Test | DST |
| Modular Formation Dynamics Tester | MDT |
| Wireline Formation Test | WFT |

# Summary

Following report includes the description of updated South Azadegan oil field OFM up to November 2024. The daily production table of OFM updated. Based on these new production data, this field produces around 141 MSTB/d in this month. The total number of producer wells of this field is 169 wells, which 140 wells are active on average in this month. The total cumulative oil production of the field is around 351.8 MMSTB until the end of this month (the field produced 4.2 MMSTB oil during this month). It should be noted that other new data related to recent operations are added to OFM. These updates are descripted in Section 3 in detail.

# OFM Project

The current OFM project contains all related data of south Azadegan oil field (and Also south Yaran field). All received data for each well are given in their related tables. 207 completions (202 wellbores) in South Azadegan field (up to well AZNS-206) and 18 completions in South Yaran field are defined in this database. The Schema Tables related to this OFM project is depicted in Figure ‎1-1. Also, the base map of the field is shown in Figure ‎1-2.

Figure ‎1-1: OFM Schema Tables

Figure ‎1-2: OFM base map

The database of OFM model for South Azadegan field contains following tables:

## Header ID:

Header ID table of Azadegan OFM model defines the Azadegan wells and include information of drilling, production and geology of Azadegan field wells (TD, production interval, production formation, RTE, PVT and etc.).

## Daily Production

The daily production table, contains the daily production data of south Azadegan wells contains daily production rates (Oil, gas, water), chock size, GOR, BSW%, Wellhead pressure, downstream pressure, salt content, API, H2S content for the wells of south Azadegan and south Yaran fields.

## Completion

This table contains perforation depths intervals and existed plug (if any).

## H2S

In this table, the H2S content for those wells that the H2S are recorded and their depth intervals are given.

## Coring

The coring interval, recovery percent are given in this table.

## DST

In this table, all the information related to DST tests includes depth intervals, Formation and zone, test duration, production rate (oil, gas, and, water rates), ratios (GOR, BSW), chock size, gauge depth and gauge pressure and etc. are given.

## Production Tests

This table contains the information related to Production tests includes depth intervals, Formation and zone, test duration, production rate (oil, gas, and, water rates), ratios (GOR, BSW), choke size, gauge depth and gauge pressure, separator pressure and temperature, before and after choke pressure and choke temperature and, wellhead pressure and temperature are given.

## PLT

The PLT data includes oil and water rates, oil contributions and their depth intervals are given in this table.

## Formation Water

The results of water analysis include depth intervals, color, PH, salinity, and the chemical element percentages for those wells that have a water sample are gathered in this table.

## Static Pressure Tests

The results of the static surveys contain pressure gradient, pressure at datum and, pressure at the mid perforation could be found in this table.

## Flowing Pressure Tests

The results of the flowing surveys contain pressure gradient, pressure at datum and, pressure at the mid perforation could be found in this table.

## Stimulation

In the stimulation table, all data related to stimulation jobs (acidizing and clean-up), total volume of injected acid, injected acid per foot, depth intervals, main acid recipe, and, their PI values before and after acidizing) are given.

## WFT

The data related to wireline formation tests such as WFT type, gauge type, formation pressure and mobility (for MDT) are given.

# Updated OFM Model

- Production data of November 2024 is added to database.

# Updated OFM results

In this section, the summary of production data up to November 2024 related to south Azadegan reservoirs presented in Figure ‎3-1. 207 completions (202 wellbores) in South Azadegan field (up to well AZNS-206) and 18 completions in South Yaran field are defined in this database. A classification related to well status of existed wells of South Azadegan field up to this month are listed in Table ‎3-1 and Depicted in Figure ‎3-2. This field produces around 141 MSTB/d with an average of 140 active wells in this month. The total number of producer wells is 169 wells. Figure ‎3-4 shows the total producer wells and active wells of south Azadegan oil field during the production history. The cumulative produced oil of the field up to this month is around 351.8 MMSTB (the field produces 4.2 MMSTB oil during this month). The cumulative production profile of field (and also for each reservoir) and bubble map of cumulative oil production for producer wells of south Azadegan field along with the list of top ten producing wells (for field and each reservoir individually)  are shown in Figure ‎3-5 to respectively.

| Reservoir; Cum Oil (MMSTB); Cum Gas (Bscf); Cum Water (MSTB). Sarvak; 171.3; 60.0; 1430.5. Kazhdumi; 37.3; 30.0; 2258.8. Gadvan; 99.0; 73.1; 555.0. Fahliyan; 44.2; 55.1; 178.8. Total; 351.8; 218.2; 4423.1. |  |
| --- | --- |

Figure ‎3-1: Cumulative oil production Comparison of South Azadegan oil field reservoirs

Table ‎3-1: Well status classification of existed wells of South Azadegan Field in OFM (up to November 2024)

| Well Status | No. of Wellbore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reservoir by Reservoir |  |  |  |  | Field |
|  | Sarvak | Kazhdumi | Gadvan | Fahliyan | Asmari(Disposal) | South Azadegan |
| Oil producer | 133 | 10 | 14 | 12 |  | 169 |
| Completed | 6 |  | 3 | 5 | 4 | 18 |
| On-going |  |  | 1 | 5 | 1 | 7 |
| Observation |  |  |  | 1 |  | 1 |
| Suspended | 1 |  | 4 | 3 |  | 8 |
| Total | 140 | 10 | 21 | 27 | 5 | 203 |

Figure ‎3-2: Well Status of South Azadegan Wells (Reservoir by Reservoir) (up to November 2024)

Table ‎3-2: Production status classification of existed wells (Active, Close) of South Azadegan Field in OFM (November 2024)

| Well Status | No. of Wellbore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reservoir by Reservoir |  |  |  |  | Field |
|  | Sarvak | Kazhdumi | Gadvan | Fahliyan | Asmari(Disposal) | South Azadegan |
| Active | 113 | 6 | 13 | 11 |  | 143 |
| Close | 20 | 4 | 1 | 1 |  | 26 |
| Total | 133 | 10 | 14 | 12 |  | 169 |

Figure ‎3-3: Well Status of South Azadegan Wells (Active, Not Active Wells) (up to November 2024)

In the South Azadegan field, the following wells have been suspended: AZN-12, AZNS-51, AZNS-52, AZNS-75, AZNS-113, AZNS-174, AZNS-193, and AZNS-204. These wells are summarized in Table ‎3-3.

Table ‎3-3: South Azadegan Wells which their status were suspended in November 2024

| Well | Formation | Pad | Remark |
| --- | --- | --- | --- |
| AZN-012 | Gadvan | Near 75 | Planned for Workover by MSA |
| AZNS-051 | Gadvan | 14 | Planned for Workover by TDDC due to completion failure |
| AZNS-052 | Fahliyan | 63 | Planned for completion by TDDC |
| AZNS-075 | Gadvan | 80 | Planned for Workover by MSA |
| AZNS-113 | Fahliyan | 82 | Planned for Plug-Prong retrieval by NIDC |
| AZNS-174 | Fahliyan | 59 | Planned for Workover by NIDC due to completion failure |
| AZNS-193 | Sarvak | 14 | Planned for Workover by TDDC due to completion failure |
| AZNS-204 | Gadvan | 79 | Planned for completion by TDDC |

The following wells are classified in the ongoing operation category: AZNS-122, AZNS-175, AZNS-179, AZNS-196, AZNS-200, AZNS-201 and AZNS-207. These wells are summarized in Table ‎3-4.

Table ‎3-4: South Azadegan Wells which their status were Ongoing in November 2024

| Item | Well | Formation | Pad | Remark |
| --- | --- | --- | --- | --- |
| 1 | AZNS-122 | Fahliyan | 55 | Preparing for Side Tracking Operation |
| 2 | AZNS-175 | Fahliyan | 92 | Decompletion-Recompletion |
| 3 | AZNS-179 | Asmari (Water Disposal) | 85 | Fishing (Wire line Logging) |
| 4 | AZNS-196 | Fahliyan | 61 | Dry Test of 7 inch tie back liner hanger |
| 5 | AZNS-200 | Fahliyan | 94 | Perforation Operation |
| 6 | AZNS-201 | Gadvan | 74 | Decompletion due to packer failure |
| 7 | AZNS-207 | Fahliyan | 93 | Drilling to 1552 m MD |

In the South Azadegan field, the status of the following wells were completed in November 2024 which are summarized in Table ‎3-5.

Table ‎3-5: South Azadegan Wells which their status were completed in November 2024

| Item | Well | Pad | Formation | Well Status | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | AZN-010 | near50 | Fahliyan | Completed | Waiting for commissioning |
| 2 | AZNS-066 | 84 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 3 | AZNS-085 | 79 | Fahliyan | Completed | Waiting for production line and surface facilities installation |
| 4 | AZNS-116 | 81 | Fahliyan | Completed | Planned for Acidizing by MSA-OEOC |
| 5 | AZNS-120 | NL-2-5 | Gadvan | Completed | Waiting for commissioning |
| 6 | AZNS-127 | 54 | Fahliyan | Completed | Acidizing By OEOC, Waiting for commissioning |
| 7 | AZNS-149 | 69 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 8 | AZNS-159 | 76 | Sarvak | Completed | Planned for ESP installation |
| 9 | AZNS-165 | 38 | Sarvak | Completed | Planned for Work-over or ESP installation |
| 10 | AZNS-173 | 72 | Gadvan | Completed | Acidizing By NIDC, Waiting for production line and surface facilities installation and commissioning |
| 11 | AZNS-180 | 72 | Sarvak | Completed | Acidizing By NIDC, Waiting for production line and surface facilities installation and commissioning |
| 12 | AZNS-183 | 59 | Sarvak | Completed | Planned for Acidizing by NIDC |
| 13 | AZNS-195 | 64 | Fahliyan | Completed | Planned for Acidizing by OEOC |
| 14 | AZNS-198 | 82 | Sarvak | Completed | Planned for Acidizing by NIDC |
| 15 | AZNS-202 | 87 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 16 | AZNS-203 | 72 | Sarvak | Completed | Acidizing By NIDC, Waiting for production line and surface facilities installation and commissioning |
| 17 | AZNS-205 | 86 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 18 | AZNS-206 | 82 | Gadvan | Completed | Acidizing By NIDC, Waiting for commissioning |

In the South Azadegan field, the status of the following wells were closed in November 2024 which are summarized in Table ‎3-6.

Table ‎3-6: South Azadegan Wells which their status were closed in November 2024

| Item | Well Name | Formation | Zone | Pad Name | Closed Date | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | AZN-018 | Sarvak | Sv.3.4-6 | Pad02 | 16-06-24 | Wellhead pressure drop. The well pressure dropped suddenly at the end of May. 2024 |
| 2 | AZN-019 | Sarvak | Sv.3 | Pad13 | 04-10-21 | It probably closed due to water in the wellbore and pressure drop |
| 3 | AZNS-043 | Sarvak | Sv.3.4-6 | Pad11 | 05-03-24 | Wellhead pressure drop |
| 4 | AZNS-055 | Sarvak | Sv.3.4-6 | Pad04 | 31-08-24 | Wellhead pressure drop |
| 5 | AZNS-060 | Fahliyan | Fa.4a | Pad78 | 01-11-24 | Closed due to fishing operation |
| 6 | AZNS-061 | Sarvak | Sv.3 | Pad77 | 26-05-24 | Wellhead pressure drop |
| 7 | AZNS-073 | Sarvak | Sv.3.4-6 | Pad05 | 25-05-22 | Wellhead pressure drop |
| 8 | AZNS-077 | Sarvak | Sv.3.4-6 | Pad13 | 31-12-22 | Wellhead pressure drop |
| 9 | AZNS-078 | Sarvak | Sv.3.4-6 | Pad63 | 14-05-22 | Delivered to PEDEC for workover operation on pad, however based on production data the last WHP was low |
| 10 | AZNS-081 | Sarvak | Sv.3.4-6 | Pad18 | 31-12-22 | Wellhead pressure drop, water production also reported, WHP drop could be due to water production |
| 11 | AZNS-088 | Sarvak | Sv.3 | Pad80 | 13-03-24 | Wellhead pressure drop |
| 12 | AZNS-106 | Sarvak | Sv.3 | Pad81 | 29-04-24 | Delivered to PEDEC for Workover in other well on pad (AZMS-116) |
| 13 | AZNS-117 | Sarvak | Sv.3.4-6 | Pad53 | 25-05-22 | Wellhead pressure drop |
| 14 | AZNS-129 | Sarvak | Sv.8 | Pad12 | 31-08-24 | Wellhead pressure drop |
| 15 | AZNS-131 | Sarvak | Sv.3.4-6 | Pad32 | 27-11-23 | Salty oil production |
| 16 | AZNS-132 | Sarvak | Sv.3 | Pad73 | 31-08-24 | Wellhead pressure drop |
| 17 | AZNS-141 | Sarvak | Sv.3.4-6 | Pad54 | 03-04-24 | Delivered to PEDEC for Workover in other well on pad (AZMS-127) |
| 18 | AZNS-146 | Sarvak | Sv.3.4-6 | Pad70 | 31-08-24 | Wellhead pressure drop |
| 19 | AZNS-147 | Sarvak | Sv.3 | Pad71 | 16-06-24 | Wellhead pressure drop |
| 20 | AZNS-156 | Sarvak | Sv.3 | Pad54 | 03-04-24 | Delivered to PEDEC for Workover in other well on pad (AZMS-127) |
| 21 | AZNS-164 | Sarvak | Sv.3.4-6 | Pad29 | 31-08-24 | Wellhead pressure drop |
| 22 | AZNS-028 | Kazhdumi | Kz.2.3.4 | Pad 11 | 01-05-19 | closed due to asphaltene barrier on 1398/08/08 |
| 23 | AZNS-041 | Kazhdumi | Kz.2.4 | Pad 20 | 01-10-24 | well closed due to asphaltene barrier in the well on 1402/10/11 |
| 24 | AZNS-064 | Kazhdumi | Kz.2.4 | Pad13 | 05-03-20 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| 25 | AZNS-130 | Gadvan | Gd.5a | Pad 81 | 30-04-24 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| 26 | AZN-021 | Kazhdumi | Kz.2 | Pad 15 | 30-04-20 | closed due to pressure drop due to water production on 1399/02/11 |

Table ‎3-7: South Azadegan wells status, oil production rate and cumulative

| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (PSI) | Cum (First prod--Last prod) MMSTB | Oil Rate (STBD) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AZN-001 | Near 19 | Gadvan | Gd.5a.5c | Producer | 36 | 500 | 14.1 | 1522 | well opened at 13:30 on 1403/03/21 |
| AZN-002 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 32 | 180 | 5.91 | 901 |  |
| AZN-004 | Near 56 | Kazhdumi | Kz.1.2 | Producer | 32 | 1420 | 9.65 | 2766 | the well was closed and opened for spool replacement on 1402/12/14 |
| AZN-004_F | Near56 | Fahliyan | _ | Comp. Changed |  |  | _ | _ |  |
| AZN-004_G | Near56 | Gadvan | _ | Comp. Changed |  |  | _ | _ |  |
| AZN-005 | NEAR 1 | Sarvak | Sv.3.4-6 | Producer | 24 | 160 | 4.95 | 408 |  |
| AZN-005_K | Near1 | Kazhdumi | _ | Comp. Changed |  |  |  |  |  |
| AZN-006 | NEAR 79 | Sarvak | Sv.4-6 | Producer | 36 | 330 | 13.56 | 1658 |  |
| AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 34 | 200 | 10.22 | 940 |  |
| AZN-009 | 16 | Gadvan | Gd.5c | Producer | 50 | 900 | 28.84 | 3855 | well closed due to fault WK at 02:55 and opened at 13:30 on 1402/09/05 |
| AZN-010 | near50 | Fahliyan |  | Completed |  |  | _ | _ |  |
| AZN-011 | Near 66 | Fahliyan | Fa.3a | Producer | 34 | 1200 | 9.98 | 2878 | well closed at 05:45 due to fault of panel and opened at 10:10 on 1403/02/04 |
| AZN-012 | Near 75 | Gadvan |  | Suspended |  |  | 8.12 | _ |  |
| AZN-014 | NEAR 73 | Sarvak | Sv.3.4-6 | Producer | 20 | 120 | 3.57 | 259 |  |
| AZN-014_G | near 73 | Gadvan |  | Comp. Changed |  |  | _ |  |  |
| AZN-015 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 120 | 6.04 | 502 |  |
| AZN-016 | 25 | Sarvak | Sv.3.4-6 | Producer | 20 | 60 | 2.56 | 143 |  |
| AZN-017 | 50 | Kazhdumi | Kz.2 | Producer | 20 | 1440 | 13.86 | 1325 | 10' line( well was closed from 11:00 to 16:00 due to MPFM connection) in 1403/08/12 |
| AZN-018 | 2 | Sarvak | Sv.3.4-6 | Close |  |  | 7.46 | _ |  |
| AZN-019 | 13 | Sarvak | Sv.3 | Close |  |  | 10.14 | _ |  |
| AZN-020 | 7 | Gadvan | Gd.5c | Producer | 24 | 50 | 5.25 | 99 | Well reopened at 16:40. in 1403/06/02 |
| AZN-021 | 15 | Kazhdumi | Kz.2 | Close | 24 | 0 | 5.36 | _ | closed due to pressure drop due to water production on 1399/02/11 |
| AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 120 | 2.62 | 272 |  |
| AZN-023 | Near 75 | Gadvan | Gd.5a.5c | Producer | 34 | 1200 | 16.11 | 2953 | well closed due to fault WK at 03:10 and opened at 14:50 on 1402/09/05 |
| AZN-024 | Near 75 | Gadvan | Gd.3.5c.7 | Producer | 28 | 600 | 1.89 | 1048 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZNS-026 | 23 | Gadvan | Gd.1-6 | Producer | 32 | 600 | 5.7 | 1256 | 10" line. Well opened at 21:20 in 1403/06/03 |
| AZNS-027 | 4 | Sarvak | Sv.3 | Close |  |  | 1.09 | _ |  |
| AZNS-028 | 11 | Kazhdumi | Kz.2.3.4 | Close | 24 | 0 | 0.53 | _ | closed due to asphalthene barrier on 1398/08/08 |
| AZNS-029 | 5 | Kazhdumi | Kz.3.4 | Producer | 28 | 190 | 0.45 | 375 | well closed at 11:00 due to disconnect of MPFM and opened at 17:30 in 1403/07/01 |
| AZNS-030 | 27 | Sarvak | Sv.3 | Producer | 28 | 360 | 1.44 | 1358 |  |
| AZNS-031 | 35 | Sarvak | Sv.3.4-6 | Producer | 28 | 300 | 0.03 | 305 |  |
| AZNS-032 | 21 | Sarvak | Sv.3 | Producer | 28 | 350 | 0.14 | 1324 |  |
| AZNS-033 | 7 | Sarvak | Sv.3 | Producer | 28 | 310 | 2.4 | 946 |  |
| AZNS-034 | 23 | Sarvak | Sv.3 | Producer | 32 | 390 | 2 | 1401 |  |
| AZNS-035 | 33 | Sarvak | Sv.3 | Producer | 24 | 350 | 0.55 | 1222 |  |
| AZNS-036 | 50 | Sarvak | Sv.3 | Producer | 28 | 240 | 1.95 | 931 |  |
| AZNS-037 | 7 | Sarvak | Sv.3 | Producer | 24 | 250 | 2.17 | 617 |  |
| AZNS-038 | 21 | Sarvak | Sv.3 | Producer | 28 | 280 | 0.12 | 790 |  |
| AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 28 | 350 | 4.24 | 975 |  |
| AZNS-040 | 50 | Fahliyan | Fa.1b.1c | Producer | 28 | 1700 | 0.49 | 2390 | closed at 12:25 pm by order of management in 1403/06/12 |
| AZNS-041 | 20 | Kazhdumi | Kz.2.4 | Close | 28 | 0 | 1.06 | _ | well closed due to asphaltene barrier in the well on 1402/10/11 |
| AZNS-042 | 10 | Sarvak | Sv.3.4-6 | Producer | 15 | 200 | 1.18 | 245 |  |
| AZNS-043 | 11 | Sarvak | Sv.3.4-6 | Close |  |  | 2.04 | _ |  |
| AZNS-044 | 15 | Sarvak | Sv.3.4-6 | Producer | 15 | 230 | 0.51 | 198 |  |
| AZNS-045 | 56 | Fahliyan | Fa.4a.4b | Producer | 24 | 840 | 0.65 | 295 | well reopened after finish well test 23:15. in 1403/09/08 |
| AZNS-046 | 62 | Fahliyan | Fa.1.3a | Producer | 49 | 1230 | 3.71 | 5392 | the well was closed from 10:15 to 13:15 to clean the stone trap in 1403/05/01 |
| AZNS-047 | 77 | Gadvan | Gd.5a.5c | Producer | 30 | 1400 | 2.62 | 3017 | well opened at 14:30 on 1403/02/11 |
| AZNS-048 | 20 | Sarvak | Sv.3.4-6 | Producer | 24 | 200 | 1.4 | 622 |  |
| AZNS-049 | 69 | Fahliyan | Fa.1a.1c.3a | Producer | 32 | 1500 | 6.48 | 2706 | closed at 12:35 pm by order of management in 1403/06/12 |
| AZNS-050 | 57 | Fahliyan | Fa.4a | Producer | 28 | 900 | 2.09 | 1726 | well closed at 00:30 due to leakage of 20" line and opened at 04:15 in 1403/08/11 |
| AZNS-051 | 14 | Gadvan |  | Suspended |  |  | _ | _ |  |
| AZNS-052 | 63 | Fahliyan |  | Suspended |  |  | _ | _ |  |
| AZNS-053 | 51 | Sarvak | Sv.3 | Producer | 20 | 200 | 1.2 | 480 |  |
| AZNS-054 | 15 | Sarvak | Sv.3.4-6 | Producer | 15 | 200 | 0.99 | 222 |  |
| AZNS-055 | 4 | Sarvak | Sv.3.4-6 | Close |  |  | 2.16 | _ |  |
| AZNS-056 | 5 | Sarvak | Sv.3.4-6 | Producer | 22 | 330 | 1.64 | 655 |  |
| AZNS-057 | 25 | Sarvak | Sv.3.4-6 | Producer | 22 | 190 | 0.81 | 565 |  |
| AZNS-058 | 18 | Gadvan | Gd.5c | Producer | 32 | 700 | 2.48 | 1384 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| AZNS-059 | 20 | Sarvak | Sv.3.4-6 | Producer | 28 | 250 | 2.08 | 1058 |  |
| AZNS-060 | 78 | Fahliyan | Fa.4a | Close | 20 | 280 | 1.88 | 188 | well closed at 11:00 due to fishing operation in 1403/08/11 |
| AZNS-061 | 77 | Sarvak | Sv.3 | Close |  |  | 1.43 | _ |  |
| AZNS-062 | 56 | Sarvak | Sv.3.4-6 | Producer | 23 | 220 | 1.05 | 689 |  |
| AZNS-063 | 51 | Sarvak | Sv.3.4-6 | Producer | 24 | 220 | 1.17 | 703 |  |
| AZNS-064 | 13 | Kazhdumi | Kz.2.4 | Close | 28 | 0 | 0.54 | _ | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| AZNS-065 | 69 | Sarvak | Sv.3 | Close |  |  | 0.49 | _ |  |
| AZNS-066 | 84 | Asmari (Water Disposal) |  | Completed |  |  | _ | _ |  |
| AZNS-067 | 18 | Sarvak | Sv.3.4-6 | Producer | 28 | 350 | 1.59 | 1020 |  |
| AZNS-068 | 6 | Sarvak | Sv.8 | Producer | 15 | 250 | 1.05 | 262 |  |
| AZNS-069 | 10 | Sarvak | Sv.3.4-6 | Producer | 15 | 250 | 1.02 | 258 |  |
| AZNS-070 | 75 | Fahliyan | Fa.2a.3a | Producer | 30 | 1000 | 6.3 | 1865 | well opened at 17:30 in 1403/09/02 |
| AZNS-071 | 17 | Gadvan | Gd.5c | Producer | 28 | 550 | 3.26 | 943 | well closed from08:00 to 10:30 due to problem in control panel in 1403/08/06 |
| AZNS-072 | 57 | Sarvak | Sv.3.4-6 | Producer | 15 | 200 | 0.63 | 297 |  |
| AZNS-073 | 5 | Sarvak | Sv.3.4-6 | Close |  |  | 0.94 | _ |  |
| AZNS-074 | 83 | Sarvak | Sv.3 | Producer | 15 | 200 | 0.92 | 282 |  |
| AZNS-075 | 80 | Gadvan |  | Suspended |  |  | _ | _ |  |
| AZNS-076 | 11 | Sarvak | Sv.3.4-6 | Producer | 15 | 300 | 0.94 | 315 |  |
| AZNS-077 | 13 | Sarvak | Sv.3.4-6 | Close |  |  | 0.82 | _ |  |
| AZNS-078 | 63 | Sarvak | Sv.3.4-6 | Close |  |  | 1.28 | _ |  |
| AZNS-079 | 78 | Sarvak | Sv.3.4-6 | Producer | 26 | 320 | 2.28 | 1032 |  |
| AZNS-080 | 90 | Sarvak | Sv.8 | Producer | 15 | 230 | 1.06 | 256 |  |
| AZNS-081 | 18 | Sarvak | Sv.3.4-6 | Close |  |  | 0.19 | _ |  |
| AZNS-082 | 69 | Sarvak | Sv.3 | Producer | 24 | 290 | 1.93 | 846 |  |
| AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 28 | 300 | 2.68 | 905 |  |
| AZNS-084 | 47 | Sarvak | Sv.3.4-6 | Producer | 15 | 200 | 0.97 | 301 |  |
| AZNS-085 | 79 | Fahliyan |  | Completed |  |  | _ | _ |  |
| AZNS-086 | 1 | Gadvan | Gd.5c | Producer | 36 | 700 | 2.63 | 1780 | due to the leakage in the 32 inch line manifold (2_3) wells were closed from west side of Karun and directed to the North gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-087 | 8 | Sarvak | Sv.8 | Producer | 15 | 220 | 0.66 | 249 |  |
| AZNS-088 | 80 | Sarvak | Sv.3 | Close |  |  | 1.01 | _ |  |
| AZNS-089 | 78 | Sarvak | Sv.3.4-6 | Producer | 20 | 250 | 1.68 | 535 |  |
| AZNS-090 | 90 | Sarvak | Sv.8 | Producer | 22 | 270 | 1.46 | 516 |  |
| AZNS-091 | 83 | Sarvak | Sv.3 | Producer | 24 | 240 | 1.34 | 702 |  |
| AZNS-092 | 19 | Kazhdumi | Kz.2.4 | Producer | 32 | 1020 | 5.19 | 2388 | well opened at 00:50 in 1403/03/10 |
| AZNS-093 | 48 | Sarvak | Sv.3.4-6 | Producer | 15 | 180 | 0.42 | 279 |  |
| AZNS-094 | 89 | Sarvak | Sv.8 | Producer | 15 | 200 | 0.35 | 212 |  |
| AZNS-095 | 31 | Sarvak | Sv.3.4-6 | Producer | 32 | 250 | 2.46 | 993 |  |
| AZNS-096 | 14 | Sarvak | Sv.8 | Producer | 24 | 200 | 0.8 | 497 |  |
| AZNS-097 | 8 | Sarvak | Sv.8 | Producer | 16 | 220 | 1.37 | 278 |  |
| AZNS-098 | 46 | Sarvak | Sv.3 | Producer | 30 | 260 | 0.68 | 1192 |  |
| AZNS-099 | 91 | Sarvak | Sv.8 | Producer | 20 | 170 | 1.02 | 281 |  |
| AZNS-100 | 1 | Sarvak | Sv.4-6 | Producer | 15 | 230 | 0.25 | 243 |  |
| AZNS-101 | 47 | Sarvak | Sv.3.4-6 | Producer | 20 | 250 | 1.56 | 568 |  |
| AZNS-102 | 75 | Sarvak | Sv.3 | Producer | 28 | 250 | 2.26 | 968 |  |
| AZNS-103 | 12 | Gadvan | Gd.5c | Producer | 32 | 590 | 1.74 | 1651 | due to the leakage in the 32 inch line manifold (2_3) wells were closed from west side of Karun and directed to the North gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-104 | 52 | Sarvak | Sv.3 | Producer | 15 | 320 | 1.03 | 431 |  |
| AZNS-105 | 16 | Sarvak | Sv.8 | Producer | 15 | 250 | 0.89 | 232 |  |
| AZNS-106 | 81 | Sarvak | Sv.3 | Close |  |  | 0.89 | _ |  |
| AZNS-107 | 19 | Sarvak | Sv.3.4-6 | Producer | 26 | 100 | 1.69 | 214 |  |
| AZNS-108 | 48 | Sarvak | Sv.3.4-6 | Producer | 30 | 220 | 0.84 | 1069 |  |
| AZNS-109 | 89 | Sarvak | Sv.8 | Producer | 15 | 200 | 0.44 | 241 |  |
| AZNS-110 | 58 | Fahliyan | Fa.4a | Producer | 30 | 900 | 2.53 | 1971 | well closed at 00:35 due to leakage of 20" line and opened at 04:10 in 1403/08/11 |
| AZNS-111 | 46 | Sarvak | Sv.3 | Producer | 24 | 260 | 0.46 | 813 |  |
| AZNS-112 | 75 | Sarvak | Sv.3.4-6 | Producer | 15 | 300 | 1.74 | 370 |  |
| AZNS-113 | 82 | Fahliyan |  | Suspended |  |  | _ | _ |  |
| AZNS-114 | 1 | Sarvak | Sv.3.4-6 | Producer | 15 | 350 | 0.46 | 356 |  |
| AZNS-115 | 45 | Sarvak | Sv.3 | Producer | 15 | 180 | 0.33 | 272 |  |
| AZNS-116 | 81 | Fahliyan | _ | Completed |  |  | _ | _ |  |
| AZNS-117 | 53 | Sarvak | Sv.3.4-6 | Close |  |  | 0.22 | _ |  |
| AZNS-118 | 26 | Sarvak | Sv.3.4-6 | Producer | 24 | 280 | 0.32 | 634 |  |
| AZNS-118_G | 26 | Gadvan |  | Comp. Changed |  |  | _ | _ |  |
| AZNS-119 | 52 | Sarvak | Sv.3 | Producer | 15 | 240 | 0.91 | 343 |  |
| AZNS-120 | NL-2-5 | Gadvan |  | Completed |  |  | _ | _ |  |
| AZNS-121 | 44 | Fahliyan | Fa.1 | Producer | 34 | 2200 | 2.23 | 4081 | Well opened at 1:20 in 1403/03/29 |
| AZNS-122 | 55 | Fahliyan | Fa.4a.4b.5a | Ongoing | 34 | 0 | 1.77 | _ | well was closed at 10:00 due to install the drilling rig on 1402/09/21 |
| AZNS-123 | 67 | Fahliyan |  | Completed |  |  | _ | _ |  |
| AZNS-124 | 49 | Sarvak | Sv.3 | Producer | 30 | 250 | 0.82 | 1146 |  |
| AZNS-125 | 53 | Sarvak | Sv.3.4-6 | Producer | 15 | 180 | 0.43 | 264 |  |
| AZNS-126 | 45 | Sarvak | Sv.3.4-6 | Producer | 20 | 190 | 0.42 | 505 |  |
| AZNS-127 | 54 | Fahliyan |  | Completed |  |  | _ | _ |  |
| AZNS-128 | 42 | Sarvak | Sv.3.4-6 | Producer | 15 | 240 | 0.21 | 342 |  |
| AZNS-129 | 12 | Sarvak | Sv.8 | Close |  |  | 0.25 | 129 |  |
| AZNS-130 | 81 | Gadvan | Gd.5a | Close | 32 |  | 1.21 | _ | deliver to PEDEC for work over well 116 on 1403/02/11 |
| AZNS-131 | 32 | Sarvak | Sv.3.4-6 | Close |  |  | 0.26 | _ |  |
| AZNS-132 | 73 | Sarvak | Sv.3 | Close |  |  | 0.25 | _ |  |
| AZNS-133 | 76 | Gadvan | Gd.5a.5c | Producer | 30 | 1100 | 1.2 | 2488 | well opened at 13:50 on 1403/02/11 |
| AZNS-134 | 60 | Sarvak | Sv.3 | Producer | 15 | 240 | 1.02 | 346 |  |
| AZNS-135 | 49 | Sarvak | Sv.3.4-6 | Producer | 15 | 250 | 0.29 | 319 |  |
| AZNS-136 | 42 | Sarvak | Sv.3 | Producer | 24 | 300 | 0.34 | 906 |  |
| AZNS-137 | 68 | Sarvak | Sv.3 | Producer | 20 | 220 | 0.37 | 535 |  |
| AZNS-138 | 66 | Fahliyan | Fa.1c.2a | Producer | 32 | 1800 | 3.8 | 3047 | well closed due to pressure drop at 15:30. in 1403/07/05 |
| AZNS-139 | 32 | Sarvak | Sv.3.4-6 | Producer | 20 | 300 | 0.22 | 118 |  |
| AZNS-140 | 57 | Sarvak | Sv.3.4-6 | Producer | 15 | 220 | 1.21 | 312 |  |
| AZNS-141 | 54 | Sarvak | Sv.3.4-6 | Close |  |  | 0.24 | _ |  |
| AZNS-142 | 34 | Sarvak | Sv.3.4-6 | Producer | 24 | 200 | 0.37 | 791 |  |
| AZNS-143 | 36 | Sarvak | Sv.3 | Producer | 28 | 240 | 0.62 | 1206 |  |
| AZNS-144 | 43 | Sarvak | Sv.3.4-6 | Producer | 24 | 280 | 0.27 | 811 |  |
| AZNS-145 | 40 | Sarvak | Sv.3.4-6 | Producer | 30 | 380 | 0.55 | 1622 |  |
| AZNS-146 | 70 | Sarvak | Sv.3.4-6 | Close |  |  | 0.51 | _ |  |
| AZNS-147 | 71 | Sarvak | Sv.3 | Close |  |  | 0.23 | _ |  |
| AZNS-148 | 77 | Fahliyan | Fa.2a.3a | Producer | 28 | 1500 | 2.24 | 2207 | Crack operation done. Well closed at 13:00 due to change of 3sv needle and opened at 14:00 in 1403/05/03 |
| AZNS-149 | 69 | Asmari (Water Disposal) |  | Completed |  |  | _ | _ |  |
| AZNS-150 | 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 210 | 0.44 | 1026 |  |
| AZNS-151 | 34 | Sarvak | Sv.3 | Producer | 24 | 220 | 0.38 | 827 |  |
| AZNS-152 | 39 | Sarvak | Sv.3.4-6 | Producer | 24 | 450 | 0.39 | 1348 |  |
| AZNS-153 | 60 | Sarvak | Sv.3 | Producer | 16 | 200 | 0.56 | 341 |  |
| AZNS-154 | 70 | Sarvak | Sv.3 | Producer | 28 | 100 | 1.89 | 561 |  |
| AZNS-155 | 71 | Sarvak | Sv.3 | Producer | 20 | 200 | 0.32 | 405 |  |
| AZNS-156 | 54 | Sarvak | Sv.3 | Close |  |  | 0.44 | _ |  |
| AZNS-157 | 38 | Sarvak | Sv.3 | Producer | 26 | 200 | 0.42 | 875 |  |
| AZNS-158 | 73 | Sarvak | Sv.3 | Producer | 15 | 250 | 1.65 | 319 |  |
| AZNS-159 | 76 | Sarvak |  | Completed |  |  | _ | _ |  |
| AZNS-160 | 22 | Gadvan | Gd.3 | Producer | 32 | 570 | 1.95 | 1550 | well open at 17:00 in 1403/04/03 |
| AZNS-161 | 68 | Sarvak | Sv.3 | Producer | 20 | 200 | 0.45 | 484 |  |
| AZNS-162 | 65 | Sarvak | Sv.3 | Producer | 30 | 300 | 0.88 | 1522 |  |
| AZNS-163 | 28 | Sarvak | Sv.3.4-6 | Producer | 32 | 550 | 0.67 | 2308 |  |
| AZNS-164 | 29 | Sarvak | Sv.3.4-6 | Close |  |  | 0.26 | _ |  |
| AZNS-165 | 38 | Sarvak |  | Completed |  |  | _ | _ |  |
| AZNS-166 | 9 | Kazhdumi | Kz.2-5 | Producer | 32 | 1490 | 0.23 | 3193 | due to the leakage in the 32 inch line manifold (2_3) wells were closed from west side of Karun and directed to the North gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-167 | 65 | Sarvak | Sv.3 | Producer | 24 | 250 | 0.5 | 850 |  |
| AZNS-168 | 30 | Sarvak | Sv.3.4-6 | Producer | 24 | 340 | 0.72 | 1037 |  |
| AZNS-169 | 28 | Sarvak | Sv.3.4-6 | Producer | 28 | 200 | 0.15 | 891 |  |
| AZNS-170 | 29 | Sarvak | Sv.3 | Producer | 24 | 500 | 1.07 | 1481 |  |
| AZNS-171 | 41 | Sarvak | Sv.3 | Producer | 26 | 290 | 0.32 | 1008 |  |
| AZNS-172 | 22 | Sarvak | Sv.8 | Producer | 30 | 180 | 0.81 | 945 |  |
| AZNS-173 | 72 | Gadvan |  | Completed |  |  | _ | _ |  |
| AZNS-174 | 59 | Fahliyan |  | Suspended |  |  | _ | _ |  |
| AZNS-175 | 92 | Fahliyan |  | Ongoing |  |  | _ | _ |  |
| AZNS-176 | 30 | Sarvak | Sv.3.4-6 | Producer | 20 | 240 | 0.42 | 533 |  |
| AZNS-177 | 39 | Sarvak | Sv.3.4-6 | Producer | 24 | 450 | 0.37 | 1277 |  |
| AZNS-178 | 43 | Sarvak | Sv.3 | Producer | 24 | 300 | 0.31 | 938 |  |
| AZNS-179 | 78 | Asmari (Water Disposal) |  | Ongoing |  |  | _ | _ |  |
| AZNS-180 | 72 | Sarvak |  | Completed |  |  | _ | _ |  |
| AZNS-181 | 3 | Sarvak | Sv.8 | Producer | 24 | 450 | 0.05 | 538 |  |
| AZNS-182 | 12 | Sarvak | Sv.8 | Producer | 20 | 300 | 0.35 | 489 |  |
| AZNS-183 | 59 | Sarvak |  | Completed |  |  | _ | _ |  |
| AZNS-184 | 9 | Sarvak | Sv.3.4-6 | Producer | 24 | 230 | 0.27 | 563 |  |
| AZNS-185 | 37 | Sarvak | Sv.3.4-6 | Producer | 28 | 450 | 0.17 | 1958 |  |
| AZNS-186 | 3 | Sarvak | Sv.8 | Producer | 20 | 220 | 0.02 | 551 |  |
| AZNS-187 | 40 | Sarvak | Sv.3.4-6 | Producer | 24 | 280 | 0.29 | 838 |  |
| AZNS-188 | 91 | Sarvak | Sv.8 | Producer | 24 | 200 | 0.46 | 683 |  |
| AZNS-189 | 24 | Sarvak | Sv.3.4-6 | Producer | 28 | 400 | 0.63 | 1490 |  |
| AZNS-190 | 19 | Sarvak | Sv.3.4-6 | Producer | 20 | 300 | 0.38 | 665 |  |
| AZNS-191 | 33 | Sarvak | Sv.3 | Producer | 20 | 230 | 0.36 | 651 |  |
| AZNS-192 | 9 | Sarvak | Sv.4-6 | Producer | 24 | 240 | 0.28 | 592 |  |
| AZNS-193 | 14 | Sarvak |  | Suspended |  |  | _ | _ |  |
| AZNS-194 | 37 | Sarvak | Sv.3 | Producer | 28 | 300 | 0.16 | 1458 |  |
| AZNS-195 | 64 | Fahliyan |  | Completed |  |  | _ | _ |  |
| AZNS-196 | 61 | Fahliyan |  | Ongoing |  |  | _ | _ |  |
| AZNS-197 | 2 | Kazhdumi | Kz.4 | Producer | 28 | 300 | 0.43 | 667 | well opened at00:20 in 1403/03/10 |
| AZNS-198 | 82 | Sarvak |  | Completed |  |  | _ | _ |  |
| AZNS-199 | 24 | Sarvak | Sv.3.4-6 | Producer | 16 | 200 | 0.21 | 331 |  |
| AZNS-200 | 94 | Fahliyan | 5b | Ongoing |  |  | _ | _ |  |
| AZNS-201 | 74 | Gadvan | Gd.5a.5c | Ongoing |  |  | _ | _ |  |
| AZNS-202 | 87 | Asmari (Water Disposal) |  | Completed |  |  | _ | _ |  |
| AZNS-203 | 72 | Sarvak | Sar-3 | Completed |  |  | _ | _ |  |
| AZNS-204 | 79 | Gadvan | Gd.8 | Suspended |  |  | _ | _ |  |
| AZNS-205 | 86 | Asmari (Water Disposal) |  | Completed |  |  | _ | _ |  |
| AZNS-206 | 82 | Gadvan | Gd.6 | Completed |  |  | _ | _ |  |
| AZNS-207 | 93 | Fahliyan |  | Ongoing |  |  | _ | _ |  |

Figure ‎3-4: Cumulative total producer wells and active wells of south Azadegan oil field reservoirs (up to November 2024)

Figure ‎3-5: Cumulative Production for South Azadegan field (up to November 2024)

Figure ‎3-6: Cumulative Production for Sarvak Reservoir of South Azadegan (up to November 2024)

Figure ‎3-7: Cumulative Production for Kazhdumi Reservoir of South Azadegan (up to November 2024)

Figure ‎3-8: Cumulative Production for Gadvan Reservoir of South Azadegan (up to November 2024)

Figure ‎3-9: Cumulative Production for Fahliyan Reservoir of South Azadegan (up to November 2024)

Figure ‎3-10: Cumulative oil production Bubble map and top 10 producer wells of South Azadegan Field (up to November 2024)

Figure ‎3-11: Cumulative oil production Bubble map and top 10 producer wells of Sarvak reservoir (up to November 2024)

Table ‎3-8: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to November 2024)

| Top 10 producer wells of Sarvak |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Remark |
| AZN-006 | Near 79 | Sarvak | Sv.4-6 | Producer | 36 | 350 | 1658 | well opened at 00:10 on 1403/03/10 |
| AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 34 | 200 | 940 | well opened at00:50 in 1403/03/10 |
| AZN-019 | 13 | Sarvak | Sv.3 | Close | 24 | _ | _ | closed due to pressure drop on 1400/07/12 |
| AZN-018 | 2 | Sarvak | Sv.3.4-6 | Close | 30 | _ | _ | closed at 15:00 due to pressure drop in 1403/03/27 |
| AZN-015 | Near 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 120 | 502 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-002 | Near 36 | Sarvak | Sv.3.4-6 | Producer | 32 | 180 | 901 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-005 | Near 1 | Sarvak | Sv.3.4-6 | Producer | 24 | 160 | 408 | well opened at 1:30 in 1403/03/10 |
| AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 28 | 300 | 975 | Well opened at 13:30 in 1403/06/04 |
| AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 28 | 300 | 905 | well diverted to MPFM due to well test operation at 17:00 with chock size 28/64". In 1403/07/03 |
| AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 100 | 272 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |

Figure ‎3-12: Cumulative oil production Bubble map and top 10 producer wells of Kazhdumi reservoir (up to November 2024)

Table ‎3-9: Cumulative oil production top 10 producer wells of Kazdhumi reservoir (up to November 2024)

| Top 10 producer wells of Kazhdumi |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Cum oil (MSTB) | Remark |
| AZN-017 | 50 | Kazhdumi | Kz.2 | 20 | 1440 | 1325 | 13857.98 | 10' line( well was closed from 11:00 to 16:00 due to MPFM connection) in 1403/08/12 |
| AZN-004 | Near 56 | Kazhdumi | Kz.1.2 | 32 | 1420 | 2766 | 9646.63 | the well was closed and opened for spool replacement on 1402/12/14 |
| AZN-021 | 15 | Kazhdumi | Kz.2 | 24 | 0 | _ | 5361.05 | closed due to pressure drop due to water production on 1399/02/11 |
| AZNS-092 | 19 | Kazhdumi | Kz.2.4 | 32 | 1020 | 2388 | 5193.23 | well opened at00:50 in 1403/03/10 |
| AZNS-041 | 20 | Kazhdumi | Kz.2.4 | 28 | 0 | _ | 1057.50 | well closed due to asphaltene barrier in the well on 1402/10/11 |
| AZNS-064 | 13 | Kazhdumi | Kz.2.4 | 28 | 0 | _ | 538.27 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| AZNS-028 | 11 | Kazhdumi | Kz.2.3.4 | 24 | 0 | _ | 526.36 | closed due to asphaltene barrier on 1398/08/08 |
| AZNS-029 | 5 | Kazhdumi | Kz.3.4 | 28 | 190 | 375 | 436.60 | well closed at 11:00 due to disconnect of MPFM and opened at 17:30 in 1403/07/01 |
| AZNS-197 | 2 | Kazhdumi | Kz.4 | 28 | 300 | 667 | 426.22 | well opened at00:20 in 1403/03/10 |

Figure ‎3-13: Cumulative oil production Bubble map and top 10 producer wells of Gadvan reservoir (up to November 2024)

Table ‎3-10: Cumulative oil production top 10 producer wells of Gadvan reservoir (up to November 2024)

| Top 10 producer wells of Gadvan |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (PSI) | Oil Rate (STBD) | Cum oil (MSTB) | Remark |
| AZN-009 | 16 | Gadvan | Gd.5c | Producer | 50 | 900 | 3855 | 28837.29 | well closed due to fault WK at 02:55 and opened at 13:30 on 1402/09/05 |
| AZN-023 | Near 75 | Gadvan | Gd.5a.5c | Producer | 34 | 1200 | 2953 | 16111.16 | well closed due to fault WK at 03:10 and opened at 14:50 on 1402/09/05 |
| AZN-001 | Near 19 | Gadvan | Gd.5a.5c | Producer | 36 | 500 | 1522 | 14141.93 | well opened at 13:30 on 1403/03/21 |
| AZN-012 | Near 75 | Gadvan | Gd.5a.5c | Suspended | 43 | _ | _ | 8122.37 | establishment work over rig in program by PEDEC on 1400/04/24 |
| AZNS-026 | 23 | Gadvan | Gd.1-6 | Producer | 32 | 600 | 1256 | 5701.64 | 10" line. Well opened at 21:20 in 1403/06/03 |
| AZN-020 | 7 | Gadvan | Gd.5c | Producer | 24 | 50 | 99 | 5249.11 | well reopened at 16:40. in 1403/06/02 |
| AZNS-071 | 17 | Gadvan | Gd.5c | Producer | 28 | 550 | 943 | 3259.46 | well closed from08:00 to 10:30 due to problem in control panel in 1403/08/06 |
| AZNS-086 | 1 | Gadvan | Gd.5c | Producer | 36 | 700 | 1780 | 2631.87 | due to the leakage in the 32 inch line manifold (2_3) wells were closed from west side of Karun and directed to the North gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-047 | 77 | Gadvan | Gd.5a.5c | Producer | 30 | 1400 | 3017 | 2621.99 | well opened at 14:30 on 1403/02/11 |
| AZNS-058 | 18 | Gadvan | Gd.5c | Producer | 32 | 700 | 1384 | 2481.01 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |

Figure ‎3-14: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to November 2024)

Table ‎3-11: Cumulative oil production top 10 producer wells of Fahliyan reservoir (up to November 2024)

| Top 10 producer wells of Fahliyan |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Cum oil (MSTB) | Remark |
| AZN-011 | Near 66 | Fahliyan | Fa.3a | Producer | 34 | 1200 | 2878 | 9976.65 | well closed at 05:45 due to fault of panel and opened at 10:10 on 1403/02/04 |
| AZNS-049 | 69 | Fahliyan | Fa.1a.1c.3a | Producer | 32 | 1500 | 2706 | 6483.11 | closed at 12:35 pm by order of management in 1403/06/12 |
| AZNS-070 | 75 | Fahliyan | Fa.2a.3a | Producer | 30 | 1000 | 1865 | 6301.9 | well opened at 17:30 in 1403/09/02 |
| AZNS-138 | 66 | Fahliyan | Fa.1c.2a | Producer | 32 | 1800 | 3047 | 3786.48 | Well closed due to pressure drop at15:30. in 1403/07/05 |
| AZNS-046 | 62 | Fahliyan | Fa.1.3a | Producer | 49 | 1230 | 5392 | 3689.84 | the well was closed from 10:15 to 13:15 to clean the stone trap in 1403/05/01 |
| AZNS-110 | 58 | Fahliyan | Fa.4a | Producer | 30 | 900 | 1971 | 2525.07 | well closed at 00:35 due to leakage of 20" line and opened at 04:10 in 1403/08/11 |
| AZNS-121 | 44 | Fahliyan | Fa.1 | Producer | 34 | 2200 | 4081 | 2325.14 | Well opened at 1:20 in 1403/03/29 |
| AZNS-148 | 77 | Fahliyan | Fa.2a.3a | Producer | 28 | 1500 | 2207 | 2237.01 | crack operation done. Well closed at 13:00 due to change of 3sv needle and opened at 14:00 in 1403/05/03 |
| AZNS-050 | 57 | Fahliyan | Fa.4a | Producer | 28 | 900 | 1726 | 2080.97 | well closed at 00:30 due to leakage of 20" line and opened at 04:15 in 1403/08/11 |
| AZNS-060 | 78 | Fahliyan | Fa.4a | Close | 20 | 280 | 188 | 1881.83 | well closed at 11:00 due to fishing operation in 1403/08/11 |