گزارش ماهیانه بروزرسانی داده ها در نرم افزار OFM

Update Current OFM Database and Report Monthly

South Azadegan Oil Field

(January 2025)

کارفرما:

شرکت پترو پارس با مسئولیت محدود

مشاور:

شرکت خدمات مهندسی نفت کیش (KPE)

| 00 | February-2025 | Foad Saberi | Mehdi Pirayeh | Akbar Fotouhi |
| --- | --- | --- | --- | --- |
| Rev | Date | Prepared | Checked | Approved |

# Table of Contents

Title											        Page No.

Table of Contents	2

List of Figures	3

List of tables	4

Summary	6

1	OFM Project	6

2	Updated OFM Model	11

3	Updated OFM results	11

# List of Figures

Title											        Page No.

Figure ‎1-1: OFM Schema Tables	7

Figure ‎1-2: OFM base map	8

Figure ‎3-1: Cumulative oil production Comparison of South Azadegan oil field reservoirs	12

Figure ‎3-2: Well Status of South Azadegan Wells (Reservoir by Reservoir) (up to January 2025)	14

Figure ‎3-3: Cumulative total producer wells and active wells of south Azadegan oil field reservoirs (up to January 2025)	53

Figure ‎3-4: Cumulative Production for South Azadegan field (up to January 2025)	54

Figure ‎3-5: Cumulative Production for Sarvak Reservoir of South Azadegan (up to January 2025)	55

Figure ‎3-6: Cumulative Production for Kazhdumi Reservoir of South Azadegan (up to January 2025)	56

Figure ‎3-7: Cumulative Production for Gadvan Reservoir of South Azadegan (up to January 2025)	57

Figure ‎3-8: Cumulative Production for Fahliyan Reservoir of South Azadegan (up to January 2025)	58

Figure ‎3-9: Cumulative oil production Bubble map and top 10 producer wells of South Azadegan Field (up to January 2025)	59

Figure ‎3-10: Cumulative oil production Bubble map and top 10 producer wells of Sarvak reservoir (up to January 2025)

Figure ‎3-11: Cumulative oil production Bubble map and top 10 producer wells of Kazhdumi reservoir (up to January 2025)	62

Figure ‎3-12: Cumulative oil production Bubble map and top 10 producer wells of Gadvan reservoir (up to January 2025)

64

Figure ‎3-13: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to January 2025)	66

# List of tables

Title											        Page No.

Table ‎3-1: Well status classification of existed wells of South Azadegan Field in OFM (up to January 2025)	13

Table ‎3-2: South Azadegan Wells which their status were suspended in January 2025	15

Table ‎3-3: South Azadegan Wells which their status were Ongoing in January 2025	16

Table ‎3-4: South Azadegan Wells which their status were completed in January 2025	17

Table ‎3-5: South Azadegan Wells which their status were closed in January 2025	18

Table ‎3-6: Wells with low THP (means: upstream THP-downstream THP<50 psi)	19

Table ‎3-7: South Azadegan wells status, oil production rate and cumulative	22

Table ‎3-8: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to January 2025)

Table ‎3-9: Cumulative oil production top 10 producer wells of Kazdhumi reservoir (up to January 2025)	63

Table ‎3-10: Cumulative oil production top 10 producer wells of Gadvan reservoir (up to January 2025)	65

Table ‎3-11: Cumulative oil production top 10 producer wells of Fahliyan reservoir (up to January 2025)	67

List of abbreviation

| Description | Abbreviation |
| --- | --- |
| Schlumberger Oil Field Manager | OFM |
| Total Depth | TD |
| Rotary Table Elevation | RTE |
| Gas Oil Ratio | GOR |
| Basic sediment and water | BSW |
| American Petroleum Institute gravity | API |
| Drill Stem Test | DST |
| Modular Formation Dynamics Tester | MDT |
| Wireline Formation Test | WFT |

# Summary

Following report includes the description of updated South Azadegan oil field OFM up to January 2025. The daily production table of OFM updated. Based on these new production data, this field produces around 151 MSTB/d in this month. The total number of producer wells of this field is 175 wells, which 143 wells are active on average in this month. The total cumulative oil production of the field is around 361.1MMSTB until the end of this month (the field produced 4.7 MMSTB oil during this month). It should be noted that other new data related to recent operations are added to OFM. These updates are descripted in Section 3 in detail.

# OFM Project

The current OFM project contains all related data of south Azadegan oil field (and Also south Yaran field). All received data for each well are given in their related tables. 207 completions (202 wellbores) in South Azadegan field (up to well AZNS-206) and 18 completions in South Yaran field are defined in this database. The Schema Tables related to this OFM project is depicted in Figure ‎1-1. Also, the base map of the field is shown in Figure ‎1-2.

Figure ‎1-1: OFM Schema Tables

Figure ‎1-2: OFM base map

The database of OFM model for South Azadegan field contains following tables:

## Header ID:

Header ID table of Azadegan OFM model defines the Azadegan wells and include information of drilling, production and geology of Azadegan field wells (TD, production interval, production formation, RTE, PVT and etc.).

## Daily Production

The daily production table, contains the daily production data of south Azadegan wells contains daily production rates (Oil, gas, water), chock size, GOR, BSW%, Wellhead pressure, downstream pressure, salt content, API, H2S content for the wells of south Azadegan and south Yaran fields.

## Completion

This table contains perforation depths intervals and existed plug (if any).

## H2S

In this table, the H2S content for those wells that the H2S are recorded and their depth intervals are given.

## Coring

The coring interval, recovery percent are given in this table.

## DST

In this table, all the information related to DST tests includes depth intervals, Formation and zone, test duration, production rate (oil, gas, and, water rates), ratios (GOR, BSW), chock size, gauge depth and gauge pressure and etc. are given.

## Production Tests

This table contains the information related to Production tests includes depth intervals, Formation and zone, test duration, production rate (oil, gas, and, water rates), ratios (GOR, BSW), choke size, gauge depth and gauge pressure, separator pressure and temperature, before and after choke pressure and choke temperature and, wellhead pressure and temperature are given.

## PLT

The PLT data includes oil and water rates, oil contributions and their depth intervals are given in this table.

## Formation Water

The results of water analysis include depth intervals, color, PH, salinity, and the chemical element percentages for those wells that have a water sample are gathered in this table.

## Static Pressure Tests

The results of the static surveys contain pressure gradient, pressure at datum and, pressure at the mid perforation could be found in this table.

## Flowing Pressure Tests

The results of the flowing surveys contain pressure gradient, pressure at datum and, pressure at the mid perforation could be found in this table.

## Stimulation

In the stimulation table, all data related to stimulation jobs (acidizing and clean-up), total volume of injected acid, injected acid per foot, depth intervals, main acid recipe, and, their PI values before and after acidizing) are given.

## WFT

The data related to wireline formation tests such as WFT type, gauge type, formation pressure and mobility (for MDT) are given.

# Updated OFM Model

- Production data of January 2025 is added to database.

# Updated OFM results

In this section, the summary of production data up to January 2025 related to south Azadegan reservoirs presented in Figure ‎3-1. 207 completions (202 wellbores) in South Azadegan field (up to well AZNS-206) and 12completions in South Yaran field are defined in this database. A classification related to well status of existed wells of South Azadegan field up to this month are listed in Table ‎3-1 and Depicted in Figure ‎3-2. This field produces around 151 MSTB/d with an average of 143 active wells in this month. The total number of producer wells is 175 wells. Figure ‎3-3 shows the total producer wells and active wells of south Azadegan oil field during the production history. The cumulative produced oil of the field up to this month is around 361.1 MMSTB (the field produces 4.7 MMSTB oil during this month). The cumulative production profile of field (and also for each reservoir) and bubble map of cumulative oil production for producer wells of south Azadegan field along with the list of top ten producing wells (for field and each reservoir individually)  are shown in Figure ‎3-4 to respectively.

| Reservoir; Cum Oil (MMSTB); Cum Gas (Bscf); Cum Water (MSTB). Sarvak; 177.5; 62.7; 1479.1. Kazhdumi; 37.5; 30.2; 2275.3. Gadvan; 100.0; 74.7; 572.1. Fahliyan; 46.1; 58.0; 182.7. Total; 361.1; 225.5; 4509.1. |  |
| --- | --- |

Figure ‎3-1: Cumulative oil production Comparison of South Azadegan oil field reservoirs

Table ‎3-1: Well status classification of existed wells of South Azadegan Field in OFM (up to January 2025)

| Well Status | No. of Wellbore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reservoir by Reservoir |  |  |  |  | Field |
|  | Sarvak | Kazhdumi | Gadvan | Fahliyan | Asmari(Disposal) | South Azadegan |
| Oil producer | 135 | 10 | 16 | 14 |  | 175 |
| Completed | 4 |  | 1 | 3 | 4 | 12 |
| On-going |  |  | 1 | 5 | 1 | 7 |
| Observation |  |  |  | 1 |  | 1 |
| Suspended | 1 |  | 4 | 3 |  | 8 |
| Total | 140 | 10 | 22 | 26 | 5 | 203 |

Figure ‎3-2: Well Status of South Azadegan Wells (Reservoir by Reservoir) (up to January 2025)

In the South Azadegan field, the following wells have been suspended: AZN-12, AZNS-51, AZNS-52, AZNS-75, AZNS-113, AZNS-174, AZNS-193, and AZNS-204. These wells are summarized in Table ‎3-2.

Table ‎3-2: South Azadegan Wells which their status were suspended in January 2025

| Item | Well | Formation | Pad | Remark |
| --- | --- | --- | --- | --- |
| 1 | AZN-012 | Gadvan | Near 75 | Planned for Workover by MSA |
| 2 | AZNS-051 | Gadvan | 14 | Planned for Workover by TDDC due to completion failure |
| 3 | AZNS-052 | Fahliyan | 63 | Planned for completion by TDDC |
| 4 | AZNS-075 | Gadvan | 80 | Planned for Workover by MSA |
| 5 | AZNS-113 | Fahliyan | 82 | Planned for Plug-Prong retrieval by NIDC |
| 6 | AZNS-174 | Fahliyan | 59 | Planned for Workover by NIDC due to completion failure |
| 7 | AZNS-193 | Sarvak | 14 | Planned for Workover by TDDC due to completion failure |
| 8 | AZNS-204 | Gadvan | 79 | Planned for completion by TDDC |

The following wells are classified in the ongoing operation category: AZNS-122, AZNS-175, AZNS-179, AZNS-196, AZNS-200, AZNS-201 and AZNS-207. These wells are summarized in Table ‎3-3.

Table ‎3-3: South Azadegan Wells which their status were Ongoing in January 2025

| Item | Well | Formation | Pad | Remark |
| --- | --- | --- | --- | --- |
| 1 | AZNS-122 | Fahliyan | 55 | Side Tracking Preparing Operation |
| 2 | AZNS-175 | Fahliyan | 92 | Decompletion-Recompletion-Acidizing |
| 3 | AZNS-179 | Asmari (Water Disposal) | 85 | Drilling 9 5/8 Section till ) |
| 4 | AZNS-196 | Fahliyan | 61 | Perforation Operation |
| 5 | AZNS-200 | Fahliyan | 94 | well flowing(clean-up) |
| 6 | AZNS-201 | Gadvan | 74 | Decompletion-Recompletion-Acidizing |
| 7 | AZNS-207 | Fahliyan | 93 | Drilling to 3850 m MD |

In the South Azadegan field, the status of the following wells were completed in January 2025 which are summarized in Table ‎3-4.

Table ‎3-4: South Azadegan Wells which their status were completed in January 2025

| Item | Well | Pad | Formation | Well Status | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | AZNS-066 | 84 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 2 | AZNS-085 | 79 | Fahliyan | Completed | Waiting for production line and surface facilities installation |
| 3 | AZNS-116 | 81 | Fahliyan | Completed | Acidizing finished, Waiting for commissioning |
| 4 | AZNS-149 | 69 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 5 | AZNS-159 | 76 | Sarvak | Completed | Planned for ESP installation |
| 6 | AZNS-165 | 38 | Sarvak | Completed | Planned for Work-over or ESP installation |
| 7 | AZNS-183 | 59 | Sarvak | Completed | Planned for Acidizing by NIDC |
| 8 | AZNS-195 | 64 | Fahliyan | Completed | Planned for Acidizing by OEOC |
| 9 | AZNS-198 | 82 | Sarvak | Completed | Planned for Acidizing by NIDC |
| 10 | AZNS-202 | 87 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 11 | AZNS-205 | 86 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 12 | AZNS-206 | 82 | Gadvan | Completed | Waiting for Acidizing By NIDC |

In the South Azadegan field, the status of the following wells were closed in January 2025 which are summarized in Table ‎3-5.

Table ‎3-5: South Azadegan Wells which their status were closed in January 2025

| Item | Well Name | Formation | Zone | Pad Name | Closed Date | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | AZN-014 | Sarvak | Sv.3.4-6 | NEAR 73 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 2 | AZN-018 | Sarvak | Sv.3.4-6 | Pad02 | 16-06-24 | Wellhead pressure drop. The well pressure dropped suddenly at the end of May. 2025 |
| 3 | AZN-019 | Sarvak | Sv.3 | Pad13 | 04-10-21 | It probably closed due to water in the wellbore and pressure drop |
| 4 | AZN-020 | Gadvan | Gd.5c | Pad7 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 5 | AZNS-027 | Sarvak | Sv.3 | Pad4 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 6 | AZNS-043 | Sarvak | Sv.3.4-6 | Pad11 | 05-03-24 | Wellhead pressure drop |
| 7 | AZNS-061 | Sarvak | Sv.3 | Pad77 | 26-05-24 | well closed due to ssv packing leakage at 10:40 and reopend at 14:45 after fix problom. In 1403/05/10 |
| 8 | AZNS-073 | Sarvak | Sv.3.4-6 | Pad05 | 25-05-22 | Wellhead pressure drop |
| 9 | AZNS-077 | Sarvak | Sv.3.4-6 | Pad13 | 31-12-22 | Wellhead pressure drop |
| 10 | AZNS-078 | Sarvak | Sv.3.4-6 | Pad63 | 14-05-22 | Delivered to PEDEC for workover operation on pad, however based on production data the last WHP was low |
| 11 | AZNS-081 | Sarvak | Sv.3.4-6 | Pad18 | 31-12-22 | Wellhead pressure drop, water production also reported, WHP drop could be due to water production |
| 12 | AZNS-088 | Sarvak | Sv.3 | Pad80 | 13-03-24 | Wellhead pressure drop |
| 13 | AZNS-100 | Sarvak | Sv..4-6 | Pad 2 | 07-01-25 | closed due to pressure drop in 1403/10/19 |
| 14 | AZNS-106 | Sarvak | Sv.3 | Pad81 | 29-04-24 | Delivered to PEDEC for Workover in other well on pad (AZMS-116) |
| 15 | AZNS-115 | Sarvak | Sv.3 | Pad 45 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 15 | AZNS-117 | Sarvak | Sv.3.4-6 | Pad53 | 25-05-22 | Wellhead pressure drop |
| 16 | AZNS-125 | Sarvak | Sv.3.4-6 | Pad53 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 17 | AZNS-129 | Sarvak | Sv.8 | Pad12 | 07-01-25 | closed due to pressure drop in 1403/10/19 |
| 18 | AZNS-131 | Sarvak | Sv.3.4-6 | Pad32 | 27-11-23 | Salty oil production |
| 19 | AZNS-132 | Sarvak | Sv.3 | Pad73 | 31-08-24 | Wellhead pressure drop |
| 20 | AZNS-146 | Sarvak | Sv.3.4-6 | Pad70 | 31-08-24 | Wellhead pressure drop |
| 21 | AZNS-147 | Sarvak | Sv.3 | Pad71 | 16-06-24 | Wellhead pressure drop |
| 22 | AZNS-155 | Sarvak | Sv.3 | Pad71 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 23 | AZNS-164 | Sarvak | Sv.3.4-6 | Pad29 | 31-08-24 | Wellhead pressure drop |
| 24 | AZNS-028 | Kazhdumi | Kz.2.3.4 | Pad 11 | 01-05-19 | closed due to asphaltene barrier on 1398/08/08 |
| 25 | AZNS-041 | Kazhdumi | Kz.2.4 | Pad 20 | 01-10-24 | well closed due to asphaltene barrier in the well on 1402/10/11 |
| 26 | AZNS-064 | Kazhdumi | Kz.2.4 | Pad13 | 05-03-20 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| 26 | AZNS-130 | Gadvan | Gd.5a | Pad 81 | 30-04-24 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| 27 | AZN-021 | Kazhdumi | Kz.2 | Pad 15 | 30-04-20 | closed due to pressure drop due to water production on 1399/02/11 |

In the South Azadegan field, the status of the following wells with low THP in January 2025 which are summarized in Table ‎3-7.

Table ‎3-6: Wells with low THP (means: upstream THP-downstream THP<50 psi)

| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (psi) | Down Stream (psi) | Cum (First prod--Last prod) MMSTB | Oil Rate (STBD) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AZN-014 | NEAR 73 | Sarvak | Sv.3.4-6 | Producer | 20 | 120 | 70 | 3.57 | 259 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 2 | AZN-015 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 110 | 70 | 6.04 | 502 | 12" line( well was closed due to pressure drop from 00:30 to 06:00) in 1403/09/29 |
| 3 | AZN-020 | 7 | Gadvan | Gd.5c | Producer | 24 | 50 | 50 | 5.25 | 99 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 4 | AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 120 | 80 | 2.62 | 272 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 5 | AZNS-027 | 4 | Sarvak | Sv.3 | Producer | 15 | 70 | 50 | 1.09 | _ | well opened at 11:00 in 1403/08/10 |
| 6 | AZNS-036 | 50 | Sarvak | Sv.3 | Producer | 28 | 220 | 180 | 1.95 | 931 | Closed at11:20 due to air pressure drop & opened at 15:15 in 1403/05/05 |
| 7 | AZNS-057 | 25 | Sarvak | Sv.3.4-6 | Producer | 22 | 190 | 145 | 0.81 | 565 | well open at 02:30 in 1403/04/03 |
| 8 | AZNS-062 | 56 | Sarvak | Sv.3.4-6 | Producer | 23 | 220 | 180 | 1.05 | 689 | Well opened at 1:30 in 1403/03/29 |
| 9 | AZNS-065 | 69 | Sarvak | Sv.3 | Producer | 15 | 150 | 130 | 0.49 | _ | well opened at 18:00 witch chock size 15/64''. In 1403/08/13 |
| 10 | AZNS-074 | 83 | Sarvak | Sv.3 | Producer | 15 | 220 | 180 | 0.92 | 282 | well opened at 14:00 on 1403/02/11 |
| 11 | AZNS-084 | 47 | Sarvak | Sv.3.4-6 | Producer | 15 | 220 | 180 | 0.97 | 301 | well opened at 11:30 in 1403/07/01 |
| 12 | AZNS-091 | 83 | Sarvak | Sv.3 | Producer | 24 | 240 | 190 | 1.34 | 702 | well opened at 13:30 on 1403/02/11 |
| 13 | AZNS-093 | 93 | Sarvak | Sv.3.4-6 | Producer | 15 | 220 | 180 | 0.42 | 279 | Well opened at 2 in 1403/03/29 |
| 14 | AZNS-099 | 91 | Sarvak | Sv.8 | Producer | 20 | 170 | 170 | 1.02 | 281 | well opened at 13:30 in 1403/10/18 |
| 15 | AZNS-100 | 1 | Sarvak | Sv.4-6 | Producer | 15 | 220 | 180 | 0.25 | 243 | closed due to pressure drop in 1403/10/19 |
| 16 | AZNS-107 | 19 | Sarvak | Sv.3.4-6 | Producer | 26 | 150 | 150 | 1.69 | 214 | well opened at 15:00. In 1403/09/06 |
| 17 | AZNS-108 | 48 | Sarvak | Sv.3.4-6 | Producer | 30 | 220 | 180 | 0.84 | 1069 | Well opened at 2 in 1403/03/29 |
| 18 | AZNS-109 | 89 | Sarvak | Sv.8 | Producer | 15 | 220 | 180 | 0.44 | 241 | well reopened at 15:30 after finished slickline operation. In 1403/09/15 |
| 19 | AZNS-115 | 45 | Sarvak | Sv.3 | Producer | 15 | 200 | 180 | 0.33 | 272 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 20 | AZNS-125 | 53 | Sarvak | Sv.3.4-6 | Producer | 15 | 200 | 160 | 0.43 | 264 | Well opened at 2:40 in 1403/03/29 |
| 21 | AZNS-126 | 45 | Sarvak | Sv.3.4-6 | Producer | 20 | 220 | 180 | 0.42 | 505 | Well opened at 2:40 in 1403/03/29 |
| 22 | AZNS-129 | 12 | Sarvak | Sv.8 | Producer | 15 | 200 | 160 | 0.25 | 129 | opened at 12:10 in 1403/08/16 |
| 23 | AZNS-150 | 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 190 | 160 | 0.44 | 1026 | well open at 17:00 in 1403/04/03 |
| 24 | AZNS-155 | 71 | Sarvak | Sv.3 | Producer | 20 | 160 | 160 | 0.32 | 405 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 26 | AZNS-188 | 91 | Sarvak | Sv.8 | Producer | 24 | 200 | 170 | 0.46 | 683 | well opened at 09:45. In 1403/04/15 |

Table ‎3-7: South Azadegan wells status, oil production rate and cumulative

| Well | Pad | Formation | Zone | Well Status | BS&W (%) | GOR | Chock size (1/64)" | WHP (Psi) | Down Stream (psi) | WHT (C°) | API | H2S (PPM) | Cum (First prod--Last prod) MMSTB | Oil Rate (STBD) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AZN-001 | Near 19 | Gadvan | Gd.5a.5c | Producer | 8 | 818 | 36 | Min= 450 Max= 500 Ave= 482 Mode= 480 | Min= 60 Max= 70 Ave= 66 Mode= 65 | Min= 65 Max= 73 Ave= 71 Mode= 70 | 31 | 650 | 14.24 | Min= 1188 Max= 1651 Ave= 1481 | well opened at 13:30 on 1403/03/21 |
| AZN-002 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 32 | Min= 170 Max= 190 Ave= 187 Mode= 190 | Min= 70 Max= 90 Ave= 85 Mode= 90 | Min= 48 Max= 53 Ave= 50 Mode= 51 | 22.1 | 2000 | 5.97 | Min= 731 Max= 1043 Ave= 926 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-004 | Near 56 | Kazhdumi | Kz.1.2 | Producer | 7 | 916 | 32 | Min= 1420 Max=1450 Ave= 1426 Mode= 1420 | Min= 220 Max= 230 Ave= 224 Mode= 220 | 84 | 28 | 20 | 9.82 | Min= 2171 Max= 3134 Ave= 2806 | the well was closed and opened for spool replacement on 1402/12/14 |
| AZN-005 | NEAR 1 | Sarvak | Sv.3.4-6 | Producer | 2 | 439 | 24 | Min= 40 Max= 110 Ave= 72 Mode= 50 | Min= 40 Max= 110 Ave= 72 Mode= 50 | Min= 10 Max= 24 Ave= 18 Mode= 21 | 21.4 | _ | 4.97 | Min= 121 Max= 294 Ave= 210 | well opened at 1:30 in 1403/03/10 |
| AZN-006 | NEAR 79 | Sarvak | Sv.4-6 | Producer | 0.6 | 362 | 36 | Min= 300 Max= 340 Ave= 332 Mode= 340 | Min= 90 Max= 110 Ave= 99 Mode= 100 | Min= 60 Max= 61 Ave= 60 Mode= 60 | 21.1 | _ | 13.67 | Min= 1185 Max= 1870 Ave= 1652 | well opened at 00:10 on 1403/03/10 |
| AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 0.3 | 439 | 34 | Min= 180 Max= 210 Ave= 195 Mode= 200 | Min= 85 Max= 100 Ave= 96 Mode= 100 | Min= 37 Max= 57 Ave= 55 Mode= 55 | 20.1 | _ | 10.28 | Min= 716 Max= 1050 Ave= 904 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| AZN-009 | 16 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 50 | Min= 870 Max= 930 Ave= 916 Mode= 920 | Min= 300 Max= 350 Ave= 321 Mode= 310 | Min= 77 Max= 78 Ave= 78 Mode= 78 | 32.3 | _ | 29.08 | Min= 3021 Max= 4306 Ave= 3867 | well closed dut to fault WK at 02:55 and opened at 13:30 on 1402/09/05 |
| AZN-010 | Near 50 | Fahliyan | Fa. 1a.1b.1c | Producer | _ | 1678 | 32 | Min= 2200 Max= 2300 Ave= 2231 Mode= 2200 | Min= 170 Max= 220 Ave= 180 Mode= 175 | Min= 47 Max= 78 Ave= 73 Mode= 74 | _ | _ | 0.2 | Min= 2189 Max= 3723 Ave= 3468 | well was closed due to control panel fault from9:15 to 10:25. in 1403/09/22 |
| AZN-011 | Near 66 | Fahliyan | Fa.3a | Producer | 0.1 | 1090 | 34 | Min= 1150 Max= 1300 Ave= 1195 Mode= 1200 | Min= 310 Max= 360 Ave= 338 Mode= 360 | Min= 73 Max= 78 Ave= 75 Mode= 75 | _ | _ | 10.15 | Min= 2037 Max= 3017 Ave= 2699 | well closed at 05:45 due to fault of panel and opened at 10:10 on 1403/02/04 |
| AZN-014 | NEAR 73 | Sarvak | Sv.3.4-6 | Producer | 1.2 | 362 | 20 | Min= 60 Max= 130 Ave= 108 Mode= 110 | Min= 60 Max= 70 Ave= 68 Mode= 70 | Min= 5 Max= 14 Ave= 9 Mode= 5 | 21.5 | _ | 1.72 | Min= 134 Max= 303 Ave= 225 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZN-015 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 292 | 28 | Min= 100 Max= 120 Ave= 109 Mode= 110 | Min= 60 Max= 70 Ave= 67 Mode= 70 | Min= 43 Max= 48 Ave= 45 Mode= 45 | 21.4 | 1750 | 6.07 | Min= 373 Max= 571 Ave= 468 | 12" line( well was closed due to pressure drop from 00:30 to 06:00) in 1403/09/29 |
| AZN-016 | 25 | Sarvak | Sv.3.4-6 | Producer | 3 | 439 | 20 | Min= 160 Max= 200 Ave= 182 Mode= 170 | Min= 80 Max= 120 Ave= 108 Mode= 110 | Min= 25 Max= 40 Ave= 32 Mode= 32 | 18.7 | 1000 | 2.58 | Min= 265 Max= 395 Ave= 330 | well opened at00:50 in 1403/03/10 |
| AZN-017 | 50 | Kazhdumi | Kz.2 | Producer | 2 | 916 | 20 | Min= 1420 Max= 1500 Ave= 1446 Mode= 1450 | Min= 280 Max= 320 Ave= 299 Mode= 300 | Min= 76 Max= 78 Ave= 76 Mode= 76 | 28 | _ | 13.94 | Min= 1009 Max= 1486 Ave= 1317 | 10' line( well was closed from 11:00 to 16:00 due to MPFM connection) in 1403/08/12 |
| AZN-020 | 7 | Gadvan | Gd.5c | Producer | 0.6 | 1381 | 24 | Min= 30 Max= 40 Ave= 38 Mode= No Repeat | Min= 30 Max= 40 Ave= 38 Mode= No Repeat | Min= 12 Max= 20 Ave= 14 Mode= No Repeat | 33.7 | _ | 5.25 | Min= 47 Max= 94 Ave= 78 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 1 | 292 | 20 | Min= 90 Max= 200 Ave= 116 Mode= 100 | Min= 70 Max= 170 Ave= 78 Mode= 70 | Min= 15 Max= 18 Ave= 17 Mode= 17 | _ | _ | 2.64 | Min= 211 Max= 400 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-023 | Near 75 | Gadvan | Gd.5a.5c | Producer | 1 | 818 | 34 | Min= 1120 Max= 1150 Ave= 1144 Mode= 1150 | Min= 310 Max= 370 Ave= 343 Mode= 350 | Min= 76 Max= 80 Ave= 78 Mode= 78 | 32.9 | 23 | 16.3 | Min= 2227 Max= 3246 Ave= 2910 | well closed dut to fault WK at 03:10 and opened at 14:50 on 1402/09/05 |
| AZN-024 | Near 37 | Gadvan | Gd.3.5c.7 | Producer | 9 | 818 | 28 | Min= 530 Max= 730 Ave= 556 Mode= 530 | Min= 110 Max= 130 Ave= 116 Mode= 120 | Min= 71 Max= 74 Ave= 73 Mode= 73 | 23.6 | 5 | 1.96 | Min= 795 Max= 1449 Ave= 1058 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZNS-026 | 23 | Gadvan | Gd.1-6 | Producer | 0.1 | 1381 | 32 | Min= 570 Max= 580 Ave= 578 Mode= 580 | Min= 260 Max= 270 Ave= 262 Mode= 260 | Min= 42 Max= 47 Ave= 46 Mode= 46 | 32.3 | _ | 5.78 | Min= 957 Max= 1366 Ave= 1227 | 10" line. Well opened at 21:20 in 1403/06/03 |
| AZNS-027 | 4 | Sarvak | Sv.3 | Producer | 1 | 439 | 15 | Min= 50 Max= 200 Ave= 135 | Min= 50 Max= 150 Ave= 108 | Min= 12 Max= 29 Ave= 21 | 19.7 | _ | 1.1 | Min= 70 Max= 242 Ave= 148 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZNS-029 | 5 | Kazhdumi | Kz.3.4 | Producer | 5 | 916 | 28 | Min= 180 Max= 240 Ave= 215 Mode= 210 | Min= 70 Max= 90 Ave= 80 Mode= 75 | Min= 26 Max= 32 Ave= 30 Mode= 30 | 25.9 | _ | 0.48 | Min= 387 Max= 590 Ave= 490 | well closed at 11:00 due to disconnect of MPFM and opened at 17:30 in 1403/07/01 |
| AZNS-030 | 27 | Sarvak | Sv.3 | Producer | 1 | 439 | 28 | Min= 280 Max= 360 Ave= 346 Mode= 290 | Min= 185 Max= 195 Ave= 189 Mode= 185 | Min= 46 Max= 51 Ave= 49 Mode= 50 | 20.9 | _ | 1.53 | Min= 1189 Max= 2231 Ave= 1431 | well open at 17:00 in 1403/04/03 |
| AZNS-032 | 21 | Sarvak | Sv.3 | Producer | 0.2 | 439 | 28 | Min= 230 Max= 300 Ave= 275 Mode= 280 | Min= 120 Max= 130 Ave= 128 Mode= 130 | Min= 48 Max= 50 Ave= 50 Mode= 50 | 20.5 | _ | 0.21 | Min= 1030 Max= 2249 Ave= 1299 | well closed at 10:00 due to well down leakage and opened at 17 :00in 1403/10/07 |
| AZNS-033 | 7 | Sarvak | Sv.3 | Producer | 0.1 | 439 | 28 | Min= 300 Max= 310 Ave= 302 Mode= 300 | Min= 140 Max= 160 Ave= 151 Mode= 150 | Min= 54 Max= 57 Ave= 56 Mode= 56 | 19 | _ | 2.46 | Min= 710 Max= 1035 Ave= 919 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-034 | 23 | Sarvak | Sv.3 | Producer | 0.5 | 439 | 32 | Min= 360 Max= 380 Ave= 372 Mode= 370 | Min= 360 Max= 380 Ave= 372 Mode= 370 | Min= 37 Max= 56 Ave= 42 Mode= 41 | 18.1 | _ | 2.09 | Min= 1081 Max= 1531 Ave= 1364 | 10" line.well opened at 12:05 in 1403/06/03 |
| AZNS-035 | 33 | Sarvak | Sv.3 | Producer | 0.3 | 292 | 24 | Min= 320 Max= 370 Ave= 345 Mode= 350 | Min= 140 Max= 185 Ave= 155 Mode= 150 | Min= 32 Max= 39 Ave= 35 Mode= 34 | 20.32 | _ | 0.63 | Min= 1160 Max= 2307 Ave= 1408 | well closed at 12:05 due to change of 2sv packing and opened at 13:15 in 1403/08/02 |
| AZNS-036 | 50 | Sarvak | Sv.3 | Producer | 0.5 | 292 | 28 | Min= 200 Max= 220 Ave= 217 Mode= 220 | Min= 170 Max= 195 Ave= 182 Mode= 190 | Min= 19 Max= 60 Ave= 26 Mode= 22 | 21.77 | _ | 2 | Min= 548 Max= 925 Ave= 848 | Closed at11:20 due to air pressure drop & opened at 15:15 in 1403/05/05 |
| AZNS-037 | 7 | Sarvak | Sv.3 | Producer | 0.2 | 439 | 24 | Min= 240 Max= 280 Ave= 250 Mode= 250 | Min= 130 Max= 160 Ave= 148 Mode= 150 | Min= 48 Max= 52 Ave= 50 Mode= 50 | 19.1 | _ | 2.21 | Min= 483 Max= 723 Ave= 602 | well reopened at 15:00. in 1403/07/03 |
| AZNS-038 | 21 | Sarvak | Sv.3 | Producer | 0.3 | 439 | 28 | Min= 290 Max= 450 Ave= 406 Mode= 430 | Min= 120 Max= 170 Ave= 156 Mode= 160 | Min= 34 Max= 43 Ave= 41 Mode= 41 | 20.7 | _ | 0.19 | Min= 431 Max= 1353 Ave= 1146 | the well was closed from 02:40 to 17:00 due to high salt in 1403/11/05 |
| AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 6 | 439 | 28 | Min= 330 Max= 380 Ave= 342 Mode= 340 | Min= 240 Max= 250 Ave= 248 Mode= 250 | Min= 55 Max= 57 Ave= 56 Mode= 56 | 18 | _ | 4.3 | Min= 740 Max= 1059 Ave= 957 | 10" line.well opened at 13:30 in 1403/06/04 |
| AZNS-040 | 50 | Fahliyan | Fa.1b.1c | Producer | 0.1 | 1678 | 32 | Min= 1300 Max= 1700 Ave= 1420 Mode= 1400 | Min= 170 Max= 185 Ave= 178 Mode= 180 | Min= 56 Max= 64 Ave= 60 Mode= 60 | 36.39 | _ | 0.64 | Min= 1517 Max= 2886 Ave= 2405 | closed at 12:25 pm by order of management in 1403/06/12 |
| AZNS-042 | 10 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 439 | 15 | Min= 240 Max= 270 Ave= 256 Mode= 260 | Min= 145 Max= 165 Ave= 157 Mode= 160 | Min= 12 Max= 24 Ave= 18 Mode= 18 | 20.8 | _ | 1.19 | Min= 219 Max= 309 Ave= 270 | closed due to pressure drop in 1403/10/19 |
| AZNS-044 | 15 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 15 | Min= 50 Max= 180 Ave= 111 Mode= No Repeat | Min= 50 Max= 160 Ave= 100 Mode= No Repeat | Min= 7 Max= 16 Ave= 10 Mode= No Repeat | 20.2 | _ | 0.52 | Min= 45 Max= 122 Ave= 79 | closed due to pressure drop in 1403/10/19 |
| AZNS-045 | 56 | Fahliyan | Fa.4a.4b | Producer | 0.1 | 1090 | 28 | Min= 160 Max= 900 Ave= 362 Mode= 200 | Min= 160 Max= 200 Ave= 176 Mode= 160 | Min= 10 Max= 46 Ave= 27 Mode= 10 | 35.5 | _ | 0.71 | Min= 129 Max= 1489 Ave= 869 | well open at 22:30 in 1403/11/06 |
| AZNS-046 | 62 | Fahliyan | Fa.1.3a | Producer | 0.1 | 1678 | 49 | Min= 1200 Max= 1300 Ave= 1247 Mode= 1250 | Min= 120 Max= 145 Ave= 129 Mode= 130 | Min= 65 Max= 72 Ave= 71 Mode= 70 | 36.7 | _ | 4.1 | Min= 5523 Max= 10875 Ave= 6761 | the well was closed from 10:15 to 13:15 to clean the stone trap in 1403/05/01 |
| AZNS-047 | 77 | Gadvan | Gd.5a.5c | Producer | 0.1 | 818 | 30 | Min= 1150 Max= 1420 Ave= 1355 Mode= 1300 | Min= 165 Max= 190 Ave= 179 Mode= 180 | Min= 55 Max= 68 Ave= 65 Mode= 66 | 32.3 | _ | 2.8 | Min= 1712 Max= 3105 Ave= 2775 | well opened at 14:30 on 1403/02/11 |
| AZNS-048 | 20 | Sarvak | Sv.3.4-6 | Producer | 1.2 | 439 | 24 | Min= 100 Max= 300 Ave= 189 Mode= 180 | Min= 100 Max= 160 Ave= 129 Mode= 130 | Min= 10 Max= 35 Ave= 28 Mode= 32 | 20 | _ | 1.48 | Min= 440 Max= 1057 Ave= 712 | well opened at 1:05 in 1403/03/10 |
| AZNS-049 | 69 | Fahliyan | Fa.1a.1c.3a | Producer | 0.1 | 1678 | 32 | Min= 180 Max= 1400 Ave= 655 Mode= 200 | Min= 140 Max= 170 Ave= 153 Mode= 150 | Min= 28 Max= 72 Ave= 53 Mode= 48 | 32.2 | _ | 6.55 | Min= 454 Max= 2361 Ave= 1156 | well closed at 10:50 and discconnect of stone trap and chocks then opened at16:20 by chock manifold contractor in 1403/11/09 |
| AZNS-050 | 57 | Fahliyan | Fa.4a | Producer | 0.2 | 1090 | 28 | Min= 850 Max= 940 Ave= 886 Mode= 870 | Min= 160 Max= 180 Ave= 172 Mode= 170 | Min= 52 Max= 60 Ave= 57 Mode= 56 | _ | _ | 2.19 | Min= 418 Max= 1678 Ave= 1532 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-053 | 51 | Sarvak | Sv.3 | Producer | 3.6 | 292 | 20 | Min= 170 Max= 250 Ave= 223 Mode= 250 | Min= 170 Max= 190 Ave= 181 Mode= 190 | Min= 11 Max= 28 Ave= 18 Mode= 24 | 21 | _ | 1.24 | Min= 240 Max= 538 Ave= 469 | Well opened at 3 in 1403/03/29 |
| AZNS-054 | 15 | Sarvak | Sv.3.4-6 | Producer | 0.3 | 439 | 15 | Min= 100 Max= 250 Ave= 206 Mode= 230 | Min= 100 Max= 165 Ave= 145 Mode= 160 | Min= 8 Max= 30 Ave= 22 Mode= 26 | 20.7 | _ | 1.01 | Min= 115 Max= 292 Ave= 225 | closed at 10:10 due to air capsool discgarge and reopened at 12:00 reopened after recharge in 1403/06/20 |
| AZNS-056 | 5 | Sarvak | Sv.3.4-6 | Producer | 1.2 | 439 | 22 | Min= 230 Max= 340 Ave= 326 Mode= 330 | 155 | Min= 27 Max= 37 Ave= 31 Mode= 30 | 18.7 | _ | 1.68 | Min= 429 Max= 716 Ave= 636 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-057 | 25 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 22 | Min= 170 Max= 190 Ave= 179 Mode= 180 | Min= 125 Max= 135 Ave= 132 Mode= 130 | Min= 31 Max= 36 Ave= 34 Mode= 34 | 20.4 | _ | 0.84 | Min= 494 Max= 1005 Ave= 603 | well open at 02:30 in 1403/04/03 |
| AZNS-058 | 18 | Gadvan | Gd.5c | Producer | 0.3 | 1381 | 32 | Min= 640 Max= 720 Ave= 688 Mode= 700 | Min= 130 Max= 140 Ave= 134 Mode= 130 | Min= 33 Max= 45 Ave= 39 Mode= 34 | 30.6 | _ | 2.57 | Min= 1112 Max= 1586 Ave= 1410 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| AZNS-059 | 20 | Sarvak | Sv.3.4-6 | Producer | 2 | 439 | 28 | Min= 180 Max= 260 Ave= 240 Mode= 250 | Min= 125 Max= 160 Ave= 138 Mode= 125 | Min= 23 Max= 34 Ave= 27 Mode= 27 | 18.9 | _ | 2.14 | Min= 738 Max= 1899 Ave= 1147 | well opened at 1:05 in 1403/03/10 |
| AZNS-062 | 56 | Sarvak | Sv.3.4-6 | Producer | 0.6 | 292 | 23 | Min= 200 Max= 250 Ave= 225 Mode= 220 | Min= 165 Max= 195 Ave= 183 Mode= 195 | Min= 19 Max= 32 Ave= 26 Mode= 26 | 22.1 | _ | 1.09 | Min= 417 Max= 691 Ave= 618 | Well opened at 1:30 in 1403/03/29 |
| AZNS-063 | 51 | Sarvak | Sv.3.4-6 | Producer | 0.8 | 292 | 24 | Min= 190 Max= 270 Ave= 232 Mode= 230 | Min= 175 Max= 190 Ave= 183 Mode= 185 | Min= 16 Max= 27 Ave= 20 Mode= 19 | 21.4 | _ | 1.21 | Min= 371 Max= 801 Ave= 684 | closed at 12:20 pm by order of management in 1403/06/12 |
| AZNS-065 | 69 | Sarvak | Sv.3 | Producer | 1 | 361 | 15 | Min= 110 Max= 230 Ave= 140 Mode= 130 | Min= 110 Max= 140 Ave= 129 Mode= 130 | Min= 10 Max= 34 Ave= 17 Mode= 11 | 21.21 | _ | 0.5 | Min= 220 Max= 457 Ave= 273 | well opened at 18:00 witch chock size 15/64''. In 1403/08/13 |
| AZNS-067 | 18 | Sarvak | Sv.3.4-6 | Producer | 2 | 439 | 28 | Min= 340 Max= 350 Ave= 347 Mode= 350 | Min= 135 Max= 165 Ave= 155 Mode= 160 | Min= 37 Max= 42 Ave= 40 Mode= 39 | 18.6 | _ | 1.66 | Min= 768 Max= 1121 Ave= 1005 | installation of MPFM at 20:45 by chock 24/64@1403/07/24 |
| AZNS-068 | 6 | Sarvak | Sv.8 | Producer | 6.5 | 439 | 15 | Min= 210 Max= 280 Ave= 250 Mode= 250 | Min= 135 Max= 155 Ave= 144 Mode= 140 | Min= 20 Max= 28 Ave= 24 Mode= 23 | 18.7 | _ | 1.07 | Min= 210 Max= 283 Ave= 249 | well closed at 10:40due to leakage of 32" and high salt and opened at 18:10 in 1403/08/03 |
| AZNS-069 | 10 | Sarvak | Sv.3.4-6 | Producer | 3.9 | 439 | 15 | Min= 230 Max= 250 Ave= 242 Mode= 240 | Min= 155 Max= 175 Ave= 165 Mode= 160 | Min= 25 Max= 30 Ave= 28 Mode= 28 | 19.9 | _ | 1.04 | Min= 199 Max= 281 Ave= 249 | closed due to pressure drop on 1402/12/14 |
| AZNS-070 | 75 | Fahliyan | Fa.2a.3a | Producer | 0.5 | 1678 | 30 | Min= 1450 Max= 1580 Ave= 1508 Mode= 1500 | Min= 175 Max= 195 Ave= 188 Mode= 190 | Min= 73 Max= 74 Ave= 74 Mode= 74 | 29.6 | _ | 6.45 | Min= 1421 Max= 2412 Ave= 2248 | well was closed from 09:00 to 11:20 due to control panel problom. In 1403/09/28 |
| AZNS-071 | 17 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 28 | Min= 500 Max= 580 Ave= 549 Mode= 580 | Min= 245 Max= 265 Ave= 251 Mode= 250 | Min= 67 Max= 72 Ave= 71 Mode= 72 | 31 | _ | 3.32 | Min= 758 Max= 1082 Ave= 933 | well closed from08:00 to 10:30 due to problom in control panel in 1403/08/06 |
| AZNS-072 | 57 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 292 | 15 | Min= 180 Max= 240 Ave= 203 Mode= 200 | Min= 150 Max= 170 Ave= 167 Mode= 170 | Min= 18 Max= 24 Ave= 20 Mode= 20 | 20 | _ | 0.65 | Min= 69 Max= 319 Ave= 269 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-074 | 83 | Sarvak | Sv.3 | Producer | 0.25 | 361 | 15 | Min= 150 Max= 240 Ave= 191 Mode= 200 | Min= 150 Max= 200 Ave= 178 Mode= 180 | Min= 8 Max= 30 Ave= 20 Mode= 22 | 21.2 | _ | 0.93 | Min= 125 Max= 290 Ave= 238 | well opened at 14:00 on 1403/02/11 |
| AZNS-076 | 11 | Sarvak | Sv.3.4-6 | Producer | 0.3 | 439 | 15 | Min= 240 Max= 260 Ave= 247 Mode= 240 | Min= 140 Max= 160 Ave= 154 Mode= 160 | Min= 16 Max= 24 Ave= 19 Mode= 19 | 21 | _ | 0.96 | Min= 206 Max= 304 Ave= 263 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-079 | 78 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 361 | 26 | 350 | Min= 175 Max= 190 Ave= 181 Mode= 180 | Min= 33 Max= 47 Ave= 41 Mode= 42 | 21.4 | _ | 2.35 | Min= 645 Max= 1089 Ave= 1013 | closed due to pressure drop on 1402/12/23 |
| AZNS-080 | 90 | Sarvak | Sv.8 | Producer | 4 | 439 | 15 | Min= 200 Max= 280 Ave= 240 Mode= 250 | Min= 125 Max= 165 Ave= 145 Mode= 150 | Min= 27 Max= 33 Ave= 30 Mode= 30 | 18.8 | _ | 1.07 | Min= 179 Max= 308 Ave= 247 | well closed at 00:50 due to leakage of 20" line and opened at 10:40 in 1403/08/11 |
| AZNS-082 | 69 | Sarvak | Sv.3 | Producer | 0.1 | 361 | 24 | Min= 220 Max= 280 Ave= 254 Mode= 250 | Min= 100 Max= 110 Ave= 104 Mode= 105 | Min= 39 Max= 42 Ave= 40 Mode= 40 | 21 | _ | 1.99 | Min= 795 Max= 1600 Ave= 1009 | well opened at 15:20 in 1403/03/10 |
| AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 28 | Min= 230 Max= 290 Ave= 266 Mode= 270 | Min= 135 Max= 155 Ave= 147 Mode= 155 | Min= 32 Max= 38 Ave= 35 Mode= 34 | 20.6 | _ | 2.73 | Min= 653 Max= 944 Ave= 829 | well diverted to MPFM due to well test operation at 17:00 with chock size 28/64". In 1403/07/03 |
| AZNS-084 | 47 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 292 | 15 | Min= 180 Max= 290 Ave= 206 Mode= 190 | Min= 170 Max= 195 Ave= 182 Mode= 185 | Min= 11 Max= 35 Ave= 25 Mode= 30 | 22.4 | _ | 0.99 | Min= 161 Max= 376 Ave= 275 | well opened at 11:30 in 1403/07/01 |
| AZNS-086 | 1 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 36 | Min= 540 Max= 600 Ave= 585 Mode= 600 | Min= 165 Max= 200 Ave= 182 Mode= 180 | Min= 38 Max= 49 Ave= 44 Mode= 46 | 34.4 | _ | 2.73 | Min= 1207 Max= 1723 Ave= 1520 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-087 | 8 | Sarvak | Sv.8 | Producer | 0.4 | 439 | 15 | Min= 200 Max= 250 Ave= 224 Mode= 220 | Min= 135 Max= 155 Ave= 146 Mode= 150 | Min= 23 Max= 29 Ave= 26 Mode= 26 | 20 | _ | 0.68 | Min= 196 Max= 277 Ave= 242 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-089 | 78 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 361 | 20 | Min= 220 Max= 260 Ave= 236 Mode= 230 | Min= 175 Max= 190 Ave= 180 Mode= 180 | Min= 24 Max= 32 Ave= 27 Mode= 26 | 22.3 | _ | 1.71 | Min= 291 Max= 516 Ave= 466 | well opened at 14:10 on 1403/02/11 |
| AZNS-090 | 90 | Sarvak | Sv.8 | Producer | 4.9 | 439 | 22 | Min= 200 Max= 270 Ave= 257 Mode= 260 | Min= 125 Max= 165 Ave= 147 Mode= 155 | Min= 25 Max= 35 Ave= 31 Mode= 32 | 19.2 | _ | 1.49 | Min= 387 Max= 591 Ave= 503 | well closed at 00:55 due to leakage of 20" line and opened at 10:40 in 1403/08/11 |
| AZNS-091 | 83 | Sarvak | Sv.3 | Producer | 0.2 | 361 | 24 | Min= 230 Max= 240 Ave= 237 Mode= 240 | Min= 185 Max= 200 Ave= 192 Mode= 185 | Min= 28 Max= 39 Ave= 35 Mode= 34 | 19.9 | _ | 1.38 | Min= 413 Max= 698 Ave= 641 | well opened at 13:30 on 1403/02/11 |
| AZNS-092 | 19 | Kazhdumi | Kz.2.4 | Producer | 0.1 | 916 | 32 | Min= 1020 Max=1050 Ave=1042 Mode=1040 | Min= 130 Max=195 Ave=176 Mode=180 | Min= 15 Max= 78 Ave= 73 Mode= 77 | 31.6 | _ | 5.34 | Min= 1813 Max= 2587 Ave= 2334 | well opened at00:50 in 1403/03/10 |
| AZNS-093 | 93 | Sarvak | Sv.3.4-6 | Producer | 0.6 | 292 | 15 | Min= 175 Max= 250 Ave= 205 Mode= 200 | Min= 175 Max= 195 Ave= 184 Mode= 180 | Min= 10 Max= 21 Ave= 14 Mode= 10 | 19.8 | _ | 0.44 | Min= 157 Max= 329 Ave= 274 | Well opened at 2 in 1403/03/29 |
| AZNS-094 | 89 | Sarvak | Sv.8 | Producer | 1.8 | 439 | 15 | Min= 190 Max= 220 Ave= 206 Mode= No Repeat | 170 | Min= 11 Max= 25 Ave= 18 Mode= No Repeat | 19.9 | _ | 0.35 | Min= 89 Max= 267 Ave= 219 | closed due to pressure drop in 1403/10/19 |
| AZNS-095 | 31 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 32 | Min= 230 Max= 250 Ave= 242 Mode= 240 | Min= 100 Max= 130 Ave= 120 Mode= 120 | Min= 19 Max= 24 Ave= 22 Mode= 24 | 18.4 | _ | 2.52 | Min= 773 Max= 1091 Ave= 967 | well opened at00:50 in 1403/03/10 |
| AZNS-096 | 14 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 24 | Min= 190 Max= 200 Ave= 195 Mode= 200 | Min= 130 Max= 135 Ave= 131 Mode= 130 | Min= 18 Max= 26 Ave= 21 Mode= 21 | 19.1 | _ | 0.83 | Min= 350 Max= 558 Ave= 493 | well closed at 13:15 due to change adjustable chock and opened at 14:45 in 1403/04/09 |
| AZNS-097 | 8 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 16 | Min= 180 Max= 240 Ave= 215 Mode= 200 | Min= 135 Max= 160 Ave= 147 Mode= 150 | Min= 25 Max= 31 Ave= 27 Mode= 27 | 20.9 | _ | 1.38 | Min= 216 Max= 317 Ave= 263 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-098 | 46 | Sarvak | Sv.3 | Producer | 1.2 | 292 | 30 | Min= 200 Max= 280 Ave= 256 Mode= 280 | Min= 175 Max= 190 Ave= 184 Mode= 180 | Min= 10 Max= 29 Ave= 18 Mode= 16 | 21.9 | _ | 0.74 | Min= 564 Max= 1245 Ave= 1085 | Well opened at 2:10 in 1403/03/29 |
| AZNS-099 | 91 | Sarvak | Sv.8 | Producer | 8 | 439 | 20 | Min= 140 Max= 360 Ave= 241 Mode= 190 | Min= 130 Max= 190 Ave= 161 Mode= 155 | Min= 13 Max= 36 Ave= 28 Mode= 32 | 18.2 | _ | 1.04 | Min= 197 Max= 594 Ave= 377 | well opened at 13:30 in 1403/10/18 |
| AZNS-100 | 1 | Sarvak | Sv.4-6 | Producer | 0.2 | 439 | 15 | Min= 180 Max= 250 Ave= 212 Mode= No Repeat | Min= 170 Max= 190 Ave= 179 Mode= No Repeat | Min= 9 Max= 24 Ave= 17 Mode= No Repeat | 19 | _ | 0.25 | Min= 186 Max= 292 Ave= 241 | closed due to pressure drop in 1403/10/19 |
| AZNS-101 | 47 | Sarvak | Sv.3.4-6 | Producer | 1.4 | 292 | 20 | Min= 200 Max= 300 Ave= 237 Mode= 250 | Min= 170 Max= 190 Ave= 183 Mode= 190 | Min= 13 Max= 38 Ave= 25 Mode= 26 | 20.4 | _ | 1.59 | Min= 240 Max= 601 Ave= 501 | Well opened at 2:50 in 1403/03/29 |
| AZNS-102 | 75 | Sarvak | Sv.3 | Producer | 0.1 | 361 | 28 | Min= 240 Max= 250 Ave= 246 Mode= 250 | Min= 175 Max= 195 Ave= 186 Mode= 190 | Min= 35 Max= 44 Ave= 40 Mode= 40 | 21.1 | _ | 2.32 | Min= 541 Max= 945 Ave= 866 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| AZNS-103 | 12 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 32 | Min= 550 Max= 600 Ave= 580 Mode= 570 | Min= 140 Max= 150 Ave= 145 Mode= 145 | Min= 45 Max= 56 Ave= 53 Mode= 55 | 31.1 | _ | 1.84 | Min= 1532 Max= 3075 Ave= 1879 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-104 | 52 | Sarvak | Sv.3 | Producer | 0.3 | 292 | 15 | Min= 220 Max= 330 Ave= 319 Mode= 320 | Min= 160 Max= 180 Ave= 169 Mode= 165 | Min= 16 Max= 27 Ave= 20 Mode= 22 | 21 | _ | 1.06 | Min= 104 Max= 423 Ave= 387 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-105 | 16 | Sarvak | Sv.8 | Producer | 0.1 | 439 | 15 | Min= 190 Max= 210 Ave= 200 Mode= 200 | Min= 125 Max= 160 Ave= 146 Mode= 155 | Min= 15 Max= 24 Ave= 20 Mode= 21 | 20.1 | _ | 0.9 | Min= 173 Max= 253 Ave= 223 | well closed at 12:00 due to pressure drop in 1403/07/01 |
| AZNS-107 | 19 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 26 | Min= 210 Max= 260 Ave= 229 Mode= 220 | Min= 120 Max= 160 Ave= 124 Mode= 120 | Min= 25 Max= 38 Ave= 28 Mode= 27 | 17.7 | _ | 1.75 | Min= 834 Max= 1581 Ave= 981 | well opened at 15:00 . In 1403/09/06 |
| AZNS-108 | 48 | Sarvak | Sv.3.4-6 | Producer | 0.6 | 292 | 30 | Min= 210 Max= 250 Ave= 229 Mode= 230 | Min= 180 Max= 200 Ave= 191 Mode= 195 | Min= 14 Max= 47 Ave= 36 Mode= 36 | 19.6 | _ | 0.9 | Min= 640 Max= 1102 Ave= 1003 | Well opened at 2 in 1403/03/29 |
| AZNS-109 | 89 | Sarvak | Sv.8 | Producer | 1 | 439 | 15 | Min= 170 Max= 230 Ave= 201 Mode= 190 | Min= 160 Max= 190 Ave= 178 Mode= 180 | Min= 10 Max= 24 Ave= 18 Mode= 17 | 20.6 | _ | 0.45 | Min= 176 Max= 264 Ave= 221 | well reopened at 15:30 after finished slickline operation. In 1403/09/15 |
| AZNS-110 | 58 | Fahliyan | Fa.4a | Producer | 0.1 | 1090 | 30 | Min= 700 Max= 800 Ave= 790 Mode= 800 | Min= 150 Max= 190 Ave= 178 Mode= 180 | Min= 46 Max= 66 Ave= 64 Mode= 65 | _ | _ | 2.63 | Min= 385 Max= 1738 Ave= 1562 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-111 | 46 | Sarvak | Sv.3 | Producer | 1.6 | 292 | 24 | Min= 220 Max= 290 Ave= 253 Mode= 250 | Min= 175 Max= 190 Ave= 183 Mode= 180 | Min= 21 Max= 35 Ave= 26 Mode= 24 | 20.93 | _ | 0.5 | Min= 414 Max= 850 Ave= 728 | closed due to pressure drop in 1403/06/28 |
| AZNS-112 | 75 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 361 | 15 | Min= 220 Max= 300 Ave= 277 Mode= 280 | Min= 180 Max= 195 Ave= 186 Mode= 190 | Min= 36 Max= 48 Ave= 39 Mode= 40 | 22 | _ | 1.76 | Min= 207 Max= 349 Ave= 321 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| AZNS-114 | 1 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 439 | 15 | Min= 320 Max= 350 Ave= 342 Mode= 340 | Min= 175 Max= 205 Ave= 190 Mode= 190 | Min= 20 Max= 36 Ave= 31 Mode= 32 | 19.7 | _ | 0.48 | Min= 271 Max= 385 Ave= 343 | _ |
| AZNS-115 | 45 | Sarvak | Sv.3 | Producer | 4.2 | 292 | 15 | Min= 180 Max= 210 Ave= 193 Mode= No Repeat | 180 | Min= 8 Max= 12 Ave= 9 Mode= No Repeat | 19.7 | _ | 0.34 | Min= 121 Max= 272 Ave= 235 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZNS-118 | 26 | Sarvak | Sv.3.4-6 | Producer | 3.4 | 439 | 24 | Min= 240 Max= 280 Ave= 252 Mode= 250 | Min= 140 Max= 175 Ave= 150 Mode= 145 | Min= 24 Max= 31 Ave= 27 Mode= 27 | 20.9 | _ | 0.36 | Min= 453 Max= 677 Ave= 590 | well open at 16:20 in 1403/04/03 |
| AZNS-119 | 52 | Sarvak | Sv.3 | Producer | 0.1 | 292 | 15 | Min= 220 Max= 240 Ave= 238 Mode= 240 | Min= 100 Max= 180 Ave= 168 Mode= 165 | Min= 16 Max= 26 Ave= 21 Mode= 22 | 21.8 | _ | 0.93 | Min= 83 Max= 336 Ave= 306 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-120 | NL-2-5 | Gadvan | Gd.5c | Producer | 0 | 818 | 32 | Min= 220 Max= 340 Ave= 262 Mode= 230 | Min= 105 Max= 160 Ave= 134 Mode= 155 | Min= 28 Max= 45 Ave= 38 Mode= 42 | _ | _ | 0.04 | Min= 356 Max= 1112 Ave= 796 | well opened, the end of performance test 72 in 1403/10/01 |
| AZNS-121 | 44 | Fahliyan | Fa.1 | Producer | 0.1 | 1678 | 34 | Min= 2000 Max= 2280 Ave= 2212 Mode= 2200 | Min= 280 Max= 385 Ave= 373 Mode= 380. | Min= 97 Max= 100 Ave= 99 Mode= 99 | 34.1 | _ | 2.58 | Min= 2684 Max= 4532 Ave= 4196 | Well opened at 1:20 in 1403/03/29 |
| AZNS-124 | 49 | Sarvak | Sv.3 | Producer | 1 | 292 | 30 | Min= 220 Max= 240 Ave= 232 Mode= 230 | Min= 170 Max= 195 Ave= 187 Mode= 190 | Min= 13 Max= 26 Ave= 17 Mode= 15 | 19.3 | _ | 0.88 | Min= 637 Max= 1113 Ave= 1006 | repened at 12:15 in 1403/06/28 |
| AZNS-125 | 53 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 292 | 15 | Min= 160 Max= 220 Ave= 174 Mode= 165 | Min= 160 Max= 170 Ave= 166 Mode= 165 | Min= 10 Max= 20 Ave= 16 Mode= 14 | 21.9 | _ | 0.44 | Min= 118 Max= 293 Ave= 223 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZNS-126 | 45 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 292 | 20 | Min= 185 Max= 300 Ave= 217 Mode= 200 | Min= 180 Max= 195 Ave= 188 Mode= 185 | Min= 10 Max= 20 Ave= 16 Mode= 14 | 20.4 | _ | 0.45 | Min= 282 Max= 622 Ave= 474 | Well opened at 2:40 in 1403/03/29 |
| AZNS-127 | 54 | Fahliyan | Fa.1.2a.3a. | Producer | 0.2 | 1090 | 32 | Min= 1300 Max= 1650 Ave= 1424 Mode= 1400 | Min= 190 Max= 210 Ave= 204 Mode= 200 | Min= 59 Max= 69 Ave= 67 Mode= 68 | 37.5 | _ | 0.01 | Min= 1672 Max= 3277 Ave= 2828 | well deliver to ARVANDAN after 72 hours testing in 1403/10/07 |
| AZNS-128 | 42 | Sarvak | Sv.3.4-6 | Producer | 0.8 | 292 | 15 | Min= 170 Max= 430 Ave= 237 Mode= 300 | Min= 165 Max= 200 Ave= 183 Mode= 180 | Min= 10 Max= 33 Ave= 17 Mode= 13 | 21.1 | _ | 0.23 | Min= 157 Max= 387 Ave= 302 | well opened at 14:40 in 1403/03/10 |
| AZNS-129 | 12 | Sarvak | Sv.8 | Producer | 2 | 439 | 15 | Min= 150 Max= 200 Ave= 168 Mode= No Repeat | Min= 145 Max= 160 Ave= 151 Mode= No Repeat | Min= 11 Max= 22 Ave= 15 Mode= No Repeat | 19.6 | _ | 0.25 | Min= 169 Max= 232 Ave= 195 | closed due to pressure drop in 1403/10/19 |
| AZNS-133 | 76 | Gadvan | Gd.5a.5c | Producer | 2 | 818 | 30 | Min= 1000 Max= 1370 Ave= 1059 Mode= 1100 | Min= 175 Max= 190 Ave= 181 Mode= 180 | Min= 54 Max= 59 Ave= 56 Mode= 56 | 36.3 | _ | 1.34 | Min= 1413 Max= 2823 Ave= 2229 | well opened at 13:50 on 1403/02/11 |
| AZNS-134 | 60 | Sarvak | Sv.3 | Producer | 0.8 | 292 | 15 | Min= 200 Max= 290 Ave= 259 Mode= 260 | Min= 150 Max= 175 Ave= 166 Mode= 165 | Min= 28 Max= 33 Ave= 31 Mode= 32 | 22 | _ | 1.04 | Min= 96 Max= 356 Ave= 323 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-135 | 49 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 15 | Min= 200 Max= 260 Ave= 223 Mode= 220 | Min= 170 Max= 195 Ave= 186 Mode= 190 | Min= 8 Max= 28 Ave= 15 Mode= 12 | 20.9 | _ | 0.31 | Min= 192 Max= 326 Ave= 295 | deliver to PEDEC for work over well 127 on on 1403/01/15 |
| AZNS-136 | 42 | Sarvak | Sv.3 | Producer | 2 | 292 | 24 | Min= 230 Max= 300 Ave= 273 Mode= 280 | Min= 180 Max= 200 Ave= 192 Mode= 190 | Min= 44 Max= 48 Ave= 46 Mode= 45 | 21.5 | _ | 0.39 | Min= 500 Max= 870 Ave= 772 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-137 | 68 | Sarvak | Sv.3 | Producer | 0.8 | 361 | 20 | Min= 200 Max= 230 Ave= 214 Mode= 220 | Min= 120 Max= 135 Ave= 129 Mode= 130 | Min= 25 Max= 31 Ave= 28 Mode= 30 | 22 | _ | 0.45 | Min= 521 Max= 1078 Ave= 631 | due to order of management change flow line from S-GASP to SKID at 14:30 in 1403/02/31 |
| AZNS-138 | 66 | Fahliyan | Fa.1c.2a | Producer | 0.5 | 1678 | 32 | Min= 1300 Max= 1880 Ave= 1514 Mode= 1500 | Min= 125 Max= 150 Ave= 134 Mode= 130 | Min= 70 Max= 80 Ave= 74 Mode= 72 | 35.7 | _ | 4.02 | Min= 3207 Max= 7668 Ave= 4123 | FIX CHOCK CHANGE OF 32/64 TO 34/64 in 1403/10/15 |
| AZNS-139 | 32 | Sarvak | Sv.3.4-6 | Producer | 1.5 | 439 | 20 | Min= 180 Max= 250 Ave= 213 Mode= 220 | Min= 125 Max= 180 Ave= 150 Mode= 130 | Min= 31 Max= 47 Ave= 41 Mode= 43 | 18.7 | _ | 0.24 | Min= 303 Max= 489 Ave= 380 | well opened at 10:00. in 1403/09/07 |
| AZNS-140 | 57 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 15 | Min= 200 Max= 220 Ave= 211 Mode= 220 | Min= 150 Max= 170 Ave= 166 Mode= 170 | Min= 35 Max= 40 Ave= 38 Mode= 38 | 21.1 | _ | 1.23 | Min= 72 Max= 310 Ave= 277 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-141 | 54 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 24 | Min= 200 Max= 320 Ave= 239 Mode= 260 | Min= 150 Max= 170 Ave= 166 Mode= 170 | Min= 35 Max= 40 Ave= 38 Mode= 38 | 20 | _ | 0.27 | Min= 389 Max= 960 Ave= 708 | well reopened at 14:15 with static pressur 450 and flow pressur 390psi in 1403/10/03 |
| AZNS-142 | 34 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 292 | 24 | Min= 180 Max= 280 Ave= 208 Mode= 200 | Min= 135 Max= 150 Ave= 143 Mode= 140 | Min= 14 Max= 21 Ave= 18 Mode= 19 | 21.5 | _ | 0.43 | Min= 738 Max= 1561 Ave= 929 | well open at 17:00 in 1403/04/03 |
| AZNS-143 | 36 | Sarvak | Sv.3 | Producer | 28 | 292 | 28 | Min= 200 Max= 310 Ave= 243 Mode= 250 | Min= 140 Max= 150 Ave= 145 Mode= 145 | Min= 23 Max= 50 Ave= 45 Mode= 46 | 19.7 | _ | 0.7 | Min= 1053 Max= 2802 Ave= 1398 | well open at 17:00 in 1403/04/03 |
| AZNS-144 | 43 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 292 | 24 | Min= 200 Max= 250 Ave= 231 Mode= 230 | Min= 175 Max= 190 Ave= 186 Mode= 190 | Min= 40 Max= 47 Ave= 43 Mode= 42 | 21.3 | _ | 0.32 | Min= 388 Max= 785 Ave= 686 | well opened at 15:05 in 1403/03/10 |
| AZNS-145 | 40 | Sarvak | Sv.3.4-6 | Producer | 4 | 292 | 30 | Min= 220 Max= 400 Ave= 359 Mode= 370 | Min= 180 Max= 195 Ave= 186 Mode= 185 | Min= 32 Max= 34 Ave= 32 Mode= 34 | 21.3 | _ | 0.65 | Min= 917 Max= 1585 Ave= 1391 | well opened at 13:00. on 1403/02/13 |
| AZNS-148 | 77 | Fahliyan | Fa.2a.3a | Producer | 0.1 | 1678 | 28 | Min= 1450 Max= 1600 Ave= 1513 Mode= 1500 | Min= 175 Max= 190 Ave= 183 Mode= 190 | Min= 57 Max= 62 Ave= 60 Mode= 60 | 36.7 | _ | 2.37 | Min= 1272 Max= 2200 Ave= 2007 | crack operation done.well closed at 13:00 due to change of 3sv needle and opened at 14:00 in 1403/05/03 |
| AZNS-150 | 36 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 28 | Min= 160 Max= 200 Ave= 185 Mode= 200 | Min= 140 Max= 150 Ave= 144 Mode= 140 | Min= 45 Max= 61 Ave= 54 Mode= 56 | 20.1 | _ | 0.5 | Min= 898 Max= 1806 Ave= 1104 | well open at 17:00 in 1403/04/03 |
| AZNS-151 | 34 | Sarvak | Sv.3 | Producer | 0.1 | 292 | 24 | Min= 180 Max= 260 Ave= 227 Mode= 250 | Min= 140 Max= 170 Ave= 147 Mode= 140 | Min= 34 Max= 65 Ave= 60 Mode= 62 | 20.1 | _ | 0.44 | Min= 720 Max= 1444 Ave= 995 | well open at 17:00 in 1403/04/03 |
| AZNS-152 | 39 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 24 | Min= 430 Max= 480 Ave= 449 Mode= 450 | Min= 195 Max= 210 Ave= 199 Mode= 200 | Min= 40 Max= 46 Ave= 43 Mode= 40 | 22 | _ | 0.46 | Min= 776 Max= 1267 Ave= 1171 | well opened at 14:40 in 1403/03/10 |
| AZNS-153 | 60 | Sarvak | Sv.3 | Producer | 1 | 292 | 16 | Min= 200 Max= 320 Ave= 224 Mode= 220 | Min= 150 Max= 175 Ave= 166 Mode= 165 | Min= 27 Max= 33 Ave= 30 Mode= 30 | 22.5 | _ | 0.58 | Min= 104 Max= 374 Ave= 317 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-154 | 70 | Sarvak | Sv.3 | Producer | 0.6 | 361 | 28 | Min= 200 Max= 220 Ave= 217 Mode= 220 | Min= 120 Max= 140 Ave= 127 Mode= 130 | Min= 19 Max= 32 Ave= 25 Mode= 23 | 22.3 | _ | 1.95 | Min= 945 Max= 1941 Ave= 1151 | well closed at 11:30 due to slick line operation and checking 3sv operation and opened at 17:45 in 1403/08/18 |
| AZNS-155 | 71 | Sarvak | Sv.3 | Producer | 0.3 | 361 | 20 | Min= 165 Max= 190 Ave= 171 Mode= No Repeat | Min= 160 Max= 170 Ave= 166 Mode= No Repeat | Min= 7 Max= 40 Ave= 18 Mode= No Repeat | 21.6 | _ | 0.33 | Min= 178 Max= 395 Ave= 333 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZNS-156 | 54 | Sarvak | Sv.3 | Producer | 0.3 | 292 | 20 | Min= 280 Max= 360 Ave= 314 Mode= 300 | Min= 165 Max= 185 Ave= 174 Mode= 170 | Min= 34 Max= 37 Ave= 36 Mode= 36 | 19.7 | _ | 0.47 | Min= 393 Max= 721 Ave= 641 | reopened at 14:00 with flow pressur 420psi in 1403/10/03 |
| AZNS-157 | 38 | Sarvak | Sv.3 | Producer | 0.6 | 292 | 26 | 200 | Min= 135 Max= 145 Ave= 141 Mode= 140 | Min= 30 Max= 40 Ave= 36 Mode= 36 | 21.3 | _ | 0.48 | Min= 846 Max= 1724 Ave= 1034 | well deliver to arvandan in 1403/06/17 |
| AZNS-158 | 73 | Sarvak | Sv.3 | Producer | 0.3 | 361 | 15 | 250 | Min= 165 Max= 195 Ave= 178 Mode= 180 | Min= 55 Max= 57 Ave= 55 Mode= 55 | 22.4 | _ | 1.67 | Min= 188 Max= 318 Ave= 296 | well opened at 13:50 on 1403/02/11 |
| AZNS-160 | 22 | Gadvan | Gd.3 | Producer | 0.1 | 1381 | 32 | Min= 550 Max= 600 Ave= 566 Mode= 560 | Min= 145 Max= 180 Ave= 154 Mode= 150 | Min= 57 Max= 58 Ave= 57 Mode= 57 | 29.6 | _ | 2.06 | Min= 1467 Max= 3203 Ave= 1848 | well open at 17:00 in 1403/04/03 |
| AZNS-161 | 68 | Sarvak | Sv.3 | Producer | 0.6 | 361 | 20 | Min= 200 Max= 230 Ave= 204 Mode= 200 | Min= 120 Max= 140 Ave= 129 Mode= 130 | Min= 28 Max= 34 Ave= 31 Mode= 31 | 21.3 | _ | 0.49 | Min= 491 Max= 1000 Ave= 609 | well reopene at01:15 after arrest the leakge. In 1403/08/05 |
| AZNS-162 | 65 | Sarvak | Sv.3 | Producer | 0.4 | 292 | 30 | Min= 300 Max= 310 Ave= 309 Mode= 310 | Min= 120 Max= 130 Ave= 127 Mode= 130 | Min= 20 Max= 44 Ave= 42 Mode= 43 | 21.9 | _ | 0.99 | Min= 1550 Max= 3154 Ave= 1889 | well opened at 13:00 in 1403/03/10 |
| AZNS-163 | 28 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 439 | 32 | Min= 310 Max= 350 Ave= 337 Mode= 350 | Min= 165 Max= 230 Ave= 199 Mode= 200 | Min= 55 Max= 59 Ave= 58 Mode= 58 | 19.4 | _ | 0.76 | Min= 1012 Max= 1720 Ave= 1281 | well closed due to pressure drop at 15:30. in 1403/07/03 |
| AZNS-166 | 9 | Kazhdumi | Kz.2-5 | Producer | 1 | 916 | 32 | Min= 1400 Max= 1450 Ave= 1431 Mode= 1440 | Min= 155 Max= 175 Ave= 165 Mode= 170 | Min= 43 Max= 52 Ave= 47 Mode= 49 | 22.94 | _ | 0.42 | Min= 2374 Max= 3349 Ave= 3104 | well closed at 10:00 due to control panel fault and opened at 10:30 in 1403/11/04 |
| AZNS-167 | 65 | Sarvak | Sv.3 | Producer | 0.1 | 292 | 24 | 240 | Min= 120 Max= 130 Ave= 128 Mode= 130 | Min= 28 Max= 33 Ave= 30 Mode= 30 | 21.9 | _ | 0.56 | Min= 857 Max= 1744 Ave= 1047 | _ |
| AZNS-168 | 30 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 24 | Min= 330 Max= 360 Ave= 349 Mode= 350 | Min= 130 Max= 180 Ave= 159 Mode= 140 | Min= 20 Max= 47 Ave= 27 Mode= 28 | 20.6 | _ | 0.77 | Min= 615 Max= 1069 Ave= 803 | well closed at 14:00 due to salt productin on 1403/01/18 |
| AZNS-169 | 28 | Sarvak | Sv.3.4-6 | Producer | 6 | 439 | 28 | Min= 190 Max= 350 Ave= 318 Mode= 340 | Min= 130 Max= 185 Ave= 158 Mode= 180 | Min= 29 Max= 49 Ave= 41 Mode= 41 | 17.6 | _ | 0.2 | Min= 650 Max= 1225 Ave= 907 | well opened at 10:00. in 1403/09/07 |
| AZNS-170 | 29 | Sarvak | Sv.3 | Producer | 0.2 | 439 | 24 | Min= 500 Max= 560 Ave= 532 Mode= 540 | Min= 120 Max= 175 Ave= 147 Mode= 170 | Min= 40 Max= 44 Ave= 42 Mode= 42 | 21.5 | _ | 1.14 | Min= 885 Max= 1518 Ave= 1130 | well open at 17:00 in 1403/04/03 |
| AZNS-171 | 41 | Sarvak | Sv.3 | Producer | 0.6 | 292 | 26 | Min= 220 Max= 300 Ave= 276 Mode= 280 | Min= 180 Max= 200 Ave= 190 Mode= 185 | Min= 21 Max= 38 Ave= 33 Mode= 34 | 20.7 | _ | 0.38 | Min= 585 Max= 1044 Ave= 918 | closed at 8:00pm due to pressure drop at 2sv and reopen at 10 pm in 1403/04/09 |
| AZNS-172 | 22 | Sarvak | Sv.8 | Producer | 0.5 | 439 | 30 | Min= 120 Max= 210 Ave= 191 Mode= 190 | Min= 120 Max= 160 Ave= 127 Mode= 120 | Min= 48 Max= 60 Ave= 50 Mode= 50 | 20 | _ | 0.87 | Min= 870 Max= 1873 Ave= 1081 | well open at 17:00 in 1403/04/03 |
| AZNS-173 | 72 | Gadvan | Gd.5 | Producer | 2 | 818 | 32 | Min= 520 Max= 550 Ave= 537 Mode= No Repeat | Min= 140 Max= 180 Ave= 160 Mode= No Repeat | Min= 37 Max= 45 Ave= 40 Mode= No Repeat | 33.9 | _ | 0.01 | Min= 896 Max= 1430 Ave= 1352 | delivered to arvandan(new well) in 1403/11/03 |
| AZNS-176 | 30 | Sarvak | Sv.3.4-6 | Producer | 1.6 | 439 | 20 | Min= 240 Max= 260 Ave= 248 Mode= 250 | Min= 130 Max= 175 Ave= 146 Mode= 130 | Min= 27 Max= 31 Ave= 30 Mode= 30 | 20.6 | _ | 0.45 | Min= 336 Max= 585 Ave= 437 | well closed at 14:00 due to salt productin on 1402/09/06 |
| AZNS-177 | 39 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 292 | 24 | Min= 400 Max= 480 Ave= 436 Mode= 420 | Min= 195 Max= 205 Ave= 199 Mode= 200 | 20 | 19.7 | _ | 0.44 | Min= 723 Max= 1275 Ave= 1150 | well opened at 14:00 in 1403/03/10 |
| AZNS-178 | 43 | Sarvak | Sv.3 | Producer | 0.4 | 292 | 24 | Min= 300 Max= 320 Ave= 302 Mode= 300 | Min= 180 Max= 195 Ave= 185 Mode= 185 | Min= 33 Max= 41 Ave= 38 Mode= 39 | 21.5 | _ | 0.37 | Min= 539 Max= 958 Ave= 850 | well opened at 15:05 in 1403/03/10 |
| AZNS-180 | 72 | Sarvak | Sv.3.4-6 | Producer | _ | 361 | 24 | Min= 180 Max= 220 Ave= 197 Mode= No Repeat | Min= 110 Max= 165 Ave= 146 Mode= No Repeat | Min= 17 Max= 26 Ave= 20 Mode= No Repeat | _ | _ | 0.001 | Min= 491 Max= 578 Ave= 534 | delivered to arvandan(new well) in 1403/11/07 |
| AZNS-181 | 3 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 24 | Min= 340 Max= 390 Ave= 366 Mode= 350 | Min= 130 Max= 135 Ave= 132 Mode= 130 | Min= 36 Max= 40 Ave= 38 Mode= 38 | 21.1 | _ | 0.13 | Min= 1009 Max= 2002 Ave= 1243 | well closed due to fix chock exchange from24/64'' to 28/64''. In 1403/08/05 |
| AZNS-182 | 12 | Sarvak | Sv.8 | Producer | 4 | 439 | 20 | Min= 270 Max= 380 Ave= 293 Mode= 300 | Min= 145 Max= 165 Ave= 156 Mode= 165 | Min= 13 Max= 27 Ave= 17 Mode= 16 | 19 | _ | 0.38 | Min= 308 Max= 650 Ave= 483 | well closed at 00:45 due to leakage of 20" line and opened at 10:40 in 1403/08/11 |
| AZNS-184 | 9 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 439 | 24 | Min= 200 Max= 250 Ave= 225 Mode= 230 | Min= 145 Max= 165 Ave= 155 Mode= 160 | Min= 15 Max= 22 Ave= 18 Mode= 19 | 20.2 | _ | 0.31 | Min= 445 Max= 648 Ave= 555 | well opened at 00:30 in 1403/03/10 |
| AZNS-185 | 37 | Sarvak | Sv.3.4-6 | Producer | 1 | 292 | 28 | Min= 400 Max= 430 Ave= 407 Mode= 400 | Min= 90 Max= 130 Ave= 116 Mode= 120 | Min= 40 Max= 47 Ave= 44 Mode= 45 | 22 | _ | 0.3 | Min= 1678 Max= 3416 Ave= 2077 | well open at 16:40 in 1403/04/03 |
| AZNS-186 | 3 | Sarvak | Sv.8 | Producer | _ | 439 | 20 | Min= 180 Max= 220 Ave= 197 Mode= 200 | Min= 120 Max= 135 Ave= 123 Mode= 125 | Min= 23 Max= 36 Ave= 25 Mode= 24 | _ | _ | 0.05 | Min= 438 Max= 891 Ave= 550 | Well delivered to arvandan in 1403/08/11 |
| AZNS-187 | 40 | Sarvak | Sv.3.4-6 | Producer | 4 | 292 | 24 | Min= 200 Max= 280 Ave= 244 Mode= 250 | Min= 180 Max= 195 Ave= 188 Mode= 190 | Min= 27 Max= 38 Ave= 32 Mode= 32 | 20.2 | _ | 0.33 | Min= 434 Max= 791 Ave= 692 | the check was changed from 24 to 28 by order of oil engineering. In 1403/04/23 |
| AZNS-188 | 91 | Sarvak | Sv.8 | Producer | 0.3 | 439 | 24 | Min= 200 Max= 210 Ave= 202 Mode= 200 | Min= 125 Max= 175 Ave= 159 Mode= 170 | Min= 16 Max= 22 Ave= 19 Mode= 19 | 19.6 | _ | 0.5 | Min= 625 Max= 1272 Ave= 769 | well opened at 09:45 . In 1403/04/15 |
| AZNS-189 | 24 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 28 | Min= 350 Max= 400 Ave= 379 Mode= 380 | Min= 120 Max= 135 Ave= 123 Mode= 120 | Min= 25 Max= 35 Ave= 29 Mode= 28 | 20.9 | _ | 0.72 | Min= 1383 Max= 2916 Ave= 1676 | well open at 16:20 in 1403/04/03 |
| AZNS-190 | 19 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 20 | Min= 280 Max= 300 Ave= 298 Mode= 300 | Min= 120 Max= 130 Ave= 120 Mode= 120 | Min= 18 Max= 65 Ave= 25 Mode= 24 | 20.2 | _ | 0.42 | Min= 632 Max= 1287 Ave= 771 | well open at 17:00 in 1403/04/03 |
| AZNS-191 | 33 | Sarvak | Sv.3 | Producer | 0.2 | 292 | 20 | Min= 190 Max= 250 Ave= 239 Mode= 250 | Min= 140 Max= 160 Ave= 146 Mode= 140 | Min= 16 Max= 25 Ave= 19 Mode= 17 | 21.6 | _ | 0.41 | Min= 547 Max= 1311 Ave= 761 | well deliver to arvandan in 1403/05/30 |
| AZNS-192 | 9 | Sarvak | Sv.4-6 | Producer | 0.4 | 439 | 24 | Min= 220 Max= 260 Ave= 236 Mode= 240 | Min= 150 Max= 170 Ave= 160 Mode= 165 | Min= 18 Max= 23 Ave= 20 Mode= 20 | 20.4 | _ | 0.31 | Min= 467 Max= 658 Ave= 576 | well opened at 14:00. in 1403/07/05 |
| AZNS-194 | 37 | Sarvak | Sv.3 | Producer | 1 | 292 | 28 | Min= 280 Max= 330 Ave= 299 Mode= 300 | Min= 140 Max= 160 Ave= 155 Mode= 160 | Min= 35 Max= 40 Ave= 37 Mode= 36 | 19.9 | _ | 0.26 | Min= 1330 Max= 2707 Ave= 1620 | well opened at 14:00 on 1403/03/10 |
| AZNS-197 | 2 | Kazhdumi | Kz.4 | Producer | 0.1 | 916 | 28 | Min= 250 Max= 300 Ave= 281 Mode= 280 | Min= 80 Max= 105 Ave= 87 Mode= 80 | Min= 23 Max= 33 Ave= 26 Mode= 26 | 32.5 | _ | 0.47 | Min= 498 Max= 724 Ave= 641 | well opened at00:20 in 1403/03/10 |
| AZNS-199 | 24 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 16 | Min= 100 Max= 180 Ave= 119 Mode= 120 | Min= 100 Max= 180 Ave= 118 Mode= 120 | Min= 9 Max= 20 Ave= 13 Mode= 10 | 20.3 | _ | 0.24 | Min= 204 Max= 1102 Ave= 561 | Fix choke change from 24 to 28 in 1403/10/19 |
| AZNS-203 | 72 | Sarvak | Sar-3 | Producer | 0.2 | 361 | 24 | Min=170 Max=230 Ave=203 Mode= No Repeat | Min=110 Max=180 Ave=160 Mode= No Repeat | Min=13 Max=29 Ave=21 Mode= No Repeat | 23 | _ | 0.01 | Min= 371 Max= 617 Ave= 528 | delivered to arvandan(new well) in 1403/11/03 |

Figure ‎3-3: Cumulative total producer wells and active wells of south Azadegan oil field reservoirs (up to January 2025)

Figure ‎3-4: Cumulative Production for South Azadegan field (up to January 2025)

Figure ‎3-5: Cumulative Production for Sarvak Reservoir of South Azadegan (up to January 2025)

Figure ‎3-6: Cumulative Production for Kazhdumi Reservoir of South Azadegan (up to January 2025)

Figure ‎3-7: Cumulative Production for Gadvan Reservoir of South Azadegan (up to January 2025)

Figure ‎3-8: Cumulative Production for Fahliyan Reservoir of South Azadegan (up to January 2025)

Figure ‎3-9: Cumulative oil production Bubble map and top 10 producer wells of South Azadegan Field (up to January 2025)

Figure ‎3-10: Cumulative oil production Bubble map and top 10 producer wells of Sarvak reservoir (up to January 2025)

Table ‎3-8: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to January 2025)

| Top 10 producer wells of Sarvak |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate | Cum oil | Remark |
|  |  |  |  |  |  |  | (STBD) | (MMSTB) |  |
| AZN-006 | Near 79 | Sarvak | Sv.4-6 | Producer | 36 | 350 | 1660 | 13.6 | well opened at 00:10 on 1403/03/10 |
| AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 34 | 200 | 910 | 10.2 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| AZN-019 | 13 | Sarvak | Sv.3 | Close | 24 | _ | _ | 10.1 | closed due to pressure drop on 1400/07/12 |
| AZN-018 | 2 | Sarvak | Sv.3.4-6 | Close | 30 | _ | _ | 7.4 | closed at 15:00 due to pressure drop in 1403/03/27 |
| AZN-015 | Near 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 120 | 476 | 6.1 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-002 | Near 36 | Sarvak | Sv.3.4-6 | Producer | 32 | 180 | 930 | 5.9 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-005 | Near 1 | Sarvak | Sv.3.4-6 | Producer | 24 | 160 | 200 | 4.9 | well opened at 1:30 in 1403/03/10 |
| AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 28 | 300 | 980 | 4.3 | Well opened at 13:30 in 1403/06/04 |
| AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 28 | 300 | 945 | 2.7 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 100 | 275 | 2.6 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |

Figure ‎3-11: Cumulative oil production Bubble map and top 10 producer wells of Kazhdumi reservoir (up to January 2025)

Table ‎3-9: Cumulative oil production top 10 producer wells of Kazdhumi reservoir (up to January 2025)

| Top 10 producer wells of Kazhdumi |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Cum oil (MMSTB) | Remark |
| AZN-017 | 50 | Kazhdumi | Kz.2 | Producer | 20 | 1440 | 1325 | 13.94 | 10' line( well was closed from 11:00 to 16:00 due to MPFM connection) in 1403/08/12 |
| AZN-004 | Near 56 | Kazhdumi | Kz.1.2 | Producer | 32 | 1420 | 2810 | 9.82 | the well was closed and opened for spool replacement on 1402/12/14 |
| AZN-021 | 15 | Kazhdumi | Kz.2 | Close | 24 | 0 | _ | 5.36 | closed due to pressure drop due to water production on 1399/02/11 |
| AZNS-092 | 19 | Kazhdumi | Kz.2.4 | Producer | 32 | 1020 | 2334 | 5.34 | well opened at00:50 in 1403/03/10 |
| AZNS-041 | 20 | Kazhdumi | Kz.2.4 | Close | 28 | 0 | _ | 1.06 | well closed due to asphaltene barrier in the well on 1402/10/11 |
| AZNS-064 | 13 | Kazhdumi | Kz.2.4 | Close | 28 | 0 | _ | 0.54 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| AZNS-028 | 11 | Kazhdumi | Kz.2.3.4 | Close | 24 | 0 | _ | 0.53 | closed due to asphaltene barrier on 1398/08/08 |
| AZNS-029 | 5 | Kazhdumi | Kz.3.4 | Producer | 28 | 190 | 497 | 0.48 | well closed at 11:00 due to disconnect of MPFM and opened at 17:30 in 1403/07/01 |
| AZNS-197 | 2 | Kazhdumi | Kz.4 | Producer | 28 | 300 | 650 | 0.47 | well opened at00:20 in 1403/03/10 |

Figure ‎3-12: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to January 2025)

Table ‎3-10: Cumulative oil production top 10 producer wells of Gadvan reservoir (up to January 2025)

| Top 10 producer wells of Gadvan |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (PSI) | Oil Rate (STBD) | Cum oil (MMSTB) | Remark |
| AZN-009 | 16 | Gadvan | Gd.5c | Producer | 50 | 900 | 3910 | 29.08 | well closed due to fault WK at 02:55 and opened at 13:30 on 1402/09/05 |
| AZN-023 | Near 75 | Gadvan | Gd.5a.5c | Producer | 34 | 1200 | 2930 | 16.3 | well closed due to fault WK at 03:10 and opened at 14:50 on 1402/09/05 |
| AZN-001 | Near 19 | Gadvan | Gd.5a.5c | Producer | 36 | 500 | 1481 | 14.24 | well opened at 13:30 on 1403/03/21 |
| AZN-012 | Near 75 | Gadvan | Gd.5a.5c | Suspended | 43 | _ | _ | 8.12 | establishment work over rig in program by PEDEC on 1400/04/24 |
| AZNS-026 | 23 | Gadvan | Gd.1-6 | Producer | 32 | 600 | 1250 | 5.78 | 10" line. Well opened at 21:20 in 1403/06/03 |
| AZN-020 | 7 | Gadvan | Gd.5c | Producer | 24 | 50 | 15 | 5.25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZNS-071 | 17 | Gadvan | Gd.5c | Producer | 28 | 550 | 965 | 3.32 | well closed from08:00 to 10:30 due to problem in control panel in 1403/08/06 |
| AZNS-047 | 77 | Gadvan | Gd.5a.5c | Producer | 30 | 1400 | 2840 | 2.8 | well opened at 14:30 on 1403/02/11 |
| AZNS-086 | 1 | Gadvan | Gd.5c | Producer | 36 | 700 | 1516 | 2.73 | due to the leakage in the 32 inch line manifold (2_3) wells were closed from west side of Karun and directed to the North gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-058 | 18 | Gadvan | Gd.5c | Producer | 32 | 700 | 1430 | 2.57 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |

Figure ‎3-13: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to January 2025)

Table ‎3-11: Cumulative oil production top 10 producer wells of Fahliyan reservoir (up to January 2025)

| Top 10 producer wells of Fahliyan |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Cum oil (MMSTB) | Remark |
| AZN-011 | Near 66 | Fahliyan | Fa.3a | Producer | 34 | 1200 | 2750 | 10.15 | well closed at 05:45 due to fault of panel and opened at 10:10 on 1403/02/04 |
| AZNS-049 | 69 | Fahliyan | Fa.1a.1c.3a | Producer | 32 | 1500 | 1160 | 6.55 | well opened at 20:30 in 1403/10/01 |
| AZNS-070 | 75 | Fahliyan | Fa.2a.3a | Producer | 30 | 1000 | 2265 | 6.45 | well was closed from 09:00 to 11:20 due to control panel problom. In 1403/09/28 |
| AZNS-046 | 62 | Fahliyan | Fa.1.3a | Producer | 49 | 1230 | 6800 | 4.1 | the well was closed from 10:15 to 13:15 to clean the stone trap in 1403/05/01 |
| AZNS-138 | 66 | Fahliyan | Fa.1c.2a | Producer | 32 | 1800 | 4168 | 4.02 | FIX CHOCK CHANGE OF 32/64 TO 34/64 in 1403/10/15 |
| AZNS-110 | 58 | Fahliyan | Fa.4a | Producer | 30 | 900 | 1588 | 2.63 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-121 | 44 | Fahliyan | Fa.1 | Producer | 34 | 2200 | 4200 | 2.58 | Well opened at 1:20 in 1403/03/29 |
| AZNS-148 | 77 | Fahliyan | Fa.2a.3a | Producer | 28 | 1500 | 2010 | 2.37 | crack operation done. Well closed at 13:00 due to change of 3sv needle and opened at 14:00 in 1403/05/03 |
| AZNS-050 | 57 | Fahliyan | Fa.4a | Producer | 28 | 900 | 1569 | 2.19 | well closed at 00:30 due to leakage of 20" line and opened at 04:15 in 1403/08/11 |
| AZNS-060 | 78 | Fahliyan | Fa.4a | Producer | 20 | 280 | 1030 | 1.91 | well reopened after rig down of subsurface gob facilities at 16:15. in 1403/10/26 |