گزارش ماهیانه بروزرسانی داده ها در نرم افزار OFM

Update Current OFM Database and Report Monthly

South Azadegan Oil Field

(February 2025)

کارفرما:

شرکت پترو پارس با مسئولیت محدود

مشاور:

شرکت خدمات مهندسی نفت کیش (KPE)

| 00 | February-2025 | Foad Saberi | Mehdi Pirayeh | Akbar Fotouhi |
| --- | --- | --- | --- | --- |
| Rev | Date | Prepared | Checked | Approved |

# Table of Contents

Title											        Page No.

Table of Contents	2

List of Figures	3

List of tables	4

Summary	6

1	OFM Project	6

2	Updated OFM Model	11

3	Updated OFM results	11

# List of Figures

Title											        Page No.

Figure ‎1-1: OFM Schema Tables	7

Figure ‎1-2: OFM base map	8

Figure ‎3-1: Cumulative oil production Comparison of South Azadegan oil field reservoirs	12

Figure ‎3-2: Well Status of South Azadegan Wells (Reservoir by Reservoir) (up to February 2025)	14

Figure ‎3-3: Cumulative total producer wells and active wells of south Azadegan oil field reservoirs (up to February 2025)	Error! Bookmark not defined.

Figure ‎3-4: Cumulative Production for South Azadegan field (up to February 2025)	49

Figure ‎3-5: Cumulative Production for Sarvak Reservoir of South Azadegan (up to February 2025)	50

Figure ‎3-6: Cumulative Production for Kazhdumi Reservoir of South Azadegan (up to February 2025)	51

Figure ‎3-7: Cumulative Production for Gadvan Reservoir of South Azadegan (up to February 2025)	52

Figure ‎3-8: Cumulative Production for Fahliyan Reservoir of South Azadegan (up to February 2025)	53

Figure ‎3-9: Cumulative oil production Bubble map and top 10 producer wells of South Azadegan Field (up to February 2025)	54

Figure ‎3-10: Cumulative oil production Bubble map and top 10 producer wells of Sarvak reservoir (up to February 2025)	55

Table ‎3-8: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to February 2025)	56

Table ‎3-9: Cumulative oil production top 10 producer wells of Sarvak-3 reservoir (up to February 2025)	57

Table ‎3-10: Cumulative oil production top 10 producer wells of Sarvak3-4-6 reservoir (up to February 2025)	58

Table ‎3-11: Cumulative oil production top 10 producer wells of Sarvak 8 reservoir (up to February 2025)	59

Figure ‎3-11: Cumulative oil production Bubble map and top 10 producer wells of Kazhdumi reservoir (up to February 2025)	60

62

Figure ‎3-12: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to February 2025)	62

Figure ‎3-13: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to February 2025)	64

# List of tables

Title											        Page No.

Table ‎3-1: Well status classification of existed wells of South Azadegan Field in OFM (up to February 2025)	13

Table ‎3-2: South Azadegan Wells which their status were suspended in February 2025	15

Table ‎3-3: South Azadegan Wells which their status were Ongoing in February 2025	16

Table ‎3-4: South Azadegan Wells which their status were completed in February 2025	17

Table ‎3-5: South Azadegan Wells which their status were closed in February 2025	18

Table ‎3-6: Wells with low THP (means: upstream THP-downstream THP<50 psi)	19

Table ‎3-7: South Azadegan wells status, oil production rate and cumulative	21

Table ‎3-8: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to February 2025)	56

Table ‎3-9: Cumulative oil production top 10 producer wells of Sarvak-3 reservoir (up to February 2025)	57

Table ‎3-10: Cumulative oil production top 10 producer wells of Sarvak3-4-6 reservoir (up to February 2025)	58

Table ‎3-11: Cumulative oil production top 10 producer wells of Sarvak 8 reservoir (up to February 2025)	59

Table ‎3-9: Cumulative oil production top 10 producer wells of Kazdhumi reservoir (up to February 2025)	61

Table ‎3-10: Cumulative oil production top 10 producer wells of Gadvan reservoir (up to February 2025)	63

Table ‎3-11: Cumulative oil production top 10 producer wells of Fahliyan reservoir (up to February 2025)	65

List of abbreviation

| Description | Abbreviation |
| --- | --- |
| Schlumberger Oil Field Manager | OFM |
| Total Depth | TD |
| Rotary Table Elevation | RTE |
| Gas Oil Ratio | GOR |
| Basic sediment and water | BSW |
| American Petroleum Institute gravity | API |
| Drill Stem Test | DST |
| Modular Formation Dynamics Tester | MDT |
| Wireline Formation Test | WFT |

# Summary

Following report includes the description of updated South Azadegan oil field OFM up to February 2025. The daily production table of OFM updated. Based on these new production data, this field produces around 151 MSTB/d in this month. The total number of producer wells of this field is 175 wells, which 145 wells are active on average in this month. The total cumulative oil production of the field is around 365.3MMSTB until the end of this month (the field produced 4.2 MMSTB oil during this month). It should be noted that other new data related to recent operations are added to OFM. These updates are descripted in Section 3 in detail.

# OFM Project

The current OFM project contains all related data of south Azadegan oil field (and Also south Yaran field). All received data for each well are given in their related tables. 207 completions (202 wellbores) in South Azadegan field (up to well AZNS-206) and 18 completions in South Yaran field are defined in this database. The Schema Tables related to this OFM project is depicted in Figure ‎1-1. Also, the base map of the field is shown in Figure ‎1-2.

Figure ‎1-1: OFM Schema Tables

Figure ‎1-2: OFM base map

The database of OFM model for South Azadegan field contains following tables:

## Header ID:

Header ID table of Azadegan OFM model defines the Azadegan wells and include information of drilling, production and geology of Azadegan field wells (TD, production interval, production formation, RTE, PVT and etc.).

## Daily Production

The daily production table, contains the daily production data of south Azadegan wells contains daily production rates (Oil, gas, water), chock size, GOR, BSW%, Wellhead pressure, downstream pressure, salt content, API, H2S content for the wells of south Azadegan and south Yaran fields.

## Completion

This table contains perforation depths intervals and existed plug (if any).

## H2S

In this table, the H2S content for those wells that the H2S are recorded and their depth intervals are given.

## Coring

The coring interval, recovery percent are given in this table.

## DST

In this table, all the information related to DST tests includes depth intervals, Formation and zone, test duration, production rate (oil, gas, and, water rates), ratios (GOR, BSW), chock size, gauge depth and gauge pressure and etc. are given.

## Production Tests

This table contains the information related to Production tests includes depth intervals, Formation and zone, test duration, production rate (oil, gas, and, water rates), ratios (GOR, BSW), choke size, gauge depth and gauge pressure, separator pressure and temperature, before and after choke pressure and choke temperature and, wellhead pressure and temperature are given.

## PLT

The PLT data includes oil and water rates, oil contributions and their depth intervals are given in this table.

## Formation Water

The results of water analysis include depth intervals, color, PH, salinity, and the chemical element percentages for those wells that have a water sample are gathered in this table.

## Static Pressure Tests

The results of the static surveys contain pressure gradient, pressure at datum and, pressure at the mid perforation could be found in this table.

## Flowing Pressure Tests

The results of the flowing surveys contain pressure gradient, pressure at datum and, pressure at the mid perforation could be found in this table.

## Stimulation

In the stimulation table, all data related to stimulation jobs (acidizing and clean-up), total volume of injected acid, injected acid per foot, depth intervals, main acid recipe, and, their PI values before and after acidizing) are given.

## WFT

The data related to wireline formation tests such as WFT type, gauge type, formation pressure and mobility (for MDT) are given.

# Updated OFM Model

- Production data of February 2025 is added to database.

# Updated OFM results

In this section, the summary of production data up to February 2025 related to south Azadegan reservoirs presented in Figure ‎3-1. 207 completions (202 wellbores) in South Azadegan field (up to well AZNS-206) and 12completions in South Yaran field are defined in this database. A classification related to well status of existed wells of South Azadegan field up to this month are listed in Table ‎3-1 and Depicted in Figure ‎3-2. This field produces around 151 MSTB/d with an average of 145 active wells in this month. The total number of producer wells is 175 wells. Error! Reference source not found. shows the total producer wells and active wells of south Azadegan oil field during the production history. The cumulative produced oil of the field up to this month is around 365.3 MMSTB (the field produces 4.2 MMSTB oil during this month). The cumulative production profile of field (and also for each reservoir) and bubble map of cumulative oil production for producer wells of south Azadegan field along with the list of top ten producing wells (for field and each reservoir individually)  are shown in Figure ‎3-4 to Figure 3-13 respectively.

| Reservoir; Cum Oil (MMSTB); Cum Gas (Bscf); Cum Water (MSTB). Sarvak; 178.4; 62.5; 1500.2. Kazhdumi; 38.3; 30.9; 2283.9. Gadvan; 101.2; 75.6; 584.4. Fahliyan; 47.4; 60.0; 187.5. Total; 365.3; 229.0; 4554.4. |  |
| --- | --- |

Figure ‎3-1: Cumulative oil production Comparison of South Azadegan oil field reservoirs

Table ‎3-1: Well status classification of existed wells of South Azadegan Field in OFM (up to February 2025)

| Well Status | No. of Wellbore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reservoir by Reservoir |  |  |  |  | Field |
|  | Sarvak | Kazhdumi | Gadvan | Fahliyan | Asmari(Disposal) | South Azadegan |
| Oil producer | 135 | 10 | 16 | 14 |  | 175 |
| Completed | 4 |  | 1 | 3 | 4 | 12 |
| On-going |  |  | 1 | 5 | 1 | 7 |
| Observation |  |  |  | 1 |  | 1 |
| Suspended | 1 |  | 4 | 3 |  | 8 |
| Total | 140 | 10 | 22 | 26 | 5 | 203 |

Figure ‎3-2: Well Status of South Azadegan Wells (Reservoir by Reservoir) (up to February 2025)

In the South Azadegan field, the following wells have been suspended: AZN-12, AZNS-51, AZNS-52, AZNS-75, AZNS-113, AZNS-174, AZNS-193, and AZNS-204. These wells are summarized in Table ‎3-2.

Table ‎3-2: South Azadegan Wells which their status were suspended in February 2025

| Item | Well | Formation | Pad | Remark |
| --- | --- | --- | --- | --- |
| 1 | AZN-012 | Gadvan | Near 75 | Planned for Workover by MSA |
| 2 | AZNS-051 | Gadvan | 14 | Planned for Workover by TDDC due to completion failure |
| 3 | AZNS-052 | Fahliyan | 63 | Planned for completion by TDDC |
| 4 | AZNS-075 | Gadvan | 80 | Planned for Workover by MSA |
| 5 | AZNS-113 | Fahliyan | 82 | Planned for Plug-Prong retrieval by NIDC |
| 6 | AZNS-174 | Fahliyan | 59 | Planned for Workover by NIDC due to completion failure |
| 7 | AZNS-193 | Sarvak | 14 | Planned for Workover by TDDC due to completion failure |
| 8 | AZNS-204 | Gadvan | 79 | Planned for completion by TDDC |

The following wells are classified in the ongoing operation category: AZNS-122, AZNS-175, AZNS-179, AZNS-196, AZNS-200, AZNS-201 and AZNS-207. These wells are summarized in Table ‎3-3.

Table ‎3-3: South Azadegan Wells which their status were Ongoing in February 2025

| Item | Well | Formation | Pad | Remark |
| --- | --- | --- | --- | --- |
| 1 | AZNS-122 | Fahliyan | 55 | Side Tracking Preparing Operation |
| 2 | AZNS-175 | Fahliyan | 92 | Decompletion-Recompletion-Acidizing |
| 3 | AZNS-179 | Asmari (Water Disposal) | 85 | Drilling 9 5/8 Section till ) |
| 4 | AZNS-196 | Fahliyan | 61 | Well Flowing & Clean UP |
| 5 | AZNS-200 | Fahliyan | 94 | Perforation Operation(7'' Perforated(TCP)) |
| 6 | AZNS-201 | Gadvan | 74 | Decompletion-Recompletion-Acidizing |
| 7 | AZNS-207 | Fahliyan | 93 | Drilling to TD |

In the South Azadegan field, the status of the following wells were completed in February 2025 which are summarized in Table ‎3-4.

Table ‎3-4: South Azadegan Wells which their status were completed in February 2025

| Item | Well | Pad | Formation | Well Status | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | AZNS-066 | 84 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 2 | AZNS-085 | 79 | Fahliyan | Completed | Waiting for production line and surface facilities installation |
| 3 | AZNS-116 | 81 | Fahliyan | Completed | Acidizing finished, Waiting for commissioning |
| 4 | AZNS-149 | 69 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 5 | AZNS-159 | 76 | Sarvak | Completed | Planned for ESP installation |
| 6 | AZNS-165 | 38 | Sarvak | Completed | Planned for Work-over or ESP installation |
| 7 | AZNS-183 | 59 | Sarvak | Completed | Planned for Acidizing by NIDC |
| 8 | AZNS-195 | 64 | Fahliyan | Completed | Planned for Acidizing by OEOC |
| 9 | AZNS-198 | 82 | Sarvak | Completed | Planned for Acidizing by NIDC |
| 10 | AZNS-202 | 87 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 11 | AZNS-205 | 86 | Asmari (Water Disposal) | Completed | Waiting for injection line and surface facilities installation |
| 12 | AZNS-206 | 82 | Gadvan | Completed | Waiting for Acidizing By NIDC |

In the South Azadegan field, the status of the following wells were closed in February 2025 which are summarized in Table ‎3-5.

Table ‎3-5: South Azadegan Wells which their status were closed in February 2025

| Item | Well Name | Formation | Zone | Pad Name | Closed Date | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | AZN-014 | Sarvak | Sv.3.4-6 | NEAR 73 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 2 | AZN-018 | Sarvak | Sv.3.4-6 | Pad02 | 16-06-24 | Wellhead pressure drop. The well pressure dropped suddenly at the end of May. 2025 |
| 3 | AZN-019 | Sarvak | Sv.3 | Pad13 | 04-10-21 | It probably closed due to water in the wellbore and pressure drop |
| 4 | AZN-020 | Gadvan | Gd.5c | Pad7 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 5 | AZNS-027 | Sarvak | Sv.3 | Pad4 | 05-01-25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 6 | AZNS-043 | Sarvak | Sv.3.4-6 | Pad11 | 05-03-24 | Wellhead pressure drop |
| 7 | AZNS-061 | Sarvak | Sv.3 | Pad77 | 26-05-24 | well closed due to ssv packing leakage at 10:40 and reopend at 14:45 after fix problom. In 1403/05/10 |
| 8 | AZNS-073 | Sarvak | Sv.3.4-6 | Pad05 | 25-05-22 | Wellhead pressure drop |
| 9 | AZNS-077 | Sarvak | Sv.3.4-6 | Pad13 | 31-12-22 | Wellhead pressure drop |
| 10 | AZNS-078 | Sarvak | Sv.3.4-6 | Pad63 | 14-05-22 | Delivered to PEDEC for workover operation on pad, however based on production data the last WHP was low |
| 11 | AZNS-081 | Sarvak | Sv.3.4-6 | Pad18 | 31-12-22 | Wellhead pressure drop, water production also reported, WHP drop could be due to water production |
| 12 | AZNS-088 | Sarvak | Sv.3 | Pad80 | 13-03-24 | Wellhead pressure drop |
| 13 | AZNS-100 | Sarvak | Sv..4-6 | Pad 2 | 07-01-25 | closed due to pressure drop in 1403/10/19 |
| 14 | AZNS-106 | Sarvak | Sv.3 | Pad81 | 29-04-24 | Delivered to PEDEC for Workover in other well on pad (AZMS-116) |
| 15 | AZNS-117 | Sarvak | Sv.3.4-6 | Pad53 | 25-05-22 | Wellhead pressure drop |
| 16 | AZNS-129 | Sarvak | Sv.8 | Pad12 | 07-01-25 | closed due to pressure drop in 1403/10/19 |
| 17 | AZNS-132 | Sarvak | Sv.3 | Pad73 | 31-08-24 | Wellhead pressure drop |
| 18 | AZNS-146 | Sarvak | Sv.3.4-6 | Pad70 | 31-08-24 | Wellhead pressure drop |
| 19 | AZNS-147 | Sarvak | Sv.3 | Pad71 | 16-06-24 | Wellhead pressure drop |
| 20 | AZNS-164 | Sarvak | Sv.3.4-6 | Pad29 | 31-08-24 | Wellhead pressure drop |
| 21 | AZNS-041 | Kazhdumi | Kz.2.4 | Pad 20 | 01-10-24 | well closed due to asphaltene barrier in the well on 1402/10/11 |
| 22 | AZNS-064 | Kazhdumi | Kz.2.4 | Pad13 | 05-03-20 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| 23 | AZNS-130 | Gadvan | Gd.5a | Pad 81 | 30-04-24 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| 24 | AZN-021 | Kazhdumi | Kz.2 | Pad 15 | 30-04-20 | closed due to pressure drop due to water production on 1399/02/11 |

In the South Azadegan field, the status of the following wells with low THP in February 2025 which are summarized in Table ‎3-7.

Table ‎3-6: Wells with low THP (means: upstream THP-downstream THP<50 psi)

| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (psi) | Down Stream (psi) | Cum (First prod--Last prod) MMSTB | Oil Rate (STBD) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AZN-014 | NEAR 73 | Sarvak | Sv.3.4-6 | Producer | 20 | 120 | 70 | 3.57 | 259 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 2 | AZN-015 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 80 | 70 | 6.04 | 502 | 12" line( well was closed due to pressure drop from 00:30 to 06:00) in 1403/09/29 |
| 3 | AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 120 | 80 | 2.62 | 272 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 4 | AZNS-036 | 50 | Sarvak | Sv.3 | Producer | 28 | 220 | 180 | 1.95 | 931 | Closed at11:20 due to air pressure drop & opened at 15:15 in 1403/05/05 |
| 5 | AZNS-057 | 25 | Sarvak | Sv.3.4-6 | Producer | 22 | 190 | 145 | 0.81 | 565 | well open at 02:30 in 1403/04/03 |
| 6 | AZNS-062 | 56 | Sarvak | Sv.3.4-6 | Producer | 23 | 220 | 180 | 1.05 | 689 | Well opened at 1:30 in 1403/03/29 |
| 7 | AZNS-065 | 69 | Sarvak | Sv.3 | Producer | 15 | 150 | 130 | 0.49 | _ | well opened at 18:00 witch chock size 15/64''. In 1403/08/13 |
| 8 | AZNS-074 | 83 | Sarvak | Sv.3 | Producer | 15 | 220 | 180 | 0.92 | 282 | well opened at 14:00 on 1403/02/11 |
| 9 | AZNS-084 | 47 | Sarvak | Sv.3.4-6 | Producer | 15 | 220 | 180 | 0.97 | 301 | well opened at 11:30 in 1403/07/01 |
| 10 | AZNS-091 | 83 | Sarvak | Sv.3 | Producer | 24 | 240 | 190 | 1.34 | 702 | well opened at 13:30 on 1403/02/11 |
| 11 | AZNS-093 | 93 | Sarvak | Sv.3.4-6 | Producer | 15 | 220 | 180 | 0.42 | 279 | Well opened at 2 in 1403/03/29 |
| 12 | AZNS-099 | 91 | Sarvak | Sv.8 | Producer | 20 | 170 | 170 | 1.02 | 281 | well opened at 13:30 in 1403/10/18 |
| 13 | AZNS-107 | 19 | Sarvak | Sv.3.4-6 | Producer | 26 | 150 | 150 | 1.69 | 214 | well opened at 15:00. In 1403/09/06 |
| 14 | AZNS-108 | 48 | Sarvak | Sv.3.4-6 | Producer | 30 | 220 | 180 | 0.84 | 1069 | Well opened at 2 in 1403/03/29 |
| 15 | AZNS-109 | 89 | Sarvak | Sv.8 | Producer | 15 | 220 | 180 | 0.44 | 241 | well reopened at 15:30 after finished slickline operation. In 1403/09/15 |
| 16 | AZNS-115 | 45 | Sarvak | Sv.3 | Producer | 15 | 200 | 180 | 0.33 | 272 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 17 | AZNS-125 | 53 | Sarvak | Sv.3.4-6 | Producer | 15 | 200 | 160 | 0.43 | 264 | Well opened at 2:40 in 1403/03/29 |
| 18 | AZNS-126 | 45 | Sarvak | Sv.3.4-6 | Producer | 20 | 220 | 180 | 0.42 | 505 | Well opened at 2:40 in 1403/03/29 |
| 19 | AZNS-150 | 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 190 | 160 | 0.44 | 1026 | well open at 17:00 in 1403/04/03 |
| 20 | AZNS-188 | 91 | Sarvak | Sv.8 | Producer | 24 | 200 | 170 | 0.46 | 683 | well opened at 09:45. In 1403/04/15 |

Table ‎3-7: South Azadegan wells status, oil production rate and cumulative

| Well | Pad | Formation | Zone | Well Status | BS&W (%) | GOR | Chock size (1/64)" | WHP (Psi) | Down Stream (psi) | WHT (C°) | API | H2S (PPM) | Cum (First prod--Last prod) MMSTB | Oil Rate (STBD) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AZN-001 | Near 19 | Gadvan | Gd.5a.5c | Producer | 8 | 818 | 36 | Min= 450 Max= 500 Ave= 471 Mode= 470 | Min= 60 Max= 75 Ave= 68 Mode= 70 | Min= 60 Max= 72 Ave= 69 Mode= 71 | 31 | 650 | 14.24 | Min= 1271 Max= 1739 Ave= 1481 | well opened at 13:30 on 1403/03/21 |
| AZN-002 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 32 | Min= 180 Max= 190 Ave= 186 Mode= 190 | Min= 80 Max= 95 Ave= 90 Mode= 90 | Min= 49 Max= 53 Ave= 51 Mode= 50 | 22.1 | 2000 | 5.97 | Min= 844 Max= 1145 Ave= 945 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-004 | Near 56 | Kazhdumi | Kz.1.2 | Producer | 7 | 916 | 32 | Min= 1420 Max=1450 Ave= 1427 Mode= 1420 | Min= 220 Max= 230 Ave= 224 Mode= 220 | Min= 83 Max= 84 Ave= 84 Mode= 84 | 28 | 20 | 9.82 | Min= 2556 Max= 3455 Ave= 2902 | the well was closed and opened for spool replacement on 1402/12/14 |
| AZN-005 | NEAR 1 | Sarvak | Sv.3.4-6 | Producer | 2 | 439 | 24 | Min= 30 Max= 120 Ave= 54 Mode= 50 | Min= 30 Max= 120 Ave= 53 Mode= 50 | Min= 10 Max= 29 Ave= 15 Mode= 13 | 21.4 | _ | 4.97 | Min= 108 Max= 302 Ave= 169 | well opened at 1:30 in 1403/03/10 |
| AZN-006 | NEAR 79 | Sarvak | Sv.4-6 | Producer | 0.6 | 362 | 36 | Min= 330 Max= 350 Ave= 338 Mode= 340 | Min= 100 Max= 120 Ave= 103 Mode= 100 | Min= 58 Max= 61 Ave= 60 Mode= 60 | 21.1 | _ | 13.67 | Min= 1514 Max= 2054 Ave= 1720 | well opened at 00:10 on 1403/03/10 |
| AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 0.3 | 439 | 34 | Min= 180 Max= 200 Ave= 190 Mode= 190 | Min= 90 Max= 100 Ave= 98 Mode= 100 | Min= 52 Max= 58 Ave= 55 Mode= 57 | 20.1 | _ | 10.28 | Min= 801 Max= 1076 Ave= 906 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| AZN-009 | 16 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 50 | Min= 870 Max= 940 Ave= 921 Mode= 920 | Min= 230 Max= 340 Ave= 323 Mode= 330 | Min= 77 Max= 78 Ave= 78 Mode= 78 | 32.3 | _ | 29.08 | Min= 3550 Max= 4730 Ave= 3982 | well closed dut to fault WK at 02:55 and opened at 13:30 on 1402/09/05 |
| AZN-010 | Near 50 | Fahliyan | Fa. 1a.1b.1c | Producer | _ | 1678 | 32 | Min= 2100 Max= 2400 Ave= 2156 Mode= 2100 | Min= 155 Max= 230 Ave= 177 Mode= 175 | Min= 73 Max= 74 Ave= 74 Mode= 74 | _ | _ | 0.2 | Min= 1769 Max= 3713 Ave= 3280 | well was closed due to control panel fault from9:15 to 10:25. in 1403/09/22 |
| AZN-011 | Near 66 | Fahliyan | Fa.3a | Producer | 0.1 | 1090 | 34 | Min= 1120 Max= 1290 Ave= 1185 Mode= 1200 | Min= 310 Max= 360 Ave= 334 Mode= 340 | Min= 72 Max= 76 Ave= 74 Mode= 74 | _ | _ | 10.15 | Min= 2414 Max= 3301 Ave= 2749 | well closed at 05:45 due to fault of panel and opened at 10:10 on 1403/02/04 |
| AZN-014 | NEAR 73 | Sarvak | Sv.3.4-6 | Producer | 1.2 | 362 | 20 | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | 21.5 | _ | 1.72 | Min= --- Max= --- Ave= --- | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZN-015 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 292 | 28 | Min= 50 Max= 175 Ave= 73 Mode= 70 | Min= 15 Max= 175 Ave= 67 Mode= 70 | Min= 8 Max= 45 Ave= 15 Mode= 10 | 21.4 | 1750 | 6.07 | Min= 194 Max= 664 Ave= 324 | 12" line( well was closed due to pressure drop from 00:30 to 06:00) in 1403/09/29 |
| AZN-016 | 25 | Sarvak | Sv.3.4-6 | Producer | 3 | 439 | 20 | Min= 60 Max= 290 Ave= 117 Mode= 80 | Min= 60 Max= 115 Ave= 89 Mode= 80 | Min= 10 Max= 37 Ave= 22 Mode= 17 | 18.7 | 1000 | 2.58 | Min= 145 Max= 469 Ave= 231 | well opened at00:50 in 1403/03/10 |
| AZN-017 | 50 | Kazhdumi | Kz.2 | Producer | 2 | 916 | 20 | Min= 1400 Max= 1480 Ave= 1442 Mode= 1450 | Min= 270 Max= 340 Ave= 303 Mode= 300 | Min= 44 Max= 78 Ave= 76 Mode= 77 | 28 | _ | 13.94 | Min= 1183 Max= 1607 Ave= 1346 | 10' line( well was closed from 11:00 to 16:00 due to MPFM connection) in 1403/08/12 |
| AZN-020 | 7 | Gadvan | Gd.5c | Producer | 0.6 | 1381 | 24 | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | 33.7 | _ | 5.25 | Min= --- Max= --- Ave= --- | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 1 | 292 | 20 | Min= 80 Max= 180 Ave= 119 Mode= 100 | Min= 70 Max= 130 Ave= 84 Mode= 70 | Min= 15 Max= 18 Ave= 17 Mode= 17 | _ | _ | 2.64 | Min= 204 Max= 426 Ave= 289 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZN-023 | Near 75 | Gadvan | Gd.5a.5c | Producer | 1 | 818 | 34 | Min= 1120 Max= 1150 Ave= 1136 Mode= 1150 | Min= 320 Max= 350 Ave= 343 Mode= 350 | Min= 77 Max= 79 Ave= 78 Mode= 78 | 32.9 | 23 | 16.3 | Min= 2625 Max= 3501 Ave= 2960 | well closed dut to fault WK at 03:10 and opened at 14:50 on 1402/09/05 |
| AZN-024 | Near 37 | Gadvan | Gd.3.5c.7 | Producer | 9 | 818 | 28 | Min= 520 Max= 570 Ave= 535 Mode= 530 | Min= 110 Max= 170 Ave= 124 Mode= 120 | Min= 70 Max= 73 Ave= 73 Mode= 73 | 23.6 | 5 | 1.96 | Min= 909 Max= 1222 Ave= 1020 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| AZNS-026 | 23 | Gadvan | Gd.1-6 | Producer | 0.1 | 1381 | 32 | Min= 550 Max= 570 Ave= 565 Mode= 570 | Min= 240 Max= 330 Ave= 255 Mode= 250 | Min= 44 Max= 47 Ave= 46 Mode= 46 | 32.3 | _ | 5.78 | Min= 1100 Max= 1457 Ave= 1234 | 10" line. Well opened at 21:20 in 1403/06/03 |
| AZNS-027 | 4 | Sarvak | Sv.3 | Producer | 1 | 439 | 15 | Min= --- Max= --- Ave= --- | Min= --- Max= --- Ave= --- | Min= --- Max= --- Ave= --- | 19.7 | _ | 1.1 | Min= --- Max= --- Ave= --- | well closed due to pressure drop at12:00. in 1403/10/17 |
| AZNS-028 | 11 | Kazhdumi | Kz.2.3.4 | Producer | 7 | 916 | 24 | Min= 230 Max= 400 Ave= 263 Mode= 230 | Min= 125 Max= 150 Ave= 135 Mode= 130 | Min= 25 Max= 31 Ave= 28 Mode= 28 | 29.3 | _ | 0.54 | Min= 309 Max= 989 Ave= 728 | well opened at 13:30 in 1403/11/17 |
| AZNS-029 | 5 | Kazhdumi | Kz.3.4 | Producer | 5 | 916 | 28 | Min= 200 Max= 270 Ave= 223 Mode= 220 | Min= 65 Max= 135 Ave= 95 Mode= 80 | Min= 26 Max= 35 Ave= 31 Mode= 33 | 25.9 | _ | 0.48 | Min= 437 Max= 604 Ave= 518 | well closed at 11:00 due to disconnect of MPFM and opened at 17:30 in 1403/07/01 |
| AZNS-030 | 27 | Sarvak | Sv.3 | Producer | 1 | 439 | 28 | Min= 250 Max= 300 Ave= 286 Mode= 290 | Min= 180 Max= 195 Ave= 186 Mode= 180 | Min= 48 Max= 52 Ave= 50 Mode= 50 | 20.9 | _ | 1.53 | Min= 1031 Max= 1557 Ave= 1231 | well open at 17:00 in 1403/04/03 |
| AZNS-032 | 21 | Sarvak | Sv.3 | Producer | 0.2 | 439 | 28 | Min= 230 Max= 300 Ave= 270 Mode= 280 | Min= 120 Max= 130 Ave= 127 Mode= 130 | Min= 47 Max= 50 Ave= 49 Mode= 49 | 20.5 | _ | 0.21 | Min= 910 Max= 1496 Ave= 1181 | well closed at 10:00 due to well down leakage and opened at 17 :00in 1403/10/07 |
| AZNS-033 | 7 | Sarvak | Sv.3 | Producer | 0.1 | 439 | 28 | Min= 290 Max= 350 Ave= 301 Mode= 300 | Min= 135 Max= 155 Ave= 147 Mode= 155 | Min= 52 Max= 57 Ave= 56 Mode= 56 | 19 | _ | 2.46 | Min= 828 Max= 1112 Ave= 938 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to th rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-034 | 23 | Sarvak | Sv.3 | Producer | 0.5 | 439 | 32 | Min= 360 Max= 390 Ave= 364 Mode= 360 | Min= 65 Max= 190 Ave= 107 Mode= 65 | Min= 34 Max= 46 Ave= 42 Mode= 43 | 18.1 | _ | 2.09 | Min= 1205 Max= 1620 Ave= 1374 | 10" line.well opened at 12:05 in 1403/06/03 |
| AZNS-035 | 33 | Sarvak | Sv.3 | Producer | 0.3 | 292 | 24 | Min= 320 Max= 380 Ave= 339 Mode= 340 | Min= 145 Max= 160 Ave= 150 Mode= 150 | Min= 30 Max= 38 Ave= 35 Mode= 35 | 20.32 | _ | 0.63 | Min= 1116 Max= 1656 Ave= 1283 | well closed at 12:05 due to change of 2sv packing and opened at 13:15 in 1403/08/02 |
| AZNS-036 | 50 | Sarvak | Sv.3 | Producer | 0.5 | 292 | 28 | Min= 210 Max= 250 Ave= 222 Mode= 220 | Min= 165 Max= 190 Ave= 176 Mode= 170 | Min= 17 Max= 29 Ave= 24 Mode= 22 | 21.77 | _ | 2 | Min= 445 Max= 959 Ave= 835 | Closed at11:20 due to air pressure drop & opened at 15:15 in 1403/05/05 |
| AZNS-037 | 7 | Sarvak | Sv.3 | Producer | 0.2 | 439 | 24 | Min= 230 Max= 250 Ave= 243 Mode= 240 | Min= 130 Max= 155 Ave= 142 Mode= 145 | Min= 49 Max= 56 Ave= 51 Mode= 51 | 19.1 | _ | 2.21 | Min= 521 Max= 703 Ave= 602 | well reopened at 15:00. in 1403/07/03 |
| AZNS-038 | 21 | Sarvak | Sv.3 | Producer | 0.3 | 439 | 28 | Min= 360 Max= 400 Ave= 378 Mode= 380 | Min= 150 Max= 175 Ave= 160 Mode= 150 | Min= 36 Max= 43 Ave= 41 Mode= 42 | 20.7 | _ | 0.19 | Min= 936 Max= 1240 Ave= 1054 | the well was closed from 02:40 to 17:00 due to high salt in 1403/11/05 |
| AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 6 | 439 | 28 | Min= 330 Max= 370 Ave= 340 Mode= 340 | Min= 230 Max= 310 Ave= 242 Mode= 240 | Min= 51 Max= 56 Ave= 54 Mode= 54 | 18 | _ | 4.3 | Min= 870 Max= 1183 Ave= 994 | 10" line.well opened at 13:30 in 1403/06/04 |
| AZNS-040 | 50 | Fahliyan | Fa.1b.1c | Producer | 0.1 | 1678 | 32 | Min= 1300 Max= 1400 Ave= 1367 Mode= 1400 | Min= 160 Max= 180 Ave= 173 Mode= 175 | Min= 55 Max= 62 Ave= 59 Mode= 58 | 36.39 | _ | 0.64 | Min= 1245 Max= 2436 Ave= 2267 | closed at 12:25 pm by order of management in 1403/06/12 |
| AZNS-042 | 10 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 439 | 15 | Min= 220 Max= 260 Ave= 247 Mode= 240 | Min= 140 Max= 160 Ave= 149 Mode= 150 | Min= 10 Max= 25 Ave= 18 Mode= 16 | 20.8 | _ | 1.19 | Min= 224 Max= 313 Ave= 269 | closed due to pressure drop in 1403/10/19 |
| AZNS-044 | 15 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 15 | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | 20.2 | _ | 0.52 | Min= --- Max= --- Ave= --- | closed due to pressure drop in 1403/10/19 |
| AZNS-045 | 56 | Fahliyan | Fa.4a.4b | Producer | 0.1 | 1090 | 28 | Min= 170 Max= 920 Ave= 612 Mode= 750 | Min= 160 Max= 190 Ave= 178 Mode= 190 | Min= 8 Max= 52 Ave= 38 Mode= 48 | 35.5 | _ | 0.71 | Min= 418 Max= 1599 Ave= 1082 | well open at 22:30 in 1403/11/06 |
| AZNS-046 | 62 | Fahliyan | Fa.1.3a | Producer | 0.1 | 1678 | 49 | Min= 1200 Max= 1300 Ave= 1196 Mode= 1250 | Min= 120 Max= 145 Ave= 132 Mode= 130 | Min= 65 Max= 190 Ave= 107 Mode= 65 | 36.7 | _ | 4.1 | Min= 5110 Max= 7368 Ave= 6055 | the well was closed from 10:15 to 13:15 to clean the stone trap in 1403/05/01 |
| AZNS-047 | 77 | Gadvan | Gd.5a.5c | Producer | 0.1 | 818 | 30 | Min= 1160 Max= 1420 Ave= 1343 Mode= 1300 | Min= 120 Max= 185 Ave= 175 Mode= 185 | Min= 62 Max= 66 Ave= 65 Mode= 66 | 32.3 | _ | 2.8 | Min= 1474 Max= 2916 Ave= 2676 | well opened at 14:30 on 1403/02/11 |
| AZNS-048 | 20 | Sarvak | Sv.3.4-6 | Producer | 1.2 | 439 | 24 | Min= 170 Max= 280 Ave= 198 Mode= 200 | Min= 125 Max= 170 Ave= 139 Mode= 130 | Min= 25 Max= 47 Ave= 32 Mode= 30 | 20 | _ | 1.48 | Min= 544 Max= 1188 Ave= 702 | well opened at 1:05 in 1403/03/10 |
| AZNS-049 | 69 | Fahliyan | Fa.1a.1c.3a | Producer | 0.1 | 1678 | 32 | Min= 1200 Max= 1600 Ave= 1433 Mode= 1500 | Min= 140 Max= 220 Ave= 153 Mode= 150 | Min= 70 Max= 78 Ave= 74 Mode= 75 | 32.2 | _ | 6.55 | Min= 748 Max= 4455 Ave= 3116 | well closed at 10:50 and discconnect of stone trap and chocks then opened at16:20 by chock manifold contractor in 1403/11/09 |
| AZNS-050 | 57 | Fahliyan | Fa.4a | Producer | 0.2 | 1090 | 28 | Min= 800 Max= 900 Ave= 861 Mode= 870 | Min= 155 Max= 180 Ave= 172 Mode= 180 | Min= 51 Max= 60 Ave= 56 Mode= 58 | _ | _ | 2.19 | Min= 800 Max= 1671 Ave= 1450 | well wqas closed from 16:25 to 18:00 due to 20'' line pig running. In 1403/11/27 |
| AZNS-053 | 51 | Sarvak | Sv.3 | Producer | 3.6 | 292 | 20 | Min= 180 Max= 320 Ave= 209 Mode= 180 | Min= 175 Max= 200 Ave= 184 Mode= 180 | Min= 8 Max= 28 Ave= 19 Mode= 22 | 21 | _ | 1.24 | Min= 206 Max= 622 Ave= 433 | Well opened at 3 in 1403/03/29 |
| AZNS-054 | 15 | Sarvak | Sv.3.4-6 | Producer | 0.3 | 439 | 15 | Min= 140 Max= 280 Ave= 195 Mode= 200 | Min= 135 Max= 160 Ave= 147 Mode= 140 | Min= 8 Max= 36 Ave= 25 Mode= 26 | 20.7 | _ | 1.01 | Min= 168 Max= 310 Ave= 222 | closed at 10:10 due to air capsool discgarge and reopened at 12:00 reopened after recharge in 1403/06/20 |
| AZNS-056 | 5 | Sarvak | Sv.3.4-6 | Producer | 1.2 | 439 | 22 | Min= 230 Max= 330 Ave= 320 Mode= 320 | 155 | Min= 28 Max= 43 Ave= 34 Mode= 35 | 18.7 | _ | 1.68 | Min= 442 Max= 762 Ave= 641 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-057 | 25 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 22 | Min= 160 Max= 190 Ave= 176 Mode= 180 | Min= 130 Max= 135 Ave= 132 Mode= 130 | Min= 29 Max= 35 Ave= 32 Mode= 34 | 20.4 | _ | 0.84 | Min= 466 Max= 688 Ave= 549 | well open at 02:30 in 1403/04/03 |
| AZNS-058 | 18 | Gadvan | Gd.5c | Producer | 0.3 | 1381 | 32 | Min= 640 Max= 700 Ave= 675 Mode= 670 | Min= 130 Max= 150 Ave= 137 Mode= 140 | Min= 36 Max= 46 Ave= 42 Mode= 41 | 30.6 | _ | 2.57 | Min= 1260 Max= 1700 Ave= 1423 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| AZNS-059 | 20 | Sarvak | Sv.3.4-6 | Producer | 2 | 439 | 28 | Min= 220 Max= 330 Ave= 245 Mode= 250 | Min= 125 Max= 160 Ave= 138 Mode= 125 | Min= 23 Max= 31 Ave= 26 Mode= 29 | 18.9 | _ | 2.14 | Min= 863 Max= 1763 Ave= 1082 | well opened at 1:05 in 1403/03/10 |
| AZNS-062 | 56 | Sarvak | Sv.3.4-6 | Producer | 0.6 | 292 | 23 | Min= 200 Max= 230 Ave= 220 Mode= 220 | Min= 160 Max= 195 Ave= 183 Mode= 195 | Min= 26 Max= 33 Ave= 27 Mode= 27 | 22.1 | _ | 1.09 | Min= 330 Max= 654 Ave= 592 | Well opened at 1:30 in 1403/03/29 |
| AZNS-063 | 51 | Sarvak | Sv.3.4-6 | Producer | 0.8 | 292 | 24 | Min= 175 Max= 330 Ave= 230 Mode= 230 | Min= 120 Max= 190 Ave= 178 Mode= 185 | Min= 5 Max= 32 Ave= 21 Mode= 22 | 21.4 | _ | 1.21 | Min= 355 Max= 916 Ave= 659 | closed at 12:20 pm by order of management in 1403/06/12 |
| AZNS-065 | 69 | Sarvak | Sv.3 | Producer | 1 | 361 | 15 | Min= 110 Max= 200 Ave= 138 Mode= 130 | Min= 110 Max= 140 Ave= 126 Mode= 130 | Min= 12 Max= 28 Ave= 19 Mode= 18 | 21.21 | _ | 0.5 | Min= 220 Max= 457 Ave= 273 | well closed due to pressure drop at 11:50. in 1403/12/05 |
| AZNS-067 | 18 | Sarvak | Sv.3.4-6 | Producer | 2 | 439 | 28 | Min= 300 Max= 350 Ave= 335 Mode= 340 | Min= 140 Max= 165 Ave= 147 Mode= 145 | Min= 32 Max= 43 Ave= 40 Mode= 40 | 18.6 | _ | 1.66 | Min= 859 Max= 1207 Ave= 1003 | installation of MPFM at 20:45 by chock 24/64@1403/07/24 |
| AZNS-068 | 6 | Sarvak | Sv.8 | Producer | 6.5 | 439 | 15 | Min= 120 Max= 450 Ave= 230 Mode= 250 | Min= 120 Max= 160 Ave= 139 Mode= 135 | Min= 14 Max= 32 Ave= 24 Mode= 24 | 18.7 | _ | 1.07 | Min= 141 Max= 406 Ave= 238 | well closed at 10:40due to leakage of 32" and high salt and opened at 18:10 in 1403/08/03 |
| AZNS-069 | 10 | Sarvak | Sv.3.4-6 | Producer | 3.9 | 439 | 15 | Min= 230 Max= 260 Ave= 241 Mode= 240 | Min= 140 Max= 175 Ave= 158 Mode= 160 | Min= 25 Max= 32 Ave= 29 Mode= 30 | 19.9 | _ | 1.04 | Min= 224 Max= 311 Ave= 254 | closed due to pressure drop on 1402/12/14 |
| AZNS-070 | 75 | Fahliyan | Fa.2a.3a | Producer | 0.5 | 1678 | 30 | Min= 1100 Max= 1620 Ave= 1351 Mode= 1450 | Min= 135 Max= 195 Ave= 182 Mode= 190 | Min= 65 Max= 74 Ave= 71 Mode= 73 | 29.6 | _ | 6.45 | Min= 1122 Max= 2489 Ave= 1996 | well closed at 10:15 to 11: 00 due to air pressur drop in 1403/12/07 |
| AZNS-071 | 17 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 28 | Min= 500 Max= 600 Ave= 554 Mode= 540 | Min= 225 Max= 265 Ave= 252 Mode= 250 | Min= 70 Max= 72 Ave= 71 Mode= 71 | 31 | _ | 3.32 | Min= 809 Max= 1187 Ave= 963 | well closed from08:00 to 10:30 due to problom in control panel in 1403/08/06 |
| AZNS-072 | 57 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 292 | 15 | Min= 180 Max= 300 Ave= 208 Mode= 190 | Min= 110 Max= 170 Ave= 159 Mode= 165 | 20 | 20 | _ | 0.65 | Min= 163 Max= 389 Ave= 267 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-074 | 83 | Sarvak | Sv.3 | Producer | 0.25 | 361 | 15 | Min= 120 Max= 180 Ave= 153 Mode= 160 | Min= 120 Max= 180 Ave= 152 Mode= 160 | Min= 10 Max= 20 Ave= 14 Mode= 12 | 21.2 | _ | 0.93 | Min= 100 Max= 227 Ave= 185 | well closed due to pressure drop at 11:30. in 1403/12/05 |
| AZNS-076 | 11 | Sarvak | Sv.3.4-6 | Producer | 0.3 | 439 | 15 | Min= 220 Max= 260 Ave= 238 Mode= 240 | Min= 135 Max= 160 Ave= 145 Mode= 145 | Min= 20 Max= 34 Ave= 24 Mode= 23 | 21 | _ | 0.96 | Min= 233 Max= 313 Ave= 261 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-079 | 78 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 361 | 26 | Min= 330 Max= 350 Ave= 340 Mode= 340 | Min= 125 Max= 195 Ave= 175 Mode= 180 | Min= 30 Max= 44 Ave= 39 Mode= 40 | 21.4 | _ | 2.35 | Min= 517 Max= 1037 Ave= 960 | closed due to pressure drop on 1402/12/23 |
| AZNS-080 | 90 | Sarvak | Sv.8 | Producer | 4 | 439 | 15 | Min= 200 Max= 260 Ave= 240 Mode= 240 | Min= 135 Max= 155 Ave= 145 Mode= 140 | Min= 27 Max= 34 Ave= 30 Mode= 32 | 18.8 | _ | 1.07 | Min= 204 Max= 302 Ave= 254 | well closed at 00:50 due to leakage of 20" line and opened at 10:40 in 1403/08/11 |
| AZNS-082 | 69 | Sarvak | Sv.3 | Producer | 0.1 | 361 | 24 | Min= 230 Max= 280 Ave= 254 Mode= 250 | Min= 100 Max= 110 Ave= 105 Mode= 110 | Min= 36 Max= 40 Ave= 38 Mode= 37 | 21 | _ | 1.99 | Min= 786 Max= 1301 Ave= 933 | well opened at 15:20 in 1403/03/10 |
| AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 28 | Min= 240 Max= 280 Ave= 264 Mode= 260 | Min= 135 Max= 155 Ave= 145 Mode= 140 | Min= 31 Max= 39 Ave= 35 Mode= 37 | 20.6 | _ | 2.73 | Min= 711 Max= 990 Ave= 843 | well diverted to MPFM due to well test operation at 17:00 with chock size 28/64". In 1403/07/03 |
| AZNS-084 | 47 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 292 | 15 | Min= 175 Max= 290 Ave= 215 Mode= 190 | Min= 160 Max= 190 Ave= 179 Mode= 180 | Min= 10 Max= 33 Ave= 21 Mode= 20 | 22.4 | _ | 0.99 | Min= 140 Max= 365 Ave= 274 | well opened at 11:30 in 1403/07/01 |
| AZNS-086 | 1 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 36 | Min= 570 Max= 650 Ave= 620 Mode= 620 | Min= 170 Max= 200 Ave= 179 Mode= 175 | Min= 33 Max= 64 Ave= 40 Mode= 42 | 34.4 | _ | 2.73 | Min= 1148 Max= 1602 Ave= 1349 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-087 | 8 | Sarvak | Sv.8 | Producer | 0.4 | 439 | 15 | Min= 200 Max= 240 Ave= 221 Mode= 220 | Min= 135 Max= 155 Ave= 143 Mode= 140 | Min= 22 Max= 40 Ave= 27 Mode= 26 | 20 | _ | 0.68 | Min= 208 Max= 302 Ave= 246 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-089 | 78 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 361 | 20 | Min= 220 Max= 240 Ave= 231 Mode= 230 | Min= 120 Max= 195 Ave= 174 Mode= 180 | Min= 20 Max= 33 Ave= 27 Mode= 24 | 22.3 | _ | 1.71 | Min= 239 Max= 479 Ave= 445 | well opened at 14:10 on 1403/02/11 |
| AZNS-090 | 90 | Sarvak | Sv.8 | Producer | 4.9 | 439 | 22 | Min= 240 Max= 270 Ave= 261 Mode= 260 | Min= 130 Max= 155 Ave= 143 Mode= 140 | Min= 26 Max= 34 Ave= 31 Mode= 32 | 19.2 | _ | 1.49 | Min= 461 Max= 652 Ave= 533 | well closed at 00:55 due to leakage of 20" line and opened at 10:40 in 1403/08/11 |
| AZNS-091 | 83 | Sarvak | Sv.3 | Producer | 0.2 | 361 | 24 | Min= 230 Max= 250 Ave= 239 Mode= 240 | Min= 130 Max= 200 Ave= 184 Mode= 190 | Min= 34 Max= 42 Ave= 36 Mode= 34 | 19.9 | _ | 1.38 | Min= 339 Max= 689 Ave= 629 | well opened at 13:30 on 1403/02/11 |
| AZNS-092 | 19 | Kazhdumi | Kz.2.4 | Producer | 0.1 | 916 | 32 | Min= 1000 Max= 1050 Ave= 1034 Mode= 1040 | Min= 130 Max= 185 Ave= 175 Mode= 180 | Min= 15 Max= 78 Ave= 70 Mode= 76 | 31.6 | _ | 5.34 | Min= 2112 Max= 2817 Ave= 2376 | well opened at00:50 in 1403/03/10 |
| AZNS-093 | 93 | Sarvak | Sv.3.4-6 | Producer | 0.6 | 292 | 15 | Min= 170 Max= 250 Ave= 199 Mode= 180 | Min= 160 Max= 190 Ave= 180 Mode= 190 | Min= 6 Max= 23 Ave= 17 Mode= 15 | 19.8 | _ | 0.44 | Min= 168 Max= 307 Ave= 258 | Well opened at 2 in 1403/03/29 |
| AZNS-094 | 89 | Sarvak | Sv.8 | Producer | 1.8 | 439 | 15 | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | 19.9 | _ | 0.35 | Min= --- Max= --- Ave= --- | closed due to pressure drop in 1403/10/19 |
| AZNS-095 | 31 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 32 | Min= 220 Max= 250 Ave= 236 Mode= 240 | Min= 100 Max= 130 Ave= 120 Mode= 120 | Min= 20 Max= 26 Ave= 23 Mode= 20 | 18.4 | _ | 2.52 | Min= 884 Max= 1091 Ave= 970 | well opened at00:50 in 1403/03/10 |
| AZNS-096 | 14 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 24 | Min= 190 Max= 200 Ave= 193 Mode= 190 | Min= 130 Max= 135 Ave= 131 Mode= 130 | Min= 18 Max= 26 Ave= 22 Mode= 22 | 19.1 | _ | 0.83 | Min= 437 Max= 612 Ave= 501 | well closed at 13:15 due to change adjustable chock and opened at 14:45 in 1403/04/09 |
| AZNS-097 | 8 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 16 | Min= 190 Max= 240 Ave= 215 Mode= 200 | Min= 135 Max= 155 Ave= 144 Mode= 140 | Min= 24 Max= 36 Ave= 28 Mode= 30 | 20.9 | _ | 1.38 | Min= 219 Max= 350 Ave= 269 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-098 | 46 | Sarvak | Sv.3 | Producer | 1.2 | 292 | 30 | Min= 180 Max= 280 Ave= 252 Mode= 280 | Min= 170 Max= 190 Ave= 183 Mode= 190 | Min= 9 Max= 28 Ave= 20 Mode= 15 | 21.9 | _ | 0.74 | Min= 572 Max= 1193 Ave= 1035 | Well opened at 2:10 in 1403/03/29 |
| AZNS-099 | 91 | Sarvak | Sv.8 | Producer | 8 | 439 | 20 | Min= 125 Max= 270 Ave= 163 Mode= 160 | Min= 125 Max= 180 Ave= 155 Mode= 150 | Min= 12 Max= 32 Ave= 20 Mode= 21 | 18.2 | _ | 1.04 | Min= 251 Max= 395 Ave= 291 | well opened at 13:30 in 1403/10/18 |
| AZNS-100 | 1 | Sarvak | Sv.4-6 | Producer | 0.2 | 439 | 15 | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | 19 | _ | 0.25 | Min= --- Max= --- Ave= --- | closed due to pressure drop in 1403/10/19 |
| AZNS-101 | 47 | Sarvak | Sv.3.4-6 | Producer | 1.4 | 292 | 20 | Min= 175 Max= 300 Ave= 224 Mode= 250 | Min= 170 Max= 195 Ave= 181 Mode= 175 | Min= 11 Max= 32 Ave= 22 Mode= 18 | 20.4 | _ | 1.59 | Min= 239 Max= 617 Ave= 468 | Well opened at 2:50 in 1403/03/29 |
| AZNS-102 | 75 | Sarvak | Sv.3 | Producer | 0.1 | 361 | 28 | Min= 230 Max= 250 Ave= 247 Mode= 250 | Min= 130 Max= 190 Ave= 178 Mode= 190 | Min= 34 Max= 45 Ave= 41 Mode= 40 | 21.1 | _ | 2.32 | Min= 444 Max= 909 Ave= 845 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| AZNS-103 | 12 | Gadvan | Gd.5c | Producer | 0.1 | 1381 | 32 | Min= 550 Max= 600 Ave= 571 Mode= 570 | Min= 140 Max= 155 Ave= 148 Mode= 145 | Min= 50 Max= 57 Ave= 54 Mode= 55 | 31.1 | _ | 1.84 | Min= 1474 Max= 2144 Ave= 1718 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| AZNS-104 | 52 | Sarvak | Sv.3 | Producer | 0.3 | 292 | 15 | Min= 300 Max= 330 Ave= 323 Mode= 320 | Min= 120 Max= 180 Ave= 160 Mode= 175 | Min= 14 Max= 27 Ave= 23 Mode= 22 | 21 | _ | 1.06 | Min= 211 Max= 418 Ave= 381 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-105 | 16 | Sarvak | Sv.8 | Producer | 0.1 | 439 | 15 | Min= 140 Max= 220 Ave= 183 Mode= 200 | Min= 130 Max= 155 Ave= 143 Mode= 140 | Min= 15 Max= 30 Ave= 20 Mode= 18 | 20.1 | _ | 0.9 | Min= 168 Max= 281 Ave= 210 | well closed at 12:00 due to pressure drop in 1403/07/01 |
| AZNS-107 | 19 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 26 | Min= 200 Max= 220 Ave= 210 Mode= 210 | Min= 120 Max= 125 Ave= 121 Mode= 120 | Min= 23 Max= 28 Ave= 26 Mode= 26 | 17.7 | _ | 1.75 | Min= 711 Max= 1093 Ave= 851 | well opened at 15:00 . In 1403/09/06 |
| AZNS-108 | 48 | Sarvak | Sv.3.4-6 | Producer | 0.6 | 292 | 30 | Min= 210 Max= 250 Ave= 229 Mode= 220 | Min= 175 Max= 200 Ave= 188 Mode= 195 | Min= 30 Max= 38 Ave= 35 Mode= 34 | 19.6 | _ | 0.9 | Min= 542 Max= 1065 Ave= 971 | Well opened at 2 in 1403/03/29 |
| AZNS-109 | 89 | Sarvak | Sv.8 | Producer | 1 | 439 | 15 | Min= 160 Max= 250 Ave= 221 Mode= 240 | Min= 160 Max= 190 Ave= 176 Mode= 175 | Min= 11 Max= 23 Ave= 18 Mode= 17 | 20.6 | _ | 0.45 | Min= 176 Max= 313 Ave= 248 | well reopened at 15:30 after finished slickline operation. In 1403/09/15 |
| AZNS-110 | 58 | Fahliyan | Fa.4a | Producer | 0.1 | 1090 | 30 | Min= 650 Max= 780 Ave= 739 Mode= 700 | Min= 120 Max= 190 Ave= 171 Mode= 180 | Min= 60 Max= 66 Ave= 64 Mode= 64 | _ | _ | 2.63 | Min= 827 Max= 1638 Ave= 1449 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-111 | 46 | Sarvak | Sv.3 | Producer | 1.6 | 292 | 24 | Min= 220 Max= 270 Ave= 251 Mode= 260 | Min= 165 Max= 190 Ave= 177 Mode= 180 | Min= 18 Max= 30 Ave= 25 Mode= 28 | 20.93 | _ | 0.5 | Min= 389 Max= 774 Ave= 701 | closed due to pressure drop in 1403/06/28 |
| AZNS-112 | 75 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 361 | 15 | Min= 280 Max= 290 Ave= 281 Mode= 280 | Min= 130 Max= 195 Ave= 181 Mode= 190 | Min= 29 Max= 43 Ave= 40 Mode= 40 | 22 | _ | 1.76 | Min= 170 Max= 350 Ave= 316 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| AZNS-114 | 1 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 439 | 15 | Min= 330 Max= 360 Ave= 346 Mode= 350 | Min= 175 Max= 200 Ave= 187 Mode= 185 | Min= 25 Max= 36 Ave= 32 Mode= 33 | 19.7 | _ | 0.48 | Min= 314 Max= 415 Ave= 354 | _ |
| AZNS-115 | 45 | Sarvak | Sv.3 | Producer | 4.2 | 292 | 15 | Min= 190 Max= 240 Ave= 207 Mode= 200 | Min= 170 Max= 190 Ave= 177 Mode= 170 | Min= 11 Max= 25 Ave= 15 Mode= 11 | 19.7 | _ | 0.34 | Min= 88 Max= 271 Ave= 237 | well opened at 17:00. in 1403/12/05 |
| AZNS-118 | 26 | Sarvak | Sv.3.4-6 | Producer | 3.4 | 439 | 24 | Min= 240 Max= 260 Ave= 249 Mode= 250 | Min= 145 Max= 175 Ave= 156 Mode= 145 | Min= 24 Max= 31 Ave= 27 Mode= 27 | 20.9 | _ | 0.36 | Min= 521 Max= 713 Ave= 598 | well open at 16:20 in 1403/04/03 |
| AZNS-119 | 52 | Sarvak | Sv.3 | Producer | 0.1 | 292 | 15 | Min= 220 Max= 240 Ave= 232 Mode= 230 | Min= 120 Max= 180 Ave= 159 Mode= 165 | Min= 12 Max= 26 Ave= 22 Mode= 22 | 21.8 | _ | 0.93 | Min= 163 Max= 321 Ave= 291 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-120 | NL-2-5 | Gadvan | Gd.5c | Producer | 0 | 818 | 32 | Min= 290 Max= 320 Ave= 299 Mode= 300 | Min= 110 Max= 190 Ave= 131 Mode= 110 | Min= 18 Max= 35 Ave= 32 Mode= 33 | _ | _ | 0.04 | Min= 801 Max= 1628 Ave= 1015 | well was closed from10:40 to 22:00 due to low pressure after chock activataed . In 1403/11/03 |
| AZNS-121 | 44 | Fahliyan | Fa.1 | Producer | 0.1 | 1678 | 34 | Min= 2100 Max= 2250 Ave= 2179 Mode= 2200 | Min= 370 Max= 385 Ave= 373 Mode= 370 | Min= 97 Max= 100 Ave= 98 Mode= 98 | 34.1 | _ | 2.58 | Min= 2202 Max= 4358 Ave= 4056 | Well opened at 1:20 in 1403/03/29 |
| AZNS-124 | 49 | Sarvak | Sv.3 | Producer | 1 | 292 | 30 | Min= 220 Max= 240 Ave= 228 Mode= 230 | Min= 170 Max= 200 Ave= 184 Mode= 190 | Min= 11 Max= 26 Ave= 18 Mode= 18 | 19.3 | _ | 0.88 | Min= 523 Max= 1058 Ave= 965 | repened at 12:15 in 1403/06/28 |
| AZNS-125 | 53 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 292 | 15 | Min= 200 Max= 220 Ave= 210 Mode= 210 | Min= 160 Max= 165 Ave= 164 Mode= 165 | Min= 12 Max= 20 Ave= 17 Mode= 19 | 21.9 | _ | 0.44 | Min= 141 Max= 304 Ave= 264 | well opened at 11:45. in 1403/12/05 |
| AZNS-126 | 45 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 292 | 20 | Min= 180 Max= 260 Ave= 204 Mode= 200 | Min= 170 Max= 200 Ave= 183 Mode= 190 | Min= 9 Max= 28 Ave= 22 Mode= 24 | 20.4 | _ | 0.45 | Min= 232 Max= 552 Ave= 438 | Well opened at 2:40 in 1403/03/29 |
| AZNS-127 | 54 | Fahliyan | Fa.1.2a.3a. | Producer | 0.2 | 1090 | 32 | Min= 600 Max= 1550 Ave= 1350 Mode= 1400 | Min= 160 Max= 210 Ave= 192 Mode= 200 | Min= 52 Max= 70 Ave= 66 Mode= 66 | 37.5 | _ | 0.01 | Min= 917 Max= 2998 Ave= 2640 | well deliver to ARVANDAN after 72 hours testing in 1403/10/07 |
| AZNS-128 | 42 | Sarvak | Sv.3.4-6 | Producer | 0.8 | 292 | 15 | Min= 180 Max= 400 Ave= 244 Mode= 300 | Min= 135 Max= 195 Ave= 178 Mode= 190 | Min= 14 Max= 30 Ave= 21 Mode= 24 | 21.1 | _ | 0.23 | Min= 216 Max= 450 Ave= 310 | well opened at 14:40 in 1403/03/10 |
| AZNS-129 | 12 | Sarvak | Sv.8 | Producer | 2 | 439 | 15 | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | Min= --- Max= --- Ave= --- Mode= --- | 19.6 | _ | 0.25 | Min= --- Max= --- Ave= --- | closed due to pressure drop in 1403/10/19 |
| AZNS-131 | 32 | Sarvak | Sv.3.4-6 | Producer | 4.8 | 439 | 28 | Min= 320 Max= 440 Ave= 382 Mode= 400 | Min= 140 Max= 190 Ave= 159 Mode= 140 | Min= 26 Max= 42 Ave= 35 Mode= 35 | 18.9 | _ | 0.28 | Min= 273 Max= 1371 Ave= 950 | well reopened at16:30 in 1403/11/21 |
| AZNS-133 | 76 | Gadvan | Gd.5a.5c | Producer | 2 | 818 | 30 | Min= 1000 Max= 1100 Ave= 1032 Mode= 1000 | Min= 125 Max= 190 Ave= 177 Mode= 185 | Min= 52 Max= 56 Ave= 55 Mode= 56 | 36.3 | _ | 1.34 | Min= 1132 Max= 2446 Ave= 2125 | well opened at 13:50 on 1403/02/11 |
| AZNS-134 | 60 | Sarvak | Sv.3 | Producer | 0.8 | 292 | 15 | Min= 220 Max= 290 Ave= 272 Mode= 270 | Min= 115 Max= 175 Ave= 156 Mode= 170 | Min= 28 Max= 38 Ave= 32 Mode= 32 | 22 | _ | 1.04 | Min= 178 Max= 365 Ave= 330 | well wqas closed from 16:25 to 18:00 due to 20'' line pig running. In 1403/11/27 |
| AZNS-135 | 49 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 15 | Min= 190 Max= 260 Ave= 218 Mode= 200 | Min= 170 Max= 195 Ave= 182 Mode= 190 | Min= 8 Max= 20 Ave= 15 Mode= 14 | 20.9 | _ | 0.31 | Min= 141 Max= 333 Ave= 280 | deliver to PEDEC for work over well 127 on on 1403/01/15 |
| AZNS-136 | 42 | Sarvak | Sv.3 | Producer | 2 | 292 | 24 | Min= 250 Max= 290 Ave= 270 Mode= 270 | Min= 140 Max= 200 Ave= 183 Mode= 190 | Min= 44 Max= 48 Ave= 46 Mode= 45 | 21.5 | _ | 0.39 | Min= 410 Max= 1001 Ave= 764 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-137 | 68 | Sarvak | Sv.3 | Producer | 0.8 | 361 | 20 | Min= 200 Max= 270 Ave= 221 Mode= 220 | Min= 120 Max= 130 Ave= 127 Mode= 130 | Min= 27 Max= 32 Ave= 29 Mode= 29 | 22 | _ | 0.45 | Min= 505 Max= 774 Ave= 602 | due to order of management change flow line from S-GASP to SKID at 14:30 in 1403/02/31 |
| AZNS-138 | 66 | Fahliyan | Fa.1c.2a | Producer | 0.5 | 1678 | 32 | Min= 1100 Max= 1750 Ave= 1436 Mode= 1700 | Min= 120 Max= 140 Ave= 130 Mode= 130 | Min= 70 Max= 77 Ave= 73 Mode= 72 | 35.7 | _ | 4.02 | Min= 2789 Max= 5453 Ave= 3703 | crack operation done in 1403/11/25 |
| AZNS-139 | 32 | Sarvak | Sv.3.4-6 | Producer | 1.5 | 439 | 20 | 220 | Min= 130 Max= 175 Ave= 142 Mode= 130 | Min= 24 Max= 44 Ave= 37 Mode= 27 | 18.7 | _ | 0.24 | Min= 337 Max= 677 Ave= 424 | well opened at 10:00. in 1403/09/07 |
| AZNS-140 | 57 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 15 | Min= 200 Max= 300 Ave= 215 Mode= 220 | Min= 110 Max= 175 Ave= 161 Mode= 170 | Min= 36 Max= 40 Ave= 38 Mode= 38 | 21.1 | _ | 1.23 | Min= 152 Max= 372 Ave= 274 | well wqas closed from 16:25 to 18:00 due to 20'' line pig running. In 1403/11/27 |
| AZNS-141 | 54 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 24 | Min= 200 Max= 250 Ave= 230 Mode= 240 | Min= 160 Max= 185 Ave= 175 Mode= 165 | Min= 10 Max= 24 Ave= 17 Mode= 20 | 20 | _ | 0.27 | Min= 370 Max= 741 Ave= 663 | well reopened at 14:15 with static pressur 450 and flow pressur 390psi in 1403/10/03 |
| AZNS-142 | 34 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 292 | 24 | Min= 140 Max= 260 Ave= 204 Mode= 200 | Min= 135 Max= 145 Ave= 139 Mode= 140 | Min= 6 Max= 20 Ave= 18 Mode= 20 | 21.5 | _ | 0.43 | Min= 554 Max= 1116 Ave= 852 | well open at 17:00 in 1403/04/03 |
| AZNS-143 | 36 | Sarvak | Sv.3 | Producer | 28 | 292 | 28 | Min= 200 Max= 350 Ave= 256 Mode= 300 | Min= 140 Max= 150 Ave= 144 Mode= 145 | Min= 41 Max= 50 Ave= 46 Mode= 46 | 19.7 | _ | 0.7 | Min= 959 Max= 1611 Ave= 1325 | well open at 17:00 in 1403/04/03 |
| AZNS-144 | 43 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 292 | 24 | Min= 200 Max= 250 Ave= 228 Mode= 220 | Min= 130 Max= 200 Ave= 176 Mode= 190 | Min= 37 Max= 45 Ave= 41 Mode= 42 | 21.3 | _ | 0.32 | Min= 381 Max= 890 Ave= 676 | well opened at 15:05 in 1403/03/10 |
| AZNS-145 | 40 | Sarvak | Sv.3.4-6 | Producer | 4 | 292 | 30 | Min= 220 Max= 400 Ave= 373 Mode= 380 | Min= 130 Max= 190 Ave= 176 Mode= 190 | Min= 28 Max= 36 Ave= 33 Mode= 32 | 21.3 | _ | 0.65 | Min= 768 Max= 1947 Ave= 1451 | well opened at 13:00. on 1403/02/13 |
| AZNS-148 | 77 | Fahliyan | Fa.2a.3a | Producer | 0.1 | 1678 | 28 | Min= 1400 Max= 1520 Ave= 1490 Mode= 1500 | Min= 130 Max= 190 Ave= 177 Mode= 180 | Min= 57 Max= 61 Ave= 60 Mode= 60 | 36.7 | _ | 2.37 | Min= 1042 Max= 2080 Ave= 1926 | crack operation done.well closed at 13:00 due to change of 3sv needle and opened at 14:00 in 1403/05/03 |
| AZNS-150 | 36 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 28 | Min= 170 Max= 210 Ave= 185 Mode= 190 | Min= 140 Max= 150 Ave= 144 Mode= 145 | Min= 54 Max= 58 Ave= 57 Mode= 58 | 20.1 | _ | 0.5 | Min= 847 Max= 1355 Ave= 1028 | well open at 17:00 in 1403/04/03 |
| AZNS-151 | 34 | Sarvak | Sv.3 | Producer | 0.1 | 292 | 24 | Min= 180 Max= 300 Ave= 250 Mode= 240 | Min= 120 Max= 155 Ave= 141 Mode= 140 | Min= 53 Max= 64 Ave= 61 Mode= 63 | 20.1 | _ | 0.44 | Min= 674 Max= 1220 Ave= 999 | well open at 17:00 in 1403/04/03 |
| AZNS-152 | 39 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 292 | 24 | Min= 380 Max= 470 Ave= 450 Mode= 450 | Min= 145 Max= 210 Ave= 191 Mode= 200 | Min= 36 Max= 45 Ave= 42 Mode= 43 | 22 | _ | 0.46 | Min= 625 Max= 1535 Ave= 1180 | well opened at 14:40 in 1403/03/10 |
| AZNS-153 | 60 | Sarvak | Sv.3 | Producer | 1 | 292 | 16 | Min= 200 Max= 230 Ave= 215 Mode= 220 | Min= 110 Max= 175 Ave= 155 Mode= 160 | Min= 17 Max= 33 Ave= 28 Mode= 30 | 22.5 | _ | 0.58 | Min= 169 Max= 343 Ave= 304 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| AZNS-154 | 70 | Sarvak | Sv.3 | Producer | 0.6 | 361 | 28 | Min= 200 Max= 220 Ave= 216 Mode= 220 | Min= 120 Max= 195 Ave= 133 Mode= 130 | Min= 19 Max= 30 Ave= 24 Mode= 23 | 22.3 | _ | 1.95 | Min= 885 Max= 1290 Ave= 1068 | well closed at 11:30 due to slick line operation and checking 3sv operation and opened at 17:45 in 1403/08/18 |
| AZNS-155 | 71 | Sarvak | Sv.3 | Producer | 0.3 | 361 | 20 | Min= 190 Max= 240 Ave= 223 Mode= 240 | Min= 120 Max= 160 Ave= 133 Mode= 120 | Min= 21 Max= 29 Ave= 25 Mode= 25 | 21.6 | _ | 0.33 | Min= 159 Max= 501 Ave= 404 | well opened at 16:10. in 1403/12/05 |
| AZNS-156 | 54 | Sarvak | Sv.3 | Producer | 0.3 | 292 | 20 | Min= 260 Max= 290 Ave= 273 Mode= 270 | Min= 160 Max= 190 Ave= 176 Mode= 165 | Min= 35 Max= 38 Ave= 36 Mode= 36 | 19.7 | _ | 0.47 | Min= 305 Max= 602 Ave= 555 | reopened at 14:00 with flow pressur 420psi in 1403/10/03 |
| AZNS-157 | 38 | Sarvak | Sv.3 | Producer | 0.6 | 292 | 26 | Min= 200 Max= 220 Ave= 201 Mode= 200 | Min= 130 Max= 145 Ave= 137 Mode= 140 | Min= 30 Max= 56 Ave= 39 Mode= 36 | 21.3 | _ | 0.48 | Min= 838 Max= 1237 Ave= 965 | well deliver to arvandan in 1403/06/17 |
| AZNS-158 | 73 | Sarvak | Sv.3 | Producer | 0.3 | 361 | 15 | 250 | Min= 120 Max= 185 Ave= 169 Mode= 180 | Min= 52 Max= 61 Ave= 56 Mode= 55 | 22.4 | _ | 1.67 | Min= 154 Max= 314 Ave= 287 | well opened at 13:50 on 1403/02/11 |
| AZNS-160 | 22 | Gadvan | Gd.3 | Producer | 0.1 | 1381 | 32 | Min= 540 Max= 570 Ave= 551 Mode= 550 | Min= 150 Max= 180 Ave= 152 Mode= 150 | Min= 56 Max= 77 Ave= 58 Mode= 57 | 29.6 | _ | 2.06 | Min= 1453 Max= 2144 Ave= 1672 | well open at 17:00 in 1403/04/03 |
| AZNS-161 | 68 | Sarvak | Sv.3 | Producer | 0.6 | 361 | 20 | Min= 200 Max= 220 Ave= 203 Mode= 200 | Min= 120 Max= 130 Ave= 128 Mode= 130 | Min= 24 Max= 34 Ave= 31 Mode= 31 | 21.3 | _ | 0.49 | Min= 487 Max= 775 Ave= 565 | well reopene at01:15 after arrest the leakge. In 1403/08/05 |
| AZNS-162 | 65 | Sarvak | Sv.3 | Producer | 0.4 | 292 | 30 | Min= 300 Max= 310 Ave= 307 Mode= 310 | Min= 120 Max= 130 Ave= 127 Mode= 130 | Min= 40 Max= 44 Ave= 42 Mode= 42 | 21.9 | _ | 0.99 | Min= 1512 Max= 2205 Ave= 1746 | well opened at 13:00 in 1403/03/10 |
| AZNS-163 | 28 | Sarvak | Sv.3.4-6 | Producer | 0.4 | 439 | 32 | Min= 300 Max= 350 Ave= 325 Mode= 330 | Min= 180 Max= 230 Ave= 191 Mode= 190 | Min= 57 Max= 63 Ave= 60 Mode= 61 | 19.4 | _ | 0.76 | Min= 1105 Max= 2199 Ave= 1392 | well closed due to pressure drop at 15:30. in 1403/07/03 |
| AZNS-166 | 9 | Kazhdumi | Kz.2-5 | Producer | 1 | 916 | 32 | Min= 1330 Max= 1620 Ave= 1395 Mode= 1400 | Min= 155 Max= 190 Ave= 167 Mode= 160 | Min= 42 Max= 57 Ave= 49 Mode= 48 | 22.94 | _ | 0.42 | Min= 2670 Max= 3577 Ave= 3000 | well closed at 10:00 due to control panel fault and opened at 10:30 in 1403/11/04 |
| AZNS-167 | 65 | Sarvak | Sv.3 | Producer | 0.1 | 292 | 24 | Min= 220 Max= 250 Ave= 240 Mode= 240 | Min= 120 Max= 130 Ave= 126 Mode= 125 | Min= 28 Max= 32 Ave= 30 Mode= 29 | 21.9 | _ | 0.56 | Min= 72 Max= 1167 Ave= 944 | Reopened after eliminate the leakage at 22:00 in 1403/12/07 |
| AZNS-168 | 30 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 24 | Min= 300 Max= 360 Ave= 346 Mode= 350 | Min= 130 Max= 180 Ave= 151 Mode= 135 | Min= 25 Max= 33 Ave= 29 Mode= 28 | 20.6 | _ | 0.77 | Min= 703 Max= 1437 Ave= 891 | well closed at 14:00 due to salt productin on 1403/01/18 |
| AZNS-169 | 28 | Sarvak | Sv.3.4-6 | Producer | 6 | 439 | 28 | Min= 320 Max= 350 Ave= 344 Mode= 340 | Min= 130 Max= 195 Ave= 147 Mode= 130 | Min= 40 Max= 49 Ave= 44 Mode= 44 | 17.6 | _ | 0.2 | Min= 874 Max= 1729 Ave= 1092 | well opened at 10:00. in 1403/09/07 |
| AZNS-170 | 29 | Sarvak | Sv.3 | Producer | 0.2 | 439 | 24 | Min= 500 Max= 550 Ave= 515 Mode= 500 | Min= 115 Max= 175 Ave= 145 Mode= 160 | Min= 41 Max= 46 Ave= 43 Mode= 43 | 21.5 | _ | 1.14 | Min= 969 Max= 1917 Ave= 1231 | well open at 17:00 in 1403/04/03 |
| AZNS-171 | 41 | Sarvak | Sv.3 | Producer | 0.6 | 292 | 26 | Min= 200 Max= 290 Ave= 260 Mode= 280 | Min= 135 Max= 200 Ave= 182 Mode= 190 | Min= 30 Max= 38 Ave= 35 Mode= 35 | 20.7 | _ | 0.38 | Min= 438 Max= 1169 Ave= 867 | closed at 8:00pm due to pressure drop at 2sv and reopen at 10 pm in 1403/04/09 |
| AZNS-172 | 22 | Sarvak | Sv.8 | Producer | 0.5 | 439 | 30 | Min= 50 Max= 190 Ave= 108 Mode= 100 | Min= 50 Max= 120 Ave= 106 Mode= 120 | Min= 10 Max= 52 Ave= 23 Mode= 12 | 20 | _ | 0.87 | Min= 316 Max= 1157 Ave= 636 | well open at 17:00 in 1403/04/03 |
| AZNS-173 | 72 | Gadvan | Gd.5 | Producer | 2 | 818 | 32 | Min= 500 Max= 600 Ave= 571 Mode= 580 | Min= 100 Max= 170 Ave= 155 Mode= 160 | Min= 40 Max= 55 Ave= 47 Mode= 49 | 33.9 | _ | 0.01 | Min= 712 Max= 1632 Ave= 1477 | delivered to arvandan(new well) in 1403/11/03 |
| AZNS-176 | 30 | Sarvak | Sv.3.4-6 | Producer | 1.6 | 439 | 20 | Min= 220 Max= 260 Ave= 242 Mode= 240 | Min= 130 Max= 230 Ave= 146 Mode= 140 | Min= 26 Max= 32 Ave= 30 Mode= 30 | 20.6 | _ | 0.45 | Min= 380 Max= 789 Ave= 480 | well closed at 14:00 due to salt productin on 1402/09/06 |
| AZNS-177 | 39 | Sarvak | Sv.3.4-6 | Producer | 0.1 | 292 | 24 | Min= 420 Max= 450 Ave= 424 Mode= 420 | Min= 145 Max= 210 Ave= 191 Mode= 200 | 20 | 19.7 | _ | 0.44 | Min= 604 Max= 1505 Ave= 1123 | well opened at 14:00 in 1403/03/10 |
| AZNS-178 | 43 | Sarvak | Sv.3 | Producer | 0.4 | 292 | 24 | Min= 300 Max= 320 Ave= 301 Mode= 300 | Min= 130 Max= 195 Ave= 176 Mode= 190 | Min= 36 Max= 45 Ave= 39 Mode= 38 | 21.5 | _ | 0.37 | Min= 442 Max= 1144 Ave= 849 | well opened at 15:05 in 1403/03/10 |
| AZNS-180 | 72 | Sarvak | Sv.3.4-6 | Producer | _ | 361 | 24 | Min= 145 Max= 250 Ave= 195 Mode= 200 | Min= 140 Max= 170 Ave= 149 Mode= 150 | Min= 7 Max= 32 Ave= 18 Mode= 15 | _ | _ | 0.001 | Min= 293 Max= 676 Ave= 533 | delivered to arvandan(new well) in 1403/11/07 |
| AZNS-181 | 3 | Sarvak | Sv.8 | Producer | 0.2 | 439 | 24 | Min= 340 Max= 420 Ave= 389 Mode= 400 | Min= 120 Max= 140 Ave= 130 Mode= 135 | Min= 19 Max= 39 Ave= 37 Mode= 39 | 21.1 | _ | 0.13 | Min= 1043 Max= 1601 Ave= 1211 | well closed due to fix chockexchange from24/64'' to 28/64''. In 1403/08/05 |
| AZNS-182 | 12 | Sarvak | Sv.8 | Producer | 4 | 439 | 20 | Min= 220 Max= 300 Ave= 285 Mode= 290 | Min= 140 Max= 165 Ave= 151 Mode= 150 | Min= 12 Max= 25 Ave= 18 Mode= 18 | 19 | _ | 0.38 | Min= 399 Max= 583 Ave= 482 | well closed at 00:45 due to leakage of 20" line and opened at 10:40 in 1403/08/11 |
| AZNS-184 | 9 | Sarvak | Sv.3.4-6 | Producer | 0.5 | 439 | 24 | Min= 200 Max= 250 Ave= 229 Mode= 240 | Min= 140 Max= 170 Ave= 149 Mode= 145 | Min= 17 Max= 25 Ave= 21 Mode= 18 | 20.2 | _ | 0.31 | Min= 461 Max= 707 Ave= 574 | well opened at 00:30 in 1403/03/10 |
| AZNS-185 | 37 | Sarvak | Sv.3.4-6 | Producer | 1 | 292 | 28 | Min= 390 Max= 420 Ave= 401 Mode= 400 | Min= 100 Max= 130 Ave= 118 Mode= 120 | Min= 38 Max= 47 Ave= 44 Mode= 42 | 22 | _ | 0.3 | Min= 1661 Max= 2452 Ave= 1909 | well open at 16:40 in 1403/04/03 |
| AZNS-186 | 3 | Sarvak | Sv.8 | Producer | _ | 439 | 20 | Min= 190 Max= 250 Ave= 199 Mode= 200 | Min= 120 Max= 190 Ave= 134 Mode= 120 | Min= 23 Max= 36 Ave= 26 Mode= 25 | _ | _ | 0.05 | Min= 433 Max= 666 Ave= 516 | Well delivered to arvandan in 1403/08/11 |
| AZNS-187 | 40 | Sarvak | Sv.3.4-6 | Producer | 4 | 292 | 24 | Min= 220 Max= 280 Ave= 241 Mode= 230 | Min= 135 Max= 200 Ave= 178 Mode= 190 | Min= 28 Max= 34 Ave= 31 Mode= 31 | 20.2 | _ | 0.33 | Min= 368 Max= 890 Ave= 682 | the check was changed from 24 to 28 by order of oil engineering. In 1403/04/23 |
| AZNS-188 | 91 | Sarvak | Sv.8 | Producer | 0.3 | 439 | 24 | Min= 190 Max= 210 Ave= 201 Mode= 200 | Min= 145 Max= 175 Ave= 159 Mode= 170 | Min= 15 Max= 23 Ave= 19 Mode= 20 | 19.6 | _ | 0.5 | Min= 620 Max= 913 Ave= 713 | well opened at 09:45 . In 1403/04/15 |
| AZNS-189 | 24 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 28 | Min= 280 Max= 400 Ave= 346 Mode= 370 | Min= 120 Max= 130 Ave= 123 Mode= 120 | Min= 17 Max= 35 Ave= 28 Mode= 28 | 20.9 | _ | 0.72 | Min= 1097 Max= 1879 Ave= 1445 | well open at 16:20 in 1403/04/03 |
| AZNS-190 | 19 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 20 | Min= 280 Max= 350 Ave= 295 Mode= 290 | Min= 115 Max= 130 Ave= 122 Mode= 120 | Min= 17 Max= 26 Ave= 23 Mode= 24 | 20.2 | _ | 0.42 | Min= 593 Max= 873 Ave= 707 | well open at 17:00 in 1403/04/03 |
| AZNS-191 | 33 | Sarvak | Sv.3 | Producer | 0.2 | 292 | 20 | Min= 180 Max= 250 Ave= 229 Mode= 250 | Min= 135 Max= 145 Ave= 141 Mode= 140 | Min= 13 Max= 53 Ave= 21 Mode= 21 | 21.6 | _ | 0.41 | Min= 498 Max= 844 Ave= 681 | well deliver to arvandan in 1403/05/30 |
| AZNS-192 | 9 | Sarvak | Sv.4-6 | Producer | 0.4 | 439 | 24 | Min= 210 Max= 250 Ave= 236 Mode= 240 | Min= 145 Max= 170 Ave= 153 Mode= 150 | Min= 18 Max= 27 Ave= 22 Mode= 22 | 20.4 | _ | 0.31 | Min= 498 Max= 708 Ave= 588 | well opened at 14:00. in 1403/07/05 |
| AZNS-194 | 37 | Sarvak | Sv.3 | Producer | 1 | 292 | 28 | Min= 290 Max= 320 Ave= 300 Mode= 300 | Min= 140 Max= 160 Ave= 152 Mode= 150 | Min= 32 Max= 39 Ave= 37 Mode= 37 | 19.9 | _ | 0.26 | Min= 1303 Max= 1943 Ave= 1511 | well opened at 14:00 on 1403/03/10 |
| AZNS-197 | 2 | Kazhdumi | Kz.4 | Producer | 0.1 | 916 | 28 | Min= 230 Max= 300 Ave= 275 Mode= 270 | Min= 80 Max= 165 Ave= 89 Mode= 80 | Min= 20 Max= 39 Ave= 26 Mode= 27 | 32.5 | _ | 0.47 | Min= 563 Max= 824 Ave= 647 | well opened at00:20 in 1403/03/10 |
| AZNS-199 | 24 | Sarvak | Sv.3.4-6 | Producer | 0.2 | 439 | 16 | Min= 100 Max= 150 Ave= 117 Mode= 120 | Min= 100 Max= 130 Ave= 115 Mode= 120 | Min= 10 Max= 21 Ave= 14 Mode= 13 | 20.3 | _ | 0.24 | Min= 212 Max= 791 Ave= 588 | well closed due to pressure dropat 11:20. in 1403/12/05 |
| AZNS-203 | 72 | Sarvak | Sar-3 | Producer | 0.2 | 361 | 24 | Min= 180 Max= 320 Ave= 253 Mode= 300 | Min= 100 Max= 185 Ave= 155 Mode= 160 | Min= 10 Max= 21 Ave= 14 Mode= 13 | 23 | _ | 0.01 | Min= 247 Max= 598 Ave= 488 | delivered to arvandan(new well) in 1403/11/03 |

Figure ‎3-3: Cumulative total producer wells and active wells of south Azadegan oil field reservoirs (up to February 2025)

Figure ‎3-4: Cumulative Production for South Azadegan field (up to February 2025)

Figure ‎3-5: Cumulative Production for Sarvak Reservoir of South Azadegan (up to February 2025)

Figure ‎3-6: Cumulative Production for Kazhdumi Reservoir of South Azadegan (up to February 2025)

Figure ‎3-7: Cumulative Production for Gadvan Reservoir of South Azadegan (up to February 2025)

Figure ‎3-8: Cumulative Production for Fahliyan Reservoir of South Azadegan (up to February 2025)

Figure ‎3-9: Cumulative oil production Bubble map and top 10 producer wells of South Azadegan Field (up to February 2025)

Figure ‎3-10: Cumulative oil production Bubble map and top 10 producer wells of Sarvak reservoir (up to February 2025)

Table ‎3-8: Cumulative oil production top 10 producer wells of Sarvak reservoir (up to February 2025)

|  | Top 10 producer wells of Sarvak |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate | Cum oil | Remark |
|  |  |  |  |  |  |  |  | (STBD) | (MMSTB) |  |
| 1 | AZN-006 | Near 79 | Sarvak | Sv.4-6 | Producer | 36 | 350 | 1720 | 13.6 | well opened at 00:10 on 1403/03/10 |
| 2 | AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 34 | 200 | 906 | 10.2 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| 3 | AZN-019 | 13 | Sarvak | Sv.3 | Close | 24 | _ | _ | 10.1 | closed due to pressure drop on 1400/07/12 |
| 4 | AZN-018 | 2 | Sarvak | Sv.3.4-6 | Close | 30 | _ | _ | 7.4 | closed at 15:00 due to pressure drop in 1403/03/27 |
| 5 | AZN-015 | Near 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 120 | 324 | 6.1 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 6 | AZN-002 | Near 36 | Sarvak | Sv.3.4-6 | Producer | 32 | 180 | 945 | 6 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 7 | AZN-005 | Near 1 | Sarvak | Sv.3.4-6 | Producer | 24 | 160 | 169 | 4.9 | well opened at 1:30 in 1403/03/10 |
| 8 | AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 28 | 300 | 994 | 4.3 | Well opened at 13:30 in 1403/06/04 |
| 9 | AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 28 | 300 | 843 | 2.7 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| 10 | AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 100 | 289 | 2.6 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |

Table ‎3-9: Cumulative oil production top 10 producer wells of Sarvak-3 reservoir (up to February 2025)

|  | Top 10 producer wells of Sarvak-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate | Cum oil | Remark |
|  |  |  |  |  |  |  |  | (STBD) | (MMSTB) |  |
| 1 | AZNS-039 | 23 | Sarvak | Sv.3 | Producer | 28 | 340 | 994 | 4.3 | 10" line.well opened at 13:30 in 1403/06/04 |
| 2 | AZNS-033 | 7 | Sarvak | Sv.3 | Producer | 28 | 300 | 938 | 2.46 | due to the leakage in the 32 inch lin manifold (2_3) wells were closed from west side of karun and directed to the rth gasp from 13:45 to 23:30 in 1403/05/07 |
| 3 | AZNS-102 | 75 | Sarvak | Sv.3 | Producer | 28 | 250 | 845 | 2.32 | deliver to PEDEC for work over well 116 on 1403/02/11 |
| 4 | AZNS-037 | 7 | Sarvak | Sv.3 | Producer | 24 | 243 | 602 | 2.21 | well reopened at 15:00. in 1403/07/03 |
| 5 | AZNS-034 | 23 | Sarvak | Sv.3 | Producer | 32 | 360 | 1374 | 2.09 | 10" line.well opened at 12:05 in 1403/06/03 |
| 6 | AZNS-036 | 50 | Sarvak | Sv.3 | Producer | 28 | 220 | 835 | 2 | Closed at11:20 due to air pressure drop & opened at 15:15 in 1403/05/05 |
| 7 | AZNS-082 | 69 | Sarvak | Sv.3 | Producer | 24 | 250 | 933 | 1.99 | well opened at 15:20 in 1403/03/10 |
| 8 | AZNS-154 | 70 | Sarvak | Sv.3 | Producer | 28 | 220 | 1086 | 1.95 | well closed at 11:30 due to slick line operation andchecking 3sv operation and opened at 17:45 in 1403/08/18 |
| 9 | AZNS-158 | 73 | Sarvak | Sv.3 | Producer | 15 | 250 | 287 | 1.67 | well opened at 13:50 on 1403/02/11 |
| 10 | AZNS-030 | 27 | Sarvak | Sv.3 | Producer | 28 | 290 | 1231 | 1.53 | well open at 17:00 in 1403/04/03 |

Table ‎3-10: Cumulative oil production top 10 producer wells of Sarvak3-4-6 reservoir (up to February 2025)

|  | Top 10 producer wells of Sarvak-3-4-6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate | Cum oil | Remark |
|  |  |  |  |  |  |  |  | (STBD) | (MMSTB) |  |
| 1 | AZN-006 | NEAR 79 | Sarvak | Sv.4-6 | Producer | 36 | 340 | 1720 | 13.6 | well opened at 00:10 on 1403/03/10 |
| 2 | AZN-008 | 31 | Sarvak | Sv.4-6 | Producer | 34 | 190 | 906 | 10.2 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| 3 | AZN-015 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 28 | 70 | 325 | 6.07 | 12" line( well was closed due to pressure drop from 00:30 to 06:00) in 1403/09/29 |
| 4 | AZN-002 | NEAR 36 | Sarvak | Sv.3.4-6 | Producer | 32 | 190 | 945 | 5.97 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 5 | AZN-005 | NEAR 1 | Sarvak | Sv.3.4-6 | Producer | 24 | 50 | 169 | 4.97 | well opened at 1:30 in 1403/03/10 |
| 6 | AZN-022 | 56 | Sarvak | Sv.3.4-6 | Producer | 20 | 100 | 289 | 2.64 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 7 | AZN-016 | 25 | Sarvak | Sv.3.4-6 | Producer | 20 | 80 | 231 | 2.58 | well opened at00:50 in 1403/03/10 |
| 8 | AZNS-095 | 31 | Sarvak | Sv.3.4-6 | Producer | 32 | 240 | 970 | 2.52 | well opened at00:50 in 1403/03/10 |
| 9 | AZNS-079 | 78 | Sarvak | Sv.3.4-6 | Producer | 26 | 340 | 960 | 2.35 | closed due to pressure drop on 1402/12/23 |
| 10 | AZNS-059 | 20 | Sarvak | Sv.3.4-6 | Producer | 28 | 250 | 1082 | 2.14 | well opened at 1:05 in 1403/03/10 |

Table ‎3-11: Cumulative oil production top 10 producer wells of Sarvak 8 reservoir (up to February 2025)

|  | Top 10 producer wells of Sarvak-8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate | Cum oil | Remark |
|  |  |  |  |  |  |  |  | (STBD) | (MMSTB) |  |
| 1 | AZNS-083 | 6 | Sarvak | Sv.8 | Producer | 28 | 260 | 843 | 2.73 | well opened at 00:10 on 1403/03/10 |
| 2 | AZNS-090 | 90 | Sarvak | Sv.8 | Producer | 22 | 260 | 533 | 1.49 | 12" line( well was closed due to contrl panel fault from 11:30 to12:40) in 1403/09/29 |
| 3 | AZNS-097 | 8 | Sarvak | Sv.8 | Producer | 16 | 200 | 269 | 1.38 | 12" line( well was closed due to pressure drop from 00:30 to 06:00) in 1403/09/29 |
| 4 | AZNS-068 | 6 | Sarvak | Sv.8 | Producer | 15 | 250 | 238 | 1.07 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 5 | AZNS-080 | 90 | Sarvak | Sv.8 | Producer | 15 | 240 | 254 | 1.07 | well opened at 1:30 in 1403/03/10 |
| 6 | AZNS-099 | 91 | Sarvak | Sv.8 | Producer | 20 | 160 | 291 | 1.04 | well closed at 20:10 and opened at 21:40 due to diesel generator fault in 1403/04/29 |
| 7 | AZNS-105 | 16 | Sarvak | Sv.8 | Producer | 15 | 200 | 210 | 0.9 | well opened at00:50 in 1403/03/10 |
| 8 | AZNS-172 | 22 | Sarvak | Sv.8 | Producer | 30 | 100 | 636 | 0.87 | well opened at00:50 in 1403/03/10 |
| 9 | AZNS-096 | 14 | Sarvak | Sv.8 | Producer | 24 | 190 | 501 | 0.83 | closed due to pressure drop on 1402/12/23 |
| 10 | AZNS-087 | 8 | Sarvak | Sv.8 | Producer | 15 | 220 | 246 | 0.68 | well opened at 1:05 in 1403/03/10 |

Figure ‎3-11: Cumulative oil production Bubble map and top 10 producer wells of Kazhdumi reservoir (up to February 2025)

Table ‎3-12: Cumulative oil production top 10 producer wells of Kazdhumi reservoir (up to February 2025)

| Top 10 producer wells of Kazhdumi |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Cum oil (MMSTB) | Remark |
| 1 | AZN-017 | 50 | Kazhdumi | Kz.2 | Producer | 20 | 1440 | 1345 | 13.94 | 10' line( well was closed from 11:00 to 16:00 due to MPFM connection) in 1403/08/12 |
| 2 | AZN-004 | Near 56 | Kazhdumi | Kz.1.2 | Producer | 32 | 1420 | 2902 | 9.82 | the well was closed and opened for spool replacement on 1402/12/14 |
| 3 | AZN-021 | 15 | Kazhdumi | Kz.2 | Close | 24 | 0 | _ | 5.36 | closed due to pressure drop due to water production on 1399/02/11 |
| 4 | AZNS-092 | 19 | Kazhdumi | Kz.2.4 | Producer | 32 | 1020 | 2376 | 5.34 | well opened at00:50 in 1403/03/10 |
| 5 | AZNS-041 | 20 | Kazhdumi | Kz.2.4 | Close | 28 | 0 | _ | 1.06 | well closed due to asphaltene barrier in the well on 1402/10/11 |
| 6 | AZNS-064 | 13 | Kazhdumi | Kz.2.4 | Close | 28 | 0 | _ | 0.54 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |
| 7 | AZNS-028 | 11 | Kazhdumi | Kz.2.3.4 | Close | 24 | 0 | 728 | 0.54 | closed due to asphaltene barrier on 1398/08/08 |
| 8 | AZNS-029 | 5 | Kazhdumi | Kz.3.4 | Producer | 28 | 190 | 518 | 0.48 | well closed at 11:00 due to disconnect of MPFM and opened at 17:30 in 1403/07/01 |
| 9 | AZNS-197 | 2 | Kazhdumi | Kz.4 | Producer | 28 | 300 | 650 | 0.47 | well opened at00:20 in 1403/03/10 |
| 10 | AZNS-166 | 9 | Kazhdumi | Kz.2.5 | Producer | 32 | 1400 | 3000 | 0.42 | well closed at 10:00 due to control panel fault and opened at 10:30 in 1403/11/04 |

Figure ‎3-12: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to February 2025)

Table ‎3-13: Cumulative oil production top 10 producer wells of Gadvan reservoir (up to February 2025)

| Top 10 producer wells of Gadvan |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (PSI) | Oil Rate (STBD) | Cum oil (MMSTB) | Remark |
| 1 | AZN-009 | 16 | Gadvan | Gd.5c | Producer | 50 | 900 | 3982 | 29.08 | well closed due to fault WK at 02:55 and opened at 13:30 on 1402/09/05 |
| 2 | AZN-023 | Near 75 | Gadvan | Gd.5a.5c | Producer | 34 | 1200 | 2960 | 16.3 | well closed due to fault WK at 03:10 and opened at 14:50 on 1402/09/05 |
| 3 | AZN-001 | Near 19 | Gadvan | Gd.5a.5c | Producer | 36 | 500 | 1481 | 14.24 | well opened at 13:30 on 1403/03/21 |
| 4 | AZN-012 | Near 75 | Gadvan | Gd.5a.5c | Suspended | 43 | _ | _ | 8.12 | establishment work over rig in program by PEDEC on 1400/04/24 |
| 5 | AZNS-026 | 23 | Gadvan | Gd.1-6 | Producer | 32 | 600 | 1234 | 5.78 | 10" line. Well opened at 21:20 in 1403/06/03 |
| 6 | AZN-020 | 7 | Gadvan | Gd.5c | Close | 24 | 50 | _ | 5.25 | well closed due to pressure drop at12:00. in 1403/10/17 |
| 7 | AZNS-071 | 17 | Gadvan | Gd.5c | Producer | 28 | 550 | 965 | 3.32 | well closed from08:00 to 10:30 due to problem in control panel in 1403/08/06 |
| 8 | AZNS-047 | 77 | Gadvan | Gd.5a.5c | Producer | 30 | 1400 | 2676 | 2.8 | well opened at 14:30 on 1403/02/11 |
| 9 | AZNS-086 | 1 | Gadvan | Gd.5c | Producer | 36 | 700 | 1350 | 2.73 | due to the leakage in the 32 inch line manifold (2_3) wells were closed from west side of Karun and directed to the North gasp from 13:45 to 23:30 in 1403/05/07 |
| 10 | AZNS-058 | 18 | Gadvan | Gd.5c | Producer | 32 | 700 | 1423 | 2.57 | Pressure Drop due to sand Production & Work over rig program on 1398/12/15 |

Figure ‎3-13: Cumulative oil production Bubble map and top 10 producer wells of Fahliyan reservoir (up to February 2025)

Table ‎3-14: Cumulative oil production top 10 producer wells of Fahliyan reservoir (up to February 2025)

| Top 10 producer wells of Fahliyan |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Well | Pad | Formation | Zone | Well Status | Chock size (1/64)" | WHP (Psi) | Oil Rate (STBD) | Cum oil (MMSTB) | Remark |
| 1 | AZN-011 | Near 66 | Fahliyan | Fa.3a | Producer | 34 | 1200 | 2750 | 10.15 | well closed at 05:45 due to fault of panel and opened at 10:10 on 1403/02/04 |
| 2 | AZNS-049 | 69 | Fahliyan | Fa.1a.1c.3a | Producer | 32 | 1500 | 3116 | 6.55 | well opened at 20:30 in 1403/10/01 |
| 3 | AZNS-070 | 75 | Fahliyan | Fa.2a.3a | Producer | 30 | 1350 | 1996 | 6.45 | well was closed from 09:00 to 11:20 due to control panel problom. In 1403/09/28 |
| 4 | AZNS-046 | 62 | Fahliyan | Fa.1.3a | Producer | 49 | 1250 | 6055 | 4.1 | the well was closed from 10:15 to 13:15 to clean the stone trap in 1403/05/01 |
| 5 | AZNS-138 | 66 | Fahliyan | Fa.1c.2a | Producer | 32 | 1700 | 3703 | 4.02 | FIX CHOCK CHANGE OF 32/64 TO 34/64 in 1403/10/15 |
| 6 | AZNS-110 | 58 | Fahliyan | Fa.4a | Producer | 30 | 700 | 1499 | 2.63 | the well was closed from 02:40 to 17:00 due to a 20 inch line leak in 1403/11/05 |
| 7 | AZNS-121 | 44 | Fahliyan | Fa.1 | Producer | 34 | 2200 | 4056 | 2.58 | Well opened at 1:20 in 1403/03/29 |
| 8 | AZNS-148 | 77 | Fahliyan | Fa.2a.3a | Producer | 28 | 1500 | 1926 | 2.37 | crack operation done. Well closed at 13:00 due to change of 3sv needle and opened at 14:00 in 1403/05/03 |
| 9 | AZNS-050 | 57 | Fahliyan | Fa.4a | Producer | 28 | 861 | 1450 | 2.19 | well closed at 00:30 due to leakage of 20" line and opened at 04:15 in 1403/08/11 |
| 10 | AZNS-060 | 78 | Fahliyan | Fa.4a | Producer | 20 | 280 | 1030 | 1.91 | well reopened after rig down of subsurface gob facilities at 16:15. in 1403/10/26 |